3GPP TSG RAN Meeting #110					RP-253268

Baltimore, U.S.A., December 8th – 11th, 2025

Agenda Item:	8.2.1.3.3

Source:            CATT, China Unicom, China Telecom

Title:	Operational requirements on easy operation/self organization and RAN sharing

Document for:	Approval

# 1	Introduction

This contribution presents an analysis of operational requirement on easy operation/self-organization and RAN sharing, and provides corresponding TP.

# Discussion

Easy opeartion and self organization.

Easy operation and self-organization have been essential requirements in radio access network deployment since the 4G era. Traditionally, these capabilities are divided into two main aspects: self-configuration and self-optimization.

Self-configuration refers to the automated process through which newly deployed network nodes are installed and configured with the basic settings required for operation. A key technique in this area is Automatic Neighbor Relation, which has been supported from the initial releases of both 4G and 5G. Self-configuration greatly reduces manual intervention, decreases OPEX, and supports fast, scalable network deployment. Given its importance, this operational requirement should remain a cornerstone in 6G RAN deployment.

As for self-optimization, both 4G and 5G have seen the development of Self-Organizing Networks  (SON) and Minimization of Drive Tests (MDT) within RAN working groups. SON improves network robustness and user experience by dynamically adjusting parameters in response to traffic fluctuations and potential failures. MDT offers deep insight into real-user experiences, helping operators precisely identify and resolve issues related to coverage, capacity, and mobility. Together, SON and MDT create a self-optimizing network that continuously enhances performance, ensures efficient resource use, and maintains high QoS.

Moreover, 5G has begun incorporating AI/ML-assisted optimization in areas such as mobility management, load balancing, and energy saving. In 6G, we envision an AI-native network where AI/ML-driven optimization will be integral to self-optimization, which moves the network management from being reactive and rule-driven to being predictive, adaptive and cognitive.

Based on these considerations, we have the proposal as below:

Proposal 1: Add the following requirements for easy operation and self organization:

The RAN design for 6G shall fulfill the following requirements:

- RAN shall support self-configuration to enable the automatic plug-and-play deployment of network nodes, establishing their basic operational parameters without manual intervention.

- RAN shall support SON/MDT features to  provide the foundational, rule-based automation and data-driven visibility for robust and efficient network operations.

- RAN shall support AI/ML-assisted optimization to empower 6G networks to autonomously predict issues, optimize for complex and conflicting goals, and continuously self-adapt in real-time

RAN sharing

RAN sharing has reached large-scale commercial reality in the 5G era, primarily driven by the need to reduce costs and improve network coverage. A prominent example is the collaboration between China Telecom and China Unicom, who have built the world's largest 5G shared network. As for 6G, the principles and practices of RAN sharing are expected to become even more critical. The capital and operational expenditure required to build 6G network make sharing an economic imperative. Furthermore, RAN sharing could also facilitate the deployment of new services e.g. AI/ML and sensing via sharing the data and computing resources among operators. Thereby, we have the following proposal:

Proposal 2: Add the following requirement on RAN sharing:

The RAN design for 6G shall support RAN sharing to enable the dynamic and intelligent utilization of pooled resources and reduce the capital and operational expenditures of network deployment

# 3    	Text proposal for TR 38.914

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

## 5.3	Operational requirements

Editor note: Spectrum, co-existence, topology and other operational requirements

### 5.3.1	Spectrum

Editor note: e.g. spectrum and co-existence related

6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR.

6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

### 5.3.2	Diverse device types

### 5.3.3	RAN sharing

The RAN design for 6G shall support RAN sharing to enable the dynamic and intelligent utilization of pooled resources and reduce the capital and operational expenditures of network deployment.

### 5.3.4	Ease of operation and Self Organization

The RAN design for 6G shall fulfill the following requirements:

- RAN shall support self-configuration to enable the automatic plug-and-play deployment of network nodes, establishing their basic operational parameters without manual intervention.

- RAN shall support SON/MDT features to  provide the foundational, rule-based automation and data-driven visibility for robust and efficient network operation.

- RAN shall support AI/ML-assisted optimization to empower 6G networks to autonomously predict issues, optimize for complex and conflicting goals, and continuously self-adapt in real-time.

### 5.3.5	Network Resilience

### 5.3.6	[Service awareness in RAN]

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>