3GPP TSG RAN Meeting #110												RP-253280

Baltimore, US, December 8 - 11, 2025

Agenda Item:	8.2.1.0

Source:	Huawei, HiSilicon

Title:	Consideration on 6G deployment scenarios

Document for:	Discussion and decision

Introduction

A RAN-level study on “6G Scenarios and requirements” was approved in RAN#107 [1]. Regarding the scenario and relevant key assumptions in the scenario, the objective of this SID includes:

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- …...

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

…..

In this contribution, the considerations and views on the 6G deployment scenarios and assumptions are discussed and corresponding text proposals are provided.

Requirements of deployment scenario consideration

Regarding the tables describing the attributes for the scenarios, i.e., indoor hotspot, Dense urban, Rural, Urban Macro, Sub-Urban macro, there is one row of service profile that is supposed to present the services of interest for the study for the corresponding scenario. However, currently only full buffer or non-full-buffer is stated, which does not accurately reflect the services to be studied.

Service profile

TR 38.914 [4] currently considers both full buffer and non-full-buffer traffic as the service profile for 6G study without further detailed assumptions for the study. Usually, regarding the non-full-buffer traffic, the FTP-3 model with the packet size and the arrival rate modelled is widely used for the communication traffic without the latency requirement. In 5G NR study, the traffic with the latency requirement was also considered in Rel-17, such as XR, for which the XR traffic models were defined in TR 38.838 [5].

Likewise, 6GR study should consider more services including the services with or without the latency requirements studied in 5G and the new services, e.g., immersive communication, AI/ML, etc., that are beyond 5G. RAN1 is studying the traffic models as well for the new services accordingly.

In RAN1#123 meeting [13], the study on traffic model was agreed as below:

Agreement For FTP Model 3, the packet delay budget (PDB) can be additionally considered, - The latency characteristic of the traffic in RAN side (i.e., air interface) is modelled as packet delay budget (PDB). The PDB is a limited time budget for a packet to be transmitted over the air from a BS to a UE for DL, or from a UE to a BS for UL. - For a given packet, the delay of the packet incurred in air interface is measured from the time that the packet arrives at the BS to the time that it is successfully transferred to the UE for DL, or from a UE to a BS for UL. If the delay is larger than a given PDB for the packet, the packet is said to violate PDB, otherwise the packet is said to be successfully delivered. - Values for PDB, e.g., {10ms, 20ms, 30ms, 50ms, 100ms, 200ms, 300ms, 1000ms, 2000ms} can be considered. - Which values will be used will consider the use case for the evaluations. Working Assumption For 6GR evaluations related to immersive communications services, the following two amended XR models based on the existing XR traffic model (in TR 38.838) can be considered: - Model-1: eXR model without Haptics - Regarding the statistical parameters for single stream CG traffic model defined in Table 5.4.1-1 TR 38.838, add values for immersive gaming regarding the data rate and the frame generation rate as in red: Parameters; unit; Baseline values for evaluation; Optional values for evaluation; Values for immersive gaming. data rate: R; Mbps; 30, 8; 45; 100, 300, 500. frame generation rate: F; fps or Hz; 60; 90,120. PDB; ms; 15; 10, 30; 15, or 10, 30. - Regarding the statistical parameters for packet size following truncated Gaussian distribution in Table 5.1.1.1-1 TR 38.838, add values for immersive gaming regarding STD, Max, and Min values as in red: Parameter; unit; Baseline values for evaluation; Optional values for evaluation for single eye buffer; Values for immersive gaming. Mean: M; byte; R×1e6 / F / 8; R×1e6 / F / 8; R×1e6 / F / 8. STD; byte; 10.5% of M; 3 % of M; [25 %] of M. Max; byte; 150% of M; 109% of M; 300% of M. Min; byte; 50% of M; 91% of M; 25% of M. R: data rate of the flow in Mbps. F: frame generation rate of the flow in fps. Note that the mean and STD apply before truncation applies. Note that the value of R, F depend on application.. - Regarding the statistical parameters for AR UL Model 1 defined in Table 5.5.2.1-1 TR 38.838, add values for UL-heavy video uploading regarding packet size, generate rate, data rate, and PDB values as in red: Parameters; unit; value; Values for UL video uploading. Packet size; byte; Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=10.5/50/150%); 1st candidate: Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=10.5/50/150%) 2nd candidate: Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=[25]/25/300%). packet generation rate: F; Hz; 60; 15, 30. Jitter; ms; Optional, follows the description in clause 5.1.1.2; Optional, follows the description in clause 5.1.1.2. Data rate: R; Mbps; 10 (baseline), 20 (optional); 20, 60, 100. PDB; ms; 30 (baseline), 10 or 15 or 60 (optional); 10, 15. - The jitter is modelled the same as XR traffic model. - Model-2: eXR model with Haptics - Haptics traffic is defined as XR traffic packet generation with co-generated haptics packets. - FFS on how to generate the multi-channel haptics packet including how to handle silent periods of haptics and the haptics packet sizes. - FFS on how to co-generate haptics packets and the XR traffic packets. - Haptics packets has packet delay budget (PDB) of either 12 msec or 30 msec, which can be selected as a traffic model parameter. - Send LS to SA4 to inform about the above agreement and check if SA4 has related inputs for the model. Agreement For traffic model(s) for AI/ML services, the following can be considered: - Packet size: - How to model the packet size, a fixed one or multiple values, or modelled as a random variable. - Packet arrival: - FFS the details to determine the packet arrival rate, e.g., - N multiple packets arrive together as a burst. The burst interval time is modelled as a random variable. - Within the burst, the N packets arrive according to a statistical distribution. - Packets arrive separately. - FFS whether/how to model the Jitter and the relation with the packet arrival. - FFS: Whether the packet importance is known. Whether/how to reflect the packet importance. - Whether/How to consider the PDB, e.g., Packet delay budget: The latency characteristic of the traffic in RAN side (i.e., air interface) is modelled as packet delay budget (PDB). The PDB is a limited time budget for a packet to be transmitted over the air from a base station to a UE, or from a UE to a base station - FFS Whether/how to consider the Packet success rate requirement: [xx%] and the relation with the PDB. - FFS how to model different cases, e.g., image-based GenAI, video-based GenAI, and chatbot, etc. - FFS: Whether/how other traffic models (e.g., XR, FTP1/3) can be used to reflect above characteristics.

As being studied in RAN1, for traffic with latency requirement, such as XR, immersive communication, AI/ML, etc., it is important to consider how latency requirements are included in traffic models. The XR traffic models in TR 38.838 [5] have captured packet delay budget requirements, which can be used for XR composite requirement evaluation. For immersive communication, the new traffic model under RAN1 study is to update some parameters of the current XR traffic model or add some other parameters to support the 6G immersive evaluation. For the evaluation of AI/ML related services, the new traffic models for AI/ML should consider some unique characteristics related to AI/ML services, e.g., error tolerance, by reflecting the packets with different important levels, as being studied in RAN1.

Overall, 6G RAN study should consider more service profiles.

Proposal 1: The service profiles to be captured in the 6G TR 38.914 should not only consider the services modeled with full buffer traffic or non-full-buffer traffic as in the past but also the new services with the QoS requirements, e.g.,

- for immersive communication, use the extended XR traffic models studied in RAN1 for the evaluations with the composite requirement;

- for energy efficiency consideration, use the FTP-3 model with PDB requirement considered as agreed in RAN1#123;

- for AI/ML related services, RAN1 continues the study of the traffic model considering the related QoS requirements.

UE antenna elements

From service perspective, the emerging use cases in 6G, such as applications of Extended Reality (XR), sophisticated Digital Twin (DT), and rapid advancement of AI require, necessitate data rates far exceeding those supported by current 5G networks. Advanced XR and immersive metaverse experiences will require network capacities 100 times greater than current capabilities, with data rates up to 10 Gbps per user [7]. Even higher data rates requirements than tens of Gbps are forecast for services like Digital Twins and haptic communication [8-10]. The rapid development of AI also brings higher data rates to produce synthesized data timely far exceeding human-generated data. These high-performance applications require more advanced capabilities from both UE and network sides.

On the other hand, capabilities from network and UE sides are kept evolving, from 4G to 5G, large array massive MIMO has been introduced which brings 5G higher spectral efficiency, higher data rate and coverage performance than 4G. In the upcoming 5 years, the techniques will continue to evolve and the practical parameters for future deployment shall take such development into account.

Having the above in mind, 6G shall consider to evaluate a higher number of antenna ports.

From TRP perspective, 6G already agreed BS antenna number as below in TR 38.914 [4].

BS antenna elements; Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements.

From UE perspective, the UE antenna elements in sub-3GHz band (e.g. 700MHz, 2GHz) and 5G C-band are defined as up to 4 Tx/Rx and 8 Tx/Rx in TR 38.913 [11], respectively. Also, in Rel-19, 3GPP studies the channel model in 7-24GHz spectrum, and the conclusions related to UE antenna element are agreed as below:

Agreement in RAN1#121 Agreement in RAN1#120b For CPE devices adopt the following device dimensions for UE antenna modeling: - (x cm × y cm × z cm) = (0 cm × 20 cm × 20 cm) - Total 9 candidate antenna locations in 4 corners of the vertical square plane, 4 center of the edges of the vertical square plane, and center of the vertical square plane. The figure below captures the side view on the device. - Note: a candidate antenna location (e.g., 9) can be used for multiple antennas

Based on the channel model and evaluation assumptions in the current 3GPP release, the UE supported antenna elements may vary per different bands or according to different device type capabilities. For ~7GHz the bandwidth is large, to well support massive MIMO, the UE with more advanced capabilities needs to support higher antenna elements, e.g.  16 antenna elements; while for UEs with lower capabilities, 4, 8 can also be considered. It should be noted that the number of UE Tx or Rx ports is usually smaller than the number of antenna elements, and can be further discussed in the evaluation stage, e.g. from 2T4R to 16T16R considering different device types and the way of antenna distribution within the devices. Additionally, considering there are different UE implementations, other distribution of UE antenna elements may need to be further considered.

Proposal 2: The following maximum numbers of UE antenna elements are considered for 6GR study:

- Around 7GHz: Up to 16 antenna elements

- Around 2/4GHz: Up to 8 antenna elements

- Around 700MHz: Up to 4 antenna elements

In RAN1#123 meeting [13], the assumption for UE antenna modelling was discussed but was not agreed.

Alternatively, as in 5G TR38.913, the maximum number of the antenna element can be defined in TR 38.914 for 6GR in RAN plenary meeting. The vendors can implement different configurations based on the different device type capabilities. However, all the combinations can be considered for 6GR study.

Proposal 3: All the combinations for the UT antenna modelling in this table are considered for 6GR study; specific UE antenna element configuration used in the evaluation can be different according to the device type of interest in the evaluations.

| UE antenna modelling for RAN1 evaluations | Total number of antenna elements | Total number of TXRU | Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901 | Applicable carrier frequency |
| --- | --- | --- | --- | --- |
| Combination0 | 1 | 1T1R, | Alt 1: 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) 1R: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) Alt 2: - 1T - 1R | 700MHz, 2GHz |
| Combination1 | 2 | 1T2R, | Alt 1: - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) for single polarization or (1, 1, 2, 1, 1; 1, 1) for dual polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 2R: [(1, 5), or (4, 8)] as described in section 7.3 in TR 38.901. | 700MHz, 2GHz, 4GHz |
| Combination2 | 4 | 1T4R, 2T4R, 4T4R | Alt 1: - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 4R: [(2, 4, 6, 8), or (1, 3, 5, 7)] as described in section 7.3 in TR 38.901 | 700MHz 2GHz, 4GHz, 7GHz, 15GHz |
| Combination3 | 8 | 1T8R, 4T8R, 8T8R | Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 | 2GHz, 4GHz, 7GHz, 15GHz |
| Combination4 | 16 | FFS: 4T16R, 8T16R, 16T16R | Alt 1: - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; x, y) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 16R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization | 7GHz, 15GHz |

ISAC study

ISAC is one newly defined use case in IMT-2030. It is more cost-effective to utilize communication network to provide ISAC capabilities for the practical deployment. Therefore, the scenarios for ISAC study can at least consider those deployment scenarios as captured in the RAN TR38.914, such as indoor, dense urban, rural, urban macro and so on.

The major additional assumption is the sensing target distribution and corresponding velocity. The specific value can be different for different deployment scenarios, e.g., less sensing targets can be considered for indoor scenario, compared to that for the outdoor scenarios.

Proposal 4: Sensing target distribution and velocity can be added into the deployment scenarios as additional assumption for ISAC study, including indoor hotspot, dense urban, rural, urban macro, and sub-urban macro scenarios.

NTN scenario

Non-Terrestrial-Network (e.g. Satellite) is an essential complement to TN and therefore its deployment scenario shall also be considered. Different kinds of satellite orbits need to be addressed considering different regions requirements, and evaluation on FDD/HD-FDD MSS bands already defined in FR1(S/L band) and FR2(Ka) needs to be supported as well.

In additional, the RAN1 has already agreed the candidate carrier frequencies for 6G NTN evaluation as below.

Agreement in RAN1#122bis At least the following carrier frequencies could be considered (from RAN1 perspective) for 6GR NTN evaluations: - L-band (i.e., 1.5GHz) - S-band (i.e. 2 GHz) - Ku-band (FFS detailed frequency range) - Ka-band (i.e. 30 GHz for UL, 20GHz for DL) The attached templates for NTN in R1-2507956 are endorsed in principle. Agreement in RAN1#123 For 6GR NTN evaluations, the carrier frequency for Ku-band is 14GHz for UL and 11GHz for DL.

Proposal 5: NTN shall be supported as one of deployment scenarios in S/L band, Ka band and Ku band for 6GR study.

Further, some of the assumptions in below table such as carrier frequency, duplexing mode or system and simulation bandwidth can be discussed under the general assumptions for NTN. For NR-NTN, several NTN bands in FR1 and FR2 are defined in TR 38.913, and only FDD/HD-FDD is allowed on these bands. For 6GR NTN, for the antenna model parameters, especially for phased antenna arrays expected to be used in 6G NTN, they may be discussed along with the TN parameters. Some of the main attributes of the NTN deployment are listed in the below table. Considering the (V)LEO usage will be widely deployed in the further, we propose to study the dense/sparse (V)LEO constellation with massive beam footprints per satellite.

| Attributes | S/L Band | Ku-Band | Ka-Band |
| --- | --- | --- | --- |
| Carrier Frequency | Around 2 GHz for both DL and UL | Around 14GHz for UL Around 11GHz for DL | Around 20 GHz for DL Around 30 GHz for UL |
| Duplexing Mode | FDD/HD-FDD | FDD/HD-FDD | FDD/HD-FDD |
| System Bandwidth (DL + UL) | Up to 2*30 MHz | TBD | Up to 2*400 MHz |
| Satellite architecture | Bent-pipe On-board Processing | TBD | Bent-pipe, On-Board Processing |
| Typical satellite system positioning in the 6G architecture | Access network | TBD | Access network Backhaul network |
| Satellite Orbit | GEO, MEO, (V)LEO | GEO, MEO, (V)LEO | GEO, MEO, (V)LEO |
| UE Outdoor/Indoor percentage | 100% Outdoors | 100% Outdoors | 100% Outdoors |

UAV scenario

In TR 36.777 [12], deployment scenarios were defined for aerial vehicles (AV) to investigate the coverage of LTE network deployments. In TR 38.913 [11], a light aircraft scenario was defined to allow for the provision of services for general aviation aircrafts to initiate and receive mobile services. 3GPP has provided connectivity support for UAV in both 4G and 5G. With the thriving growth of low-altitude economy, the UAV scenario should also be supported in 6G evaluation.

Proposal 6: UAV scenario shall be supported as one of deployment scenarios for 6GR study.

Summary

We have discussed the principles of deployment scenarios. The proposals are as below:

Proposal 1: The service profiles to be captured in the 6G TR 38.914 should not only consider the services modeled with full buffer traffic or non-full-buffer traffic as in the past but also the new services with the QoS requirements, e.g.,

- for immersive communication, use the extended XR traffic models studied in RAN1 for the evaluations with the composite requirement;

- for energy efficiency consideration, use the FTP-3 model with PDB requirement considered as agreed in RAN1#123;

- for AI/ML related services, RAN1 continues the study of the traffic model considering the related QoS requirements.

Proposal 2: The following maximum numbers of UE antenna elements are considered for 6GR study:

- Around 7GHz: Up to 16 antenna elements

- Around 2/4GHz: Up to 8 antenna elements

- Around 700MHz: Up to 4 antenna elements

Proposal 3: All the combinations for the UT antenna modelling in this table are considered for 6GR study; specific UE antenna element configuration used in the evaluation can be different according to the device type of interest in the evaluations.

Proposal 4: Sensing target distribution and velocity can be added into the deployment scenarios as additional assumption for ISAC study, including indoor hotspot, dense urban, rural, urban macro, and sub-urban macro scenarios.

Proposal 5: NTN shall be supported as one of deployment scenarios in S/L band, Ka band and Ku band for 6GR study.

Proposal 6: UAV scenario shall be supported as one of deployment scenarios for 6GR study.

Proposal 7: Adopt the corresponding text proposals present in section 5.

References

- RP-250810, “New SID: Study on 6G Scenarios and requirements”, RAN#107, March 2025.

- RP-243325, “DRAFT LTI Response to LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, RAN#106, December 2024.

- RP-250819, “Moderator's summary for IMT-2030 TPRs”, RAN#107, March 2025.

- RP-251873, 3GPP TR 38.914 V0.1.0, “Study on 6G Scenarios and Requirements”

- 3GPP TSG RAN WG1 #122, Chair notes RAN1#122, Bengaluru, India, Aug 25th – 29th, 2025.

- 3GPP TR 38.838, Study on XR (Extended Reality) evaluations for NR

- RP-252129, Consideration on 3GPP requirements and KPIs in addition to ITU-R IMT-2030 TPRs, Huawei, HiSilicon

- Qualcomm, "Why and what you need to know about 6G in 2022," [Online]. Available: https://www.qualcomm.com/content/dam/qcomm-martech/dm-assets/documents/web-why-and-what-you-need-to-know-about-6G-in-2022.pdf

- Ericsson: 6G - Follow the journey to the next generation networks, Available: https://www.ericsson.com/en/6g

- S. Tarkoma et al., "Exploring the 6G Potentials: Immersive, Hyper-Reliable, and Low-Latency Communication," arXiv preprint arXiv:2310.06425, 2023.

- "The Digital Twin Technology Applied To 6G Communication," Available: https://semiengineering.com/the-digital-twin-technology-applied-to-6g-communication/

- 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

- 3GPP TSG RAN WG1 #122bis, Chair notes RAN1#122bis, Prague, Czech Republic, October 13-17, 2025.

- 3GPP TSG RAN WG1 #123, Chair notes RAN1#123, Dallas, USA, Nov 17 – 21, 2025.

Text proposal for the TR

< Unchanged parts are omitted >

4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

Some of its attributes are listed in Table 4.1.

Table 4.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz |
| Aggregated system bandwidth NOTE1, NOTE2 | Around 2 GHz: Up to 200 MHz (DL+UL) . Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1GHz (DL+UL) |
| Layout | Single layer: - Indoor floor (Open office) , 120m x 50m |
| ISD | 20m (Equivalent to [12] TRxPs per 120m x 50m) |
| BS antenna elements | Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements NOTE3 | Around 2 GHz: Up to 8 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 16 Tx and Rx antenna elements Around 15 GHz: TBD Around 30 GHz: TBD |
| User distribution and UE speed | 100% Indoor, 3km/h, [10, 30, 50] users per TRxP NOTE4 |
| Sensing target distribution and Sensing target speed | Sensing target distribution and Sensing target speed 100% Indoor, 3km/h [1] sensing target per TRxP. NOTE5 Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI NOTE6. |
| NOTE1: It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations. NOTE2: "DL + UL" refers to either of the following two cases: 1. FDD with symmetric bandwidth allocations between DL and UL. 2. TDD with the system bandwidth used for either DL or UL via switching in time-domain. NOTE3: The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types. NOTE4: [10] users per TRxP is the baseline. Other number of users per TRxP (e.g., 30, 50) is not precluded. NOTE5: Sensing target number per TRxP depends on the use cases and sensing target type. [1] sensing target per TRxP are starting point and other number of sensing targets per TRxP (e.g., 5 or others) is not precluded. NOTE6: For non-full-buffer traffic with QoS requirement, use the extended XR traffic models for the composite requirement evaluations for immersive communication, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use the AI/ML traffic models for AI/ML related capability evaluations. |  |

4.2	Dense urban

The dense urban microcellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 4.2.

Table 4.2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700MHz Around 2GHz Around 4 GHz Around 7 GHz Around 15GHz Around 2 GHz + Around 4 GHz Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700 MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth NOTE1, NOTE2 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 200MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD | Macro: 200m NOTE3 Micro: 3 micro TRxPs per macro TRxP Micro: [100]m All micro TRxPs are all outdoor |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements NOTE4 | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 2 GHz: Up to 8 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 16 Tx and Rx antenna elements Around 15 GHz: TBD Around 30 GHz: TBD |
| User distribution and UE speed | Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP [10, 30, 50] users per TRxP with single-layer only NOTE5. 80% indoor (3km/h), 20% outdoor (30km/h) NOTE6. |
| Sensing target distribution and Sensing target speed | [10] sensing targets per TRxP NOTE7 UAV is up to 160km/h, Vehicle is up to 60km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI NOTE8. |
| NOTE1: It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations. NOTE2: "DL + UL" refers to either of the following two cases: 1. FDD with symmetric bandwidth allocations between DL and UL. 2. TDD with the system bandwidth used for either DL or UL via switching in time-domain. NOTE3: ISD 200m is the baseline. Other number of ISD (e.g., 250 m) is not precluded. NOTE4: The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types. NOTE5: [10, 30, 50] users per TRxP is the baseline. Other number of users per TRxP (e.g., 20) is not precluded. NOTE6: 80% indoor (3km/h) and 20% outdoor (30km/h) are baseline. 80% indoor (3km/h) and 20% outdoor (3km/h) is optional. Other UE speed is not precluded. NOTE7: Sensing target number per TRxP depends on the use cases and sensing target type. [10] sensing targets per TRxP are starting point. Other number of sensing targets per TRxP (e.g. 20 or others) is not precluded. NOTE8: For non-full-buffer traffic with QoS requirement, use the extended XR traffic models for the composite requirement evaluations for immersive communication, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use the AI/ML traffic models for AI/ML related capability evaluations. |  |

4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 4.3.

Table 4.3: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7 GHz (ISD 1) Around 700 MHz + Around 7 GHz (for ISD 1) |
| Aggregated system bandwidth NOTE1, NOTE2 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m [ISD 3: 7500m assuming 700MHz] |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements NOTE3 | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 2 GHz: Up to 8 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 16 Tx and Rx antenna elements |
| User distribution and UE speed | 50% outdoor vehicles (120km/h), 50% indoor (3 km/h) [10,30, 50] users per TRxP NOTE4 |
| Sensing target distribution and Sensing target speed | [5] sensing targets per TRxP NOTE5 UAV is up to 160km/h, Vehicle is up to 120km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. NOTE6 |
| NOTE1: It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations. NOTE2: "DL + UL" refers to either of the following two cases: 1. FDD with symmetric bandwidth allocations between DL and UL. 2. TDD with the system bandwidth used for either DL or UL via switching in time-domain. NOTE3: The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types. NOTE4: [10, 30, 50] users per TRxP is the baseline. Other number of users per TRxP (e.g., 20) is not precluded. NOTE5: Sensing target number per TRxP depends on the use cases and sensing target type. [5] sensing targets per TRxP are starting point. Other number of sensing targets per TRxP (e.g., 10 or others) is not precluded. NOTE6: For non-full-buffer traffic with QoS requirement, use the extended XR traffic models for the composite requirement evaluations for immersive communication, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use the AI/ML traffic models for AI/ML related capability evaluations. |  |

4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 2 GHz + Around 4 GHz Around 2 GHz + Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 2 GHz + Around 30GHz Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth NOTE1,2 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD | Macro: 500m [TBD on micro layout] |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements NOTE3 | Around 700 MHz: Up to 4 Tx and Rx antenna elements Around 2 GHz: Up to 8 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7GHz: Up to 16 Tx and Rx antenna elements Around 15 GHz: TBD Around 30 GHz: TBD |
| User distribution and UE speed | 80% indoor (3km/h), 20% outdoor(30km/h). NOTE4 [10, 30, 50] users per TRxP NOTE5 |
| Sensing target distribution and Sensing target speed | [5] sensing targets per TRxP NOTE6 UAV is up to 160km/h, Vehicle is up to 120km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. NOTE7 |
| NOTE1: It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations. NOTE2: "DL + UL" refers to either of the following two cases: 1. FDD with symmetric bandwidth allocations between DL and UL. 2. TDD with the system bandwidth used for either DL or UL via switching in time-domain. NOTE3: The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types. NOTE4: 80% indoor (3km/h) and 20% outdoor (30km/h) are baseline. 80% indoor (3km/h) and 20% outdoor (3km/h) is optional. Other UE speed is not precluded. NOTE5: [10, 30, 50] users per TRxP is the baseline. Other number of users per TRxP (e.g., 20) is not precluded. NOTE6: Sensing target number per TRxP depends on the use cases and sensing target type. [5] sensing targets per TRxP are the baseline. Other number of sensing targets per TRxP (e.g., 10 or others) is not precluded. NOTE7: For non-full-buffer traffic with QoS requirement, use the extended XR traffic models for the composite requirement evaluations for immersive communication, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use the AI/ML traffic models for AI/ML related capability evaluations. |  |

4.5	Sub-Urban macro

The sub urban deployment scenario focuses on large and continuous coverage. [The key characteristics of this scenario are continuous wide area coverage supporting pedestrian and vehicles, energy saving, positioning performance.] This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5.

Table 4.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 700 MHz + Around 7 GHz Around 4 GHz +Around 7 GHz Around 2 GHz + Around 7 GHz + Around 30 GHz Around 2 GHz+ Around 700 MHz Around 7 GHz +Around 4 GHz + Around 2 GHz+Around 700 MHz |
| Aggregated bandwidth NOTE1, NOTE2 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer Hex. Grid [Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz ([single layer or two layers])]:TBD |
| ISD | ISD 1: 1299m ISD 2: 1732m |
| BS antenna elements | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements NOTE3 | TBD |
| User distribution and UE speed | 10% Outdoor pedestrian: 3km/h, 10% Outdoor in cars: 40km/h, 80% Indoor in houses: 3km/h [10, 30, 50] users per TRxP NOTE4 |
| Sensing target distribution and Sensing target speed | [5] sensing targets per TRxP NOTE5 UAV is up to 160km/h, Vehicle is up to 120km/h, Human is up to 10km/h. Whether to use or sense environmental object depends on the evaluation methodology. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. NOTE6 |
| NOTE1: It is allowed to simulate a smaller bandwidth than the system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations. NOTE2: "DL + UL" refers to either of the following two cases: 1. FDD with symmetric bandwidth allocations between DL and UL. 2. TDD with the system bandwidth used for either DL or UL via switching in time-domain. NOTE3: The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types. NOTE4: [10, 30, 50] users per TRxP is the baseline. Other number of users per TRxP (e.g., 20) is not precluded. NOTE5: Sensing target number per TRxP depends on the use cases and sensing target type. [5] sensing targets per TRxP are the baseline. Other number of sensing targets per TRxP (e.g., 10 or others) is not precluded. NOTE6: For non-full-buffer traffic with QoS requirement, use the extended XR traffic models for the composite requirement evaluations for immersive communication, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use the AI/ML traffic models for AI/ML related capability evaluations. |  |

< Unchanged parts are omitted >

…

< Unchanged parts are omitted >

## 4.10	Non-Terrestrial Network

Editor note: orbit type and payload type will be captured in section 4.10

This deployment scenario is defined for non-Terrestrial network. Satellite acts as a fill-in especially on roadways and rural/disaster areas where the terrestrial service isn’t available.

Some of its attributes are listed in Table 4.10.

Table 4.10: Attributes for Non-Terrestrial Network

| Attributes | Deployment-1 | Deployment-2 | Deployment-23 |
| --- | --- | --- | --- |
| Carrier Frequency | Around 2 GHz for both DL and UL | Around 14GHz for UL Around 11GHz for DL | Around 20 GHz for DL Around 30 GHz for UL |
| Duplexing | FDD/HD-FDD | FDD/HD-FDD | FDD/HD-FDD |
| Satellite architecture | Bent-pipe On-board Processing | TBD | Bent-pipe, On-Board Processing |
| Typical satellite system positioning in the 6G architecture | Access network | TBD | Access network Backhaul network |
| System Bandwidth (DL + UL) | Up to 2*30 MHz | TBD | Up to 2*400 MHz |
| Satellite Orbit | GEO, MEO, (V)LEO | GEO, MEO, (V)LEO | GEO, MEO, (V)LEO |
| UE Distribution | 100% Outdoors | 100% Outdoors | 100% Outdoors |
| UE Mobility | Fixed, Portable, Mobile | TBD | Fixed, Portable, Mobile |

< Unchanged parts are omitted >

…

< Unchanged parts are omitted >

## 4.XX	UAV

The Unmanned Aerial Vehicle (UAV) scenario is defined to allow for the provision of services for general UAV and Urban Air Mobility (UAMs) to initiate and receive mobile services. The key characteristics of this scenario are deploying dedicated or shared (i.e, for both aerial and terrestrial users) network to support data services for aerial users that might be travelling at high speeds.

Some of its attributes are listed in Table 4.XX.

Table 4.XX: Attributes for Low-altitude Airspace

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency |  |
| System bandwidth |  |
| Layout | Macro layer |
| ISD | ISD1: 200m. ISD2: 500m ISD3: 1732m |
| User distribution and UE speed | Aerial UE altitude: up to [1000] m NOTE1 Aerial UE speed: up to 250kmph Number of total aerial UEs per TRxP is up to [5] NOTE2 |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. |
| NOTE1: The altitude of aerial UEs up to [1000] m is the baseline, other altitude of aerial UEs (e.g., 300 or 600) is not precluded. NOTE2: [5] aerial UEs per TRxP are the baseline, other number of aerial UEs per TRxP (e.g., 15 or others) is not precluded. |  |

< Unchanged parts are omitted >