3GPP TSG RAN Meeting #110                                                                                 RP-253282

Baltimore, USA, December 8-11, 2025

Agenda Item:	 8.2.1.1

Source:	Huawei, HiSilicon, China Unicom, VODAFONE, Telecom Italia

Title:	Consideration on 3GPP Requirements for Coverage

Document for:	Discussion and Decision

# Introduction

In the RAN#108 meeting, a New SID: Study on 6G Radio was approved [1]. Its objectives include the following:

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

In this contribution, we give considerations for coverage requirements for 6GR.

# General consideration: co-site deployment requirement for 6G

In the 6GR SID, coverage enhancement is explicitly required at least in the following objectives:

- Enhanced overall coverage, with focus on cell-edge performance and UL coverage.

- Re-use of existing 5G mid-band (~3.5 GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

To achieve the target from the two objectives, it is critical to understand the implication of “Re-use of existing 5G mid-band site grid” and “targeting comparable coverage to 5G mid-band”.

With respect to “re-use of existing 5G mid-band site grid” and “comparable coverage to 5G mid-band”, this implies that ~7 GHz carrier is assumed to be deployed on the same sites as NR mid-band (e.g., 2.6 GHz and 3.5 GHz), and should provide continuous coverage with an inter-site distance (ISD) similar to that used for 5G mid-band deployments. In addition, ~7GHz has a quite large bandwidth, which is seen as an opportunity to provide better user experience with satisfactory data rates, particularly at the cell edge. In this case ~7GHz can support standalone deployment without relying on other bands, which can ease the deployment and maximize the value of the new band.

In TR 38.914 [2], multiple different deployment scenarios are defined. In terms of ISD, dense urban scenario assumes ISD 200 m and urban macro assumes ISD 500 m. In practice, urban macro with ISD 500 m is one typical deployment scenario for NR mid-band, and to support co-site deployment requirement, this scenario shall be considered as an important reference scenario for ~7 GHz coverage evaluation.

In RAN1#123 meeting, the following agreements have been made for coverage:

Agreement

- For the RAN1 study of “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”,

  - The link budget template candidates 1 and 2 are used to calculate the metric(s) as starting point to compare existing 5G mid-band and 6G deployments in at least around 7 GHz, with potential future update.

  - During initial access/random access

    - Coverage target is referring the bottleneck channel (i.e., Rel-15 NR Msg3) during initial access/random access for existing 5G mid-band

  - FFS target value(s) of data rate for data channels relative to 5G mid-band

  - Following deployment scenarios are considered

    - Urban macro (both O2I and outdoor)

    - Sub-urban macro (both O2I and outdoor)

  - Following carrier frequencies are considered to calculate the metric(s)

    - [4 GHz] as the existing 5G mid-band

    - 7 GHz as 6G deployment

  - Template in R1-2509615 is to be used for collecting inputs on the values from companies.

The above requirements indicate that both channels for initial access and data transmission need to provide continuous coverage to enable co-site deployment with 5G mid-band. In addition, consideration of Urban macro and sub-urban macro deployment scenarios indicate that different ISD values, e.g. 500 m or even larger, need to be addressed during evaluation.

Therefore, it is proposed to capture the below requirements in TR 38.914:

Proposal 1: To support co-site deployment with NR mid-band, ~7GHz shall provide continuous coverage with ISD similar to those used for 5G mid-band deployments. The continuous coverage includes initial access and data channels with a given data rate. The corresponding design should be scalable to accommodate a range of deployed 5G mid-band spectrum, e.g. 2.6GHz, 3.5GHz.

At last plenary it was expected that the specific coverage target will be decided at future plenary meetings. According to the latest agreement from RAN1, the link budget template candidates 1 and 2 are used. Details of candidate 1 and 2 are listed as below:

- Candidate 1: the link budget template from TR 38.830, which defines three metrics–MCL, MIL, and MPL–as follows

MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (component 2). MIL = MCL – Tx loss – Rx loss + gNB antenna gain (component 3 + 4) + UE antenna gain. MPL = MCL – Tx loss – Rx loss + gNB antenna gain (component 3 + 4) + UE antenna gain – Shadow fading margin + BS selection/macro-diversity gain – Penetration margin + Other gains.

- Candidate 2: the link budget template from TR 38.913, which defines a metric MaxCL, as follows.

MaxCL = Total transmit power – Receiver sensitivity.

The template for candidate 1 and 2 consists of multiple parameters and need further discussion in RAN1 to setup common assumptions. Before having the common understanding of the assumptions, the specific coverage target will be premature to be set. Therefore, it is proposed to further discuss coverage target and evaluation template in RAN1.

Proposal 2: Specific coverage target and evaluation template are to be discussed and decided by RAN1.

# Conclusions

According to the above discussions, we have the following proposals:

Proposal 1: To support co-site deployment with NR mid-band, ~7GHz shall provide continuous coverage with ISD similar to those used for 5G mid-band deployments. The continuous coverage includes initial access and data channels with a given data rate. The corresponding design should be scalable to accommodate a range of deployed 5G mid-band spectrum, e.g. 2.6GHz, 3.5GHz.

Proposal 2: Specific coverage target and evaluation template are to be discussed and decided by RAN1.

# Text proposal to 38914

-----------------------------------------Text proposal------------------------------------------------------

# Requirements and key technical principles

## Key performance indicators

Editor note: More KPIs can be added

### 5.1.x	Coverage

To support co-site deployment with NR mid-band, ~7GHz shall provide continuous coverage with ISD similar to those used for 5G mid-band deployments. The continuous coverage includes initial access and data channels with a given data rate. The corresponding design should be scalable to accommodate a range of deployed 5G mid-band spectrum, e.g. 2.6GHz, 3.5GHz.

# References

RP-251881, “New SID: Study on 6G Radio”, 3GPP TSG RAN meeting #108, Prague, Czech Republic, June 9-13, 2025.

RP-252882 TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements.

R1-2508198, FL summary#5 on overview of 6GR air interface, Moderator (NTT DOCOMO), Prague, Czech Republic, October 13-17, 2025

R1-2508733, Overview of 6GR air interface, Huawei, HiSilicon, Dallas, USA, November 17 – 21, 2025