3GPP TSG RAN Meeting #110					                      RP-253287

Baltimore, US, December 08-11, 2025

Agenda item:	8.2.1.4

Source:	Huawei, HiSilicon

Title:	Text proposal for Immersive communication KPIs

Document for:	Discussion

Text proposal for TR 38.914

According to the description of our accompanying paper [1], different levels of composite requirement should be supported for different immersive services. We propose to modify Section 5.1.16 Composite requirement of TR 38.914 as follows.

### 5.1.16	Composite requirement

Editor note: The text captured in RAN#109 is based on ITU-R WP5D and is for ITU-R evaluation; 3GPP internal evaluation metrics will be captured in future meetings.

The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per TRxP.

This requirement is defined as the number of users per TRxP of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface.

The minimum performance requirement of composite requirement is summarized in Table 5.1.16-1.

Table 5.1.16-1: Composite requirements for Immersive Communication services

| [Test environment] | Data rate (Mbit/s) | Latency (ms) | Packet success probability | Number of users (TRxP) |
| --- | --- | --- | --- | --- |
| Dense Urban - Immersive Communication | DL = 30 Mbps/UL=10 Mbps | DL=10 ms / UL=30 ms (or DL+UL total 40 ms) | 99% | 6 |
| UL |  |  |  |  |
| Dense Urban - AI robots with 6*1080 video uploading | UL = 20 Mbps | UL = 15 ms | 99% | ~30/TRxP （ISD 200m） |
| Dense Urban - AI robots with 4*1080 + 2*4K video uploading | UL = 60 Mbps | UL = 15 ms | 99% | ~6/TRxP （ISD 200m） |
| Dense Urban –AI robots with 2*1080 + 4*4K video uploading | UL = 100 Mbps | UL = 15 ms | 99% | ~2/TRxP （ISD 200m） |
| DL |  |  |  |  |
| Dense Urban - 8K immersive gaming | DL = 100 Mbps | DL = 10 ms | 99% | ~30/TRxP （ISD 200m） |
| Dense Urban - 12K immersive gaming | DL = 300 Mbps | DL = 10 ms | 99% | ~6/TRxP （ISD 200m） |
| Dense Urban - 16K immersive gaming | DL = 500 Mbps | DL = 10 ms | 99% | ~2/TRxP （ISD 200m） |

References

- RP-253286 Consideration on supporting immersive communication services.