3GPP TSG RAN Meeting #110											RP-253299

Baltimore, US, December 8-11, 2025

Agenda Item:	8.2.1.4 Requirements of new and existing services

Source:	Robert Bosch GmbH, BMW AG, Volkswagen AG, Toyota ITC, Fraunhofer IIS

Title:	Views on New and Existing Services for Vehicular Use Cases

Document for:	Discussion/Decision

# Introduction

This contribution proposes the introduction of a dedicated service category for vehicular UEs within the 6G RAN-Led SI TR 38.914 [1]. The evolving Connected and Automated Vehicles (CAVs), during the 6G timeframe, will introduce various requirements that cannot be adequately addressed by service categories designed for handheld/smartphone UEs. Thus, we outline the rationale and key requirements for establishing "Vehicular Services" in TR 38.914 to ensure 6G is future-proof for the automotive industry.

In RAN#109, the following text for RAN-led SI TR 38.914 was agreed and captured: Mobile Broadband, Immersive Communication, Massive Communication (IoT), Sensing, AI, Voice, and Regulatory Services [1]. Additionally, the following proposal was captured in the Moderator's summary for the RAN-led 6G SI: Requirements of new and existing services [2]

Moderator proposal 3: Whether to capture the following into TR 38.914 section 5.4 will be further discussed:

- [Non-Terrestrial Network]

- [Aerial]

- [FWA]

- [Vehicles]

- [Mission Critical Communication]

- [Positioning and/or Navigation]

- [TSN]

- [HRLLC]

# Vehicular Services for 6G

From the automotive point of view, 6G is seen as a smooth evolution of 5G, while additionally introducing new features for evolving vehicular use cases and services. Thus, it is expected that 6G will not only support all baseline services from 5G/5G-Advanced—ensuring high-availability for voice, emergency calls, and continuous telematics—but also introduce new capabilities for new and evolving services. Specifically, future Connected and Automated Vehicles (CAV) are envisaged to have a high level of automated driving systems assisted by seamless, reliable, and low latency 3GPP communication. Additionally, the future vehicles will experience increasing data rate demands, particularly for UL, for evolving AI-powered services. Moreover, the evolution of advanced in-vehicle infotainment requires immersive connectivity, enhancing the passenger experience while moving at high speed. Lastly, following the 5G-Advanced emerging trend, vehicular services need to be supported by seamless TN/NTN integration guaranteeing minimum interruption time and ubiquitous coverage.

To address the specific needs of the automotive domain, we propose that the 6G framework for Vehicular Services be defined by the following key services. These services build upon the general service categories identified in TR 38.914 but are specified here with the unique requirements and context of a vehicular UE.

Sections below are potential services required by automotive sectors in the IMT-2030 timeframe.

2.1 Basic Vehicular Services:

The 6G system shall support basic vehicular services such as voice, regulatory services (e.g., emergency calls), and telematics and to be supported from Day-1 of 6G deployment. The 6G system shall ensure service continuity for connected vehicles, supporting seamless mobility with minimal interruption time, both within 6G coverage areas (intra-6G) and between 5G and 6G coverage areas (inter-RAT).

  - Typical use case(s)/services examples:

    - Basic vehicular services include voice/IMS services, telematics data communication, emergency calls/messaging/etc., and other regulatory services.

  - Required vehicular deployment scenario(s)

    - Basic vehicular services support and basic vehicular service continuity shall be supported in all automotive deployment scenarios

  - Mobility condition(s):

    - Support of basic vehicular services and their continuity shall be supported in all mobility conditions

2.2 Enhanced Mobile Broadband/Immersive User Experience

This service addresses the evolution of in-vehicle infotainment beyond current 5G capabilities. The system shall support high data rates to enable advanced mobile broadband and immersive communication for in-vehicle communication and infotainment systems. Additionally, the 6G Radio (6GR) and 6G RAN architecture design shall enable a reduced user- and control-plane latency to enhance the user experience in vehicles moving at low, moderate and high speeds.

  - Typical use case(s)/services examples:

    - Enhanced Infotainment: Delivery of multi-stream, high-resolution content (e.g., 8K video, AR overlays) to the cockpit displays.

    - Passenger/Co-driver Entertainment: providing a high capacity for mobile hotspot within the vehicle, capable of supporting XR services and cloud gaming.

  - Vehicular deployment scenario(s)

    - The services should be available in different coverage scenarios, e.g., from urban-dense to rural scenarios.

    - Lower data rates are sufficient for some difficult scenarios, e.g., underground garage.

  - Mobility condition(s):

    - To be supported in different mobility profiles including slow, medium, and high speeds.

2.3 AI-driven Vehicular Services

For 6G “network for AI” aspects, the 6GR and 6G RAN architecture shall be designed to enable both automotive AI/ML services offloading and the transfer of their associated data, e.g., data collection, real-time inference, model transfer, etc. The 6GR and 6G RAN architecture shall provide an optimized data service for automotive distributed AI requirements, including high-throughput uplink and low-latency in different vehicular scenarios and speeds.

  - Examples:

    - Connected Automated Driving (AD): where (partially or all) AD AI functionalities (e.g., parking, supported maneuvering) are offloaded to the cloud

    - Intelligent and AI-powered Cockpit: where user experience is augmented by offloaded AI functionalities to 6G network or 3rd party cloud(s).

  - Required vehicular deployment scenario(s)

    - AI-driven data services should be supported in different automotive deployment scenarios (e.g., urban-dense, urban, highway, and rural scenarios).

    - It is also expected that some non-real time/offline AI services (e.g., model monitoring, data collection for training) shall also be supported in more difficult coverage conditions (e.g., extended/extreme coverage, NTN coverage (especially for narrow band operations), and indoor scenarios (e.g., covered/underground garages))

  - Mobility condition(s):

    - Based on the functionality and its requirements, automotive distributed AI shall be supported in different mobility scenarios, ranging from low- to high-speed.

2.4 Ubiquitous Connectivity via TN/NTN Integration:

For vehicle communication, integrated TN and NTN architecture is crucial for guaranteeing ubiquitous coverage. Therefore, the 6G shall support seamless mobility and session continuity for vehicular UEs switching between TN and NTN.

  - Examples:

    - Support connectivity use cases [4] in areas with limited terrestrial coverage, e.g., voice call.

  - Required vehicular deployment scenario(s)

    - 6G ubiquitous connectivity services shall be supported in different deployment scenarios.

  - Mobility condition(s):

    - 6G ubiquitous connectivity services shall be supported in all mobility conditions.

2.5 High-Reliability, Low-Latency, and Resilience

To address safety-critical communication for, e.g., autonomous driving connected services, the 6GR and 6G RAN architecture shall be designed to provide high-reliability and low-latency vehicular communication in different automotive scenarios and speeds (using 5G URLLC requirements as a baseline).

Additionally, the 6G Radio and 6G RAN architecture shall be designed to support enhanced resilience mechanisms (to minimize interruption) and high redundancy (to guarantee availability) of automotive connected services.

  - Typical use case(s):

    - Connected Autonomous Driving (AD) Service: this requires high reliability, low latency, and high availability to support the advanced driving capabilities, such as assisting driving, assisted parking and vehicle remote control.

  - Vehicular deployment scenario(s)

    - 6G high-reliability, low-latency, and resilience capabilities shall be supported for different automotive deployment scenarios (e.g., urban-dense, urban, highway, and rural scenarios)

  - Mobility condition(s):

    - High-reliability, low-latency, and resilience should be supported in all mobility conditions ranging from low- to high-speed.

Note: The requirements above are for vehicular Uu communication (V2N). For vehicular use cases that require direct communication (i.e., V2V), automotive plans to utilize existing solutions.

Observation 1: The proposed 6G vehicular services impose diverse requirements that needs to be met across various automotive deployment scenarios and mobility conditions.

Proposal 1: 6GR and 6G RAN architecture should be designed to enable a native support of the vehicular services and requirements from 6G Day 1, including:

- Basic and regulatory services shall be enabled with continuity in different mobility scenarios, i.e., intra-6G and inter-RAT.

- Enhanced mobile broadband/immersive in-vehicle communication for immersive infotainment and enhanced user experience

- AI-driven vehicular services for connected/offloaded AI functionalities by enhancing the data services of 6G

- Ubiquitous Connectivity allowing seamless TN/NTN integration in 6G

- High-reliability, low-latency, and resilience for safety-critical vehicular services such as connected autonomous driving functions

Proposal 2: TR 38.914 should consider the discussions in Section 2.0 and the associated services in subsections 2.1-2.5 when drafting the Chapter of new and existing service requirements.

# Conclusion

In this contribution, we provide automotive views on 6G requirements and services that need to be supported from Day 1, with the following observation and proposal:

Observation 1: The proposed 6G vehicular services impose diverse requirements that needs to be met across various automotive deployment scenarios and mobility conditions.

Proposal 1: 6GR and 6G RAN architecture should be designed to enable a native support of the vehicular services and requirements from 6G Day 1, including:

- Basic and regulatory services shall be enabled with continuity in different mobility scenarios, i.e., intra-6G and inter-RAT.

- Enhanced mobile broadband/immersive in-vehicle communication for immersive infotainment and enhanced user experience

- AI-driven vehicular services for connected/offloaded AI functionalities by enhancing the data services of 6G

- Ubiquitous Connectivity allowing seamless TN/NTN integration in 6G

- High-reliability, low-latency, and resilience for safety-critical vehicular services such as connected autonomous driving functions

Proposal 2: TR 38.914 should consider the discussions in Section 2.0 and the associated services in subsections 2.1-2.5 when drafting the Chapter of new and existing service requirements.

# References

## [1] TR 38.914, Study on 6G Scenarios and requirements, v 0.2.0, TSG-RAN, Oct., 2025.

## [2] RP-252814, Moderator's summary for RAN led 6G SI: Requirements of new and existing services, CMCC, RAN#109, September, 2025

## [3] RP-251973, Text Proposal for TR 38.914 for Architecture Requirements of Local Networks in 6G, Bosch et al., 3GPP TSG RAN Meeting #109, Bengaluru, India, Sept., 202

## [4] 5GAA Technical Report, “Maximising the benefit of future satellite communications for automotive,” Link, September 2024.