3GPP TSG-RAN Meeting #110			RP-253304

Baltimore, US, December 08-11, 2025

Source:	Sony

Title:	6G minimum spectrum allocation and bandwidth

Agenda Item:	8.2.1.3.1

Document for:	Discussion and decision

# 1	Background

In RANP#109, the following proposals in [3] were endorsed:

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

This document consider the minimum spectral allocation that 6GR should operate in considering the bandwidth of existing allocated bands in NR and also based on the observation that not all of the bandwidth allocated to an operator may be available for 6GR, since some of the spectrum may be allocated to a different RAT semi-statically.

The minimum spectral allocation for 6GR also provides a motivation for the minimum maximum bandwidth that the UE must support, since the 6GR design needs to be able to operate in this minimum spectral allocation anyway.

# 2	Bandwidth

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

The following editor’s note has also been endorsed for TR38.914.

### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

2.1 Minimum spectral allocation

Companies have been invited to contribute on the minimum spectral allocation for 6G. The following aspects need to be considered when considering the minimum spectral allocation for 6G:

- Current spectrum aspects of MNOs and private operators

- Potential future spectrum availability

- The amount of spectrum that is actually available for 6G services in the case that spectrum is semi-statically allocated to another RAT (such as NB-IoT or eMTC)

The following bands support a 3MHz spectral allocation: n5, n12, n26, n28, n31, n72, n85, n87, n88, n100, n106, n110 [5]. Whether these bands are to be re-farmed for 6G operation is up to operator decision.

6GR is being designed for the timeframe of 2030 onwards. While 3GPP knows about spectrum that is available today, we have less visibility of additional spectrum that may become available in the coming decade and beyond. We hence think that it would be good if 6GR were prepared to operate in diverse spectrum allocations (including both wide spectrum allocations and narrower spectrum allocations).

An operator may semi-statically configure some spectrum to support coexistence with NB-IoT and eMTC. These technologies are preferably operated in sub 1GHz spectrum, where many spectrum allocations have a bandwidth of 5MHz, while others have a wider bandwidth. If some of that spectrum is semi-statically configured for 4G-IoT technologies, the remaining spectrum for 6G operation will be less, as shown in Figure 1.

Figure 1 - Residual spectrum for 6GR once spectrum is semi-statically configured for 4G-IoT

Based on the considerations above, it is proposed that 6GR operates in a minimum spectral allocations of 3MHz.

Proposal 1: 6GR supports a minimum spectral allocation of 3MHz.

2.2 Smallest maximum supported RF and BB UE BW

At RANP#109, RAN1 was requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. We understand that this doesn’t preclude discussion on the smallest maximum supported RF and BB UE BW in RAN plenary.

In our companion contribution [4], we explain the motivation for supporting a lowest tier device that does not require band specific RF filters (i.e.  no SAW or BAW filters are required in the lowest tier device). Such a device has the following benefits:

- enables a true single stock-keeping unit (SKU) design for global operations

- improved power efficiency

- significantly reduced module size.

- lower overall BoM associated with the module.

- greater longevity of the module

We have provided potential solutions to enable a such a design in [4]. These solutions are copied in the appendix of this document. While we are open to any reasonble approach to enabling such a SAWless device, one possible solution is to support an UL bandwidth of 3MHz for the lowest tier device type. Note that we are NOT proposing support of a DL bandwidth of 3MHz: a SAWless device can support a DL bandwidth of 5MHz, 10MHz or 20MHz.

Observation 1: One solution for supporting a SAWless device is to restrict the UL bandwidth to 3MHz.

Observation 2: A SAWless device supporting an UL bandwidth of 3MHz does not in any way impact DL bandwidth of the SSB, SIB, CORESET#0 or other downlink channels.

We think that it is possible to support a SAWless device with an asymmetric UL / DL capability of 3MHz UL and [5-20] MHz DL.

Proposal 2: As one potential solution for supporting a lowest tier SAWless device, the 6GR study considers support of a device with 3MHz UL bandwidth and [5-20] MHz DL bandwidth.

# 3	Conclusion

In this contribution we considered the minimum spectral allocation and minimum maximum UE supported bandwidth. We make the following observations and proposals:

Proposal 1: 6GR supports a minimum spectral allocation of 3MHz.

Observation 1: One solution for supporting a SAWless device is to restrict the UL bandwidth to 3MHz.

Observation 2: A SAWless device supporting an UL bandwidth of 3MHz does not in any way impact DL bandwidth of the SSB, SIB, CORESET#0 or other downlink channels.

Proposal 2: As one potential solution for supporting a lowest tier SAWless device, the 6GR study considers support of a device with 3MHz UL bandwidth and [5-20] MHz DL bandwidth.

# References

Recommendation ITU-R M.2160-0: Framework and overall objectives of the future development of IMT for 2030 and beyond, ITU-R, November 2023.

RP-252912, “Study on 6G Radio,” 3GPP TSG RAN Meeting #109, Beijing, September 2025.

RP-252873. “Moderator’s summary on 6G diverse device types (incl. minimum UE bandwidth). Apple (moderator). RANP#109. Beijing. September 2025.

RP-253235. “6G massive IoT requirements”. Sony, Nordic Semiconductor, AT&T, Semtech. RANP#110, Baltimore. December 2025.

TS38.101-1. “UE radio transmission and reception”.

# Appendix A: Potential Solutions to Enable a Full SAW-less HD-FDD design

As illustrated in Fig. 1, the main technical challenge to enable a full SAW-less HD-FDD design is to avoid the high order IMD components, especially IMD 7, falling into the protected bands and exceeding the emission limit. Therefore, to resolve the issue, there one of multiple solutions can be adopted in the 6G design to enable a full SAW-less design for 6G massive IoT.

Solution 1: Since the issue is most severe when the device is configured on the edge of the transmission bandwidth that is near the protected band, the most straightforward solution would be avoid configuring the UE in such an allocation as illustrated in Fig. 3. Based on our initial assessment based on the 5G NR waveform, allocating the UE around 2-3 MHz away from the lower edge should be sufficient

Fig. 3 Allocation restriction based approach to enable a full SAW-less implementation

Solution 2: A more elegant approach would be limiting the UE transmission bandwidth to a relatively smaller number, e.g., 3MHz, when the UE is configured on the edge that is near to the protected band. In such a configuration, the corresponding IMD region will also be narrower which can effectively avoid the IMD7 component falling into the protected band. Theoretically, such a reduced UE transmission bandwidth can be either configured as RF bandwidth or baseband bandwidth, where the corresponding solutions are shown in Fig. 4 (a) and (b), respectively.

(a)

(b)

Fig. 4 (a) RF bandwidth limitation and (b) BB bandwidth limitation-based approach to enable a full SAW-less implementation

Solution 3: As a final approach, reducing the output power when the UE is configured on the edge that is near to the protected band as shown in Fig. 5. Based on the assessment of a few commercial PAs in the market, about 2-3 dB power backoff may be needed to enable full SAW-less implementation.

Fig. 5 Power backoff based approach to enable a full SAW-less implementation