3GPP TSG RAN Meeting #110		RP-253319

Baltimore, USA, December 8th-11th, 2025

Source: 	Apple

Title:                     Other Operational requirements

Agenda item:	8.2.1.3.3

Document for:	Discussion and Decision

# 1 Introduction

This document discusses the additional operational requriements for 6GR, with a focus on the enhanced UE capabilities framwork and the key aspects that should be prirotized.

# 2.	Discussions

## 2.1 	UE Capability Report Framework

For 6GR, a more efficient and unified mechanism for dynamic UE capability updates is essential to avoid forcing UEs into the RRC_IDLE state whenever UE capability changes occur.

In 5G NR, UE capabilities are stored in the AMF after initial reporting and can only be modified through a new registration procedure; when this occurs during RRC_CONNECTED, the UE must transition to IDLE, causing significant latency and performance degradation—an issue already recognized during the 5G design phase. Although UAI signaling was introduced in 5G NR to address this issue and allow capability adjustments while the UE stays in RRC_CONNECTED, its applicability is limited and, more importantly, it is often not promptly handled by the network—or not configured at all. As a result, UAI has been considerably less effective in practical 5G deployments.

Dynamic UE capability reporting is essential for several key scenarios:

- Multi-SIM (MUSIM) operation: In MUSIM devices, RF and baseband resources maybe shared across SIMs, meaning capabilities that are static in single-SIM cases—such as maximum supported layers, number of CA carriers/RF chains, or achievable throughput—can vary depending on activity on the other SIM. For instance, when one SIM becomes activated e.g., VoIP call, the UE may need to reduce supported layers or carriers on the second SIM to prevent resource contention and maintain stable performance. Without timely capability updates, the network may rely on outdated assumptions, resulting in inefficient scheduling, degraded performance, or even service disruptions. Dynamic UE capability update enables the network to adjust resource allocation intelligently, ensuring optimal operation across both SIMs while respecting the UE’s dynamic hardware constraints.

- Other use cases: Dynamic UE capability update is also essential for managing overheating and power saving functions identified in 5G NR because the UE’s operational limits are not static—they vary depending on thermal conditions and energy constraints. When a device approaches thermal thresholds or activates power-saving modes, its effective capabilities, such as supported CCs, maximum MIMO layers, or throughput, must be dynamically adjusted and communicated to the network.

To support the current 5G NR and emerging use cases in 6GR, it is therefore vital to establish a dynamic UE capability update framework in 6G Day-1 that enables timely, flexible, and reliable UE’s capability modifications without interrupting RRC_CONNECTED operation.

Proposal 1: An enhanced UE capability report framework should be specified as 6G Day-1 operational feature, allowing the UE to dynamically update its capabilities in the RRC_CONNECTED state.

## 2.2	IoDT Capabilities

A key lesson from 5G NR that should be lesson-learn by 6G design is that several features defined as mandatory for the UE were never fully commercialized by networks. The following is two mandatory 5G NR UE features examp that were not commercialized despite being specified, resulting in wasted implementation effort:

- Example #1: The use of 15 kHz SCS for FR1 TDD and 30 kHz SCS for FR1 FDD was defined as “mandatory without capability signaling,” meaning all UEs were required to support these numerologies even though no corresponding capability indication was defined. In practice, however, these configurations were not adopted in commercial deployments, leaving the UE-side investment unused.

- Example #2: The features FG8-8 and FG8-9, which define two closed-loop power-control mechanisms for PUSCH and PUCCH, were specified as “mandatory with capability signaling.” Despite explicit capability definitions and implementation by UE vendors, networks did not deploy these power-control schemes commercially

This mismatch led to significant underutilization of the engineering effort and hardware resources invested by UE vendors, as capabilities implemented to meet specification requirements delivered little to no practical value in real deployments. For 6GR day-1 operational features, it is essential that  mandatory UE features translate into widely supported, commercially deployed functionality, rather than remaining unused or unrealized in practice

Proposal 2: A general rule shall be defined to ensure that 6GR day-1 mandatory UE features are IoDTed and deployed in commercial network (e.g., by making any feature that is mandatory for the UE equally mandatory for the network)

## 2.3	Capabilities for Device Types

Device types can be established around a core set of mandatory UE capabilities, as shown in Table 1, thereby reducing the size of capability reports. Optional or additional capabilities may then be layered onto these base device types, providing flexibility while maintaining interoperability. To further optimize capability management, feature sets can be defined as logical groupings of related UE capabilities that extend or enhance a given device type. For instance, Feature Set 1 could include capabilities 1–5, while Feature Set 2 covers capabilities 6–10. This structured approach enables networks to quickly interpret a device’s functional profile, simplifies capability updates, and ensures efficient support for both current and future device categories.

Table 1 – Examples of Device type specific UE capability set

| Device Type | Description | UE Capability Feature Sets |
| --- | --- | --- |
| 1 | eMBB smartphone, low-end | Basic eMBB capability, Peak data rate =X1 |
| 2 | eMBB smartphone, normal | Basic eMBB capability, Peak data rate =X2 |
| 3 | RedCap wearable, low-end | Basic RedCap capability, BW=Y1 |
| 4 | RedCap wearable, high-end | Basic RedCap capability, BW=Y2 |
| 5 | … | … |

Proposal 3: A device type is determined by a core set of mandatory UE capabilities, with any additional UE capabilities serving as complementary features that build upon the mandatory features.

# 3.	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: An enhanced UE capability report framework should be specified as 6G Day-1 operational feature, allowing the UE to dynamically update its capabilities in the RRC_CONNECTED state.

Proposal 2: A general rule shall be defined to ensure that 6GR day-1 mandatory UE features are IoDTed and deployed in commercial network (e.g., by making any feature that is mandatory for the UE equally mandatory for the network).

Proposal 3: A device type is determined by a core set of mandatory UE capabilities, with any additional UE capabilities serving as complementary features that build upon the mandatory features.