3GPP TSG RAN Meeting #110                                                                         RP-253328

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.4

Source:	Lenovo

Title:	On ISAC Scenarios and Requirements

Document for:	Discussion and decision

# Introduction

The new study item on 6G Radio was approved in RAN#108 [1], where the study on Sensing was agreed with the objectives cited as following:

(9) Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914]

a) PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4]

b) Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1]

c) Aspects of integration with communication services [RAN1]

d) higher layer procedures and protocol aspects [RAN2]

e) RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4]

Initial agreements have been made in RAN#109 on set of metrics and service requirements. In this contribution, we provide further views on considerations related to sensing scenarios, use-cases and performance requirements.

# Discussion

## Sensing modes

In RAN#109, it has been agreed to study the 6 sensing modes, including TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE monostatic sensing modes. Furthermore, it has been suggested to consider cooperative multi-static sensing scenarios [3]

Proposal 2-2-2: 6GR should study cooperative sensing/multi-static sensing mechanism involving multiple UEs and base stations.

In our view, the multi-static sensing operation is of high interest, given the possibility of re-using sensing signal for multiple measurements, leading to increased energy efficiency and measurement diversity/accuracy. Moreover, it opens a nice synergy among TRP/network-based sensing operations as well as UE assistance, when available. From another perspective, while sensing primarily considers a target as a physical object without a UE identity/function, sensing operation may also be applied to the physical objects associated (e.g., attached to) a UE device, which may benefit mutually, i.e., sensing may benefit from the additional information and/or measurements provided from the UE device, and at the same time, the UE benefits from the sensing input via monitoring/tracking environment and UE mobility.  As such, it is motivated that 6GR considers multi-static sensing modes, including both cases where sensing target is not associated with a UE entity as well as the case where sensing target is associated (e.g., attached) to a UE.

Observation  1. Cooperative and multi-static sensing operation facilitates re-using of the same sensing signal transmission for multiple measurements, leading to increased energy efficiency and measurement diversity/accuracy

Observation  2. While sensing primarily considers target as a physical object without a UE identity/function, sensing operation may also be applied to the physical objects associated (e.g., attached to) a UE device, with mutual benefit between sensing, positioning and communications.

Proposal 1. 6GR to study cooperative and multi-static sensing modes

Proposal 2. 6GR to study both sensing scenarios where sensing target is not associated with a UE entity as well as the case where sensing target is associated (e.g., attached) to a UE.

## Sensing scenarios

Several deployment scenarios have been defined in TR 38.914 for baseline 6GR evaluation, including Urban Macro, Dense Urban, Rural, Indoor Hotspot, Non-Terrestrial Network, Sub-Urban macro, Urban coverage for massive connection, Air-to-Ground Scenario, Non-Terrestrial Network. For sensing, it preferred to follow the same underlying communication scenario assumptions & parameters as baseline, including, e.g., carrier frequencies, system bandwidth, layout (cell/sector definition), ISD, (mechanical and electrical) tilt, antenna configurations (UE and BS), UE dropping, traffic pattern, etc. This is motivated to comply with the motivation and functionality of integrated communication and sensing operation, i.e., re-use of the deployments for both communication and sensing, without imposing additional sensing specific deployment costs. Furthermore, from the co-existence perspective, the performance evaluation of either communication or sensing needs to consider the radio resource multiplexing scheme and any potential interference and/or mutual synergy between them.

Observation  3. It is motivated to re-use the parameters defined for the deployment scenarios of the underlying communication scenarios for defining deployment scenarios for sensing, both to avoid sensing-specific deployments as well as to evaluate mutual impact (interference or constructive use) between sensing and communication transmissions

Proposal 3. Re-use the same scenario assumptions & parameters to define sensing deployment scenarios, including at least carrier frequencies, system bandwidth, (cell/sector) layout, ISD, antenna configurations (UE and BS), UE dropping, communication traffic model

Nevertheless, additional sensing-related definitions are required, which can be added on top of each deployment per-sensing use-case/scenario and/or per-target type. The additional sensing-specific parameters include at least the following:

- TRPs and UEs operating as Sensing Tx/Rx node pairs [the sensing Tx/Rx nodes can be pre-selected as part of the scenario definition or selected from the dropped/defined nodes within a scenario]

- Target related parameters, including target type, physical characteristics (e.g., RCS, site), 3D distribution, 3D mobility pattern, 3D orientation, LOS/NLOS condition,

- Channel modelling features: in some scenarios, some of the channel behaviour or modelling elements are necessary for specific sensing evaluations and may not be treated as optional, as (unlike communications) they impact the sensing performance considerably, e.g., spatial consistency modelling, blockage modelling, low-power cluster, ground reflection, etc. Such effects can be defined as part of the deployment or assumptions for each sensing scenario.

- Known/deterministic environment objects which can be considered as part of a deployment scenario when available, e.g., EO type-2s and EO type-1s.

Observation  4. Additional sensing-related definitions are required to be added on top of each deployment per-sensing use-case/scenario and/or per-target type to define an evaluation scenario for sensing.

Proposal 4. The evaluation of the sensing performance needs to consider the radio resource multiplexing scheme and any potential interference or mutual impact with communication evaluation.

Proposal 5. RAN to consider sensing Tx/Rx node definition, target related parameters/dropping, known environment objects, when available (e.g., EO type 1/2), and necessary channel features, as additional parameters for defining a sensing scenario.

Proposal 6. Given the applicability and relevance of sensing capability and use-cases for urban scenarios, the scenarios of UMa and UMi to be considered as the evaluation benchmark for outdoor targets.

## Sensing Use-cases and requirements

The following text has been captured in TR 38.914 for defining 6GR sensing capability:

5.1.17 Sensing Sensing-related capabilities are measured in terms of the following technical performance requirements: • Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. • Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error. • Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error. • [Sensing Resolution: TBD]

The above definition of the false-alarm describes the case where a target is falsely detected within an environment where no sensing target exists. Nevertheless, this definition does not directly facilitate a metric for false-alarm/detection when multiple targets may be present at the same time. In particular, in the presence of multiple targets, it is relevant to capture both when a target is detected at the absence of all targets, or at the presence of other targets (e.g., target mismatch due to positioning error). A second type of false-alarm has been defined in the NR-ISAC study for this purpose, and can be re-used for the follow-up 6GR evaluations:

Agreement [NR Rel 20 ISAC evaluation] - False alarm probability is agreed as performance metric for NR ISAC. - For cases without true target dropped in simulation area, False alarm probability Type 1 is computed and reported - For cases with true targets dropped in simulation area, False alarm probability Type 2 is computed and reported - Note: both False alarm probability Types are mandatory - KPI values for False alarm probability Type 1 and 2 can be discussed separately. Definitions: - False alarm probability Type 1 (no target dropped in simulation area): An object is detected when there is no target present in simulation area is considered a false alarm. Pf1=n=0N-1QnN Where, - Qn equal to 1 if at least one object is detected when there is no target dropped in the simulation area in the drop n, otherwise Qn equal to 0. - N is the total number of drops without targets in the simulation area. - False alarm probability Type 2 (targets dropped in simulation area): An object is detected but not associated with any true targets in the simulation area is considered as a false alarm. Pf2=0≤n<NMn'≠0Dn'Mn'K Where, - Dn' is the number of detected objects but not associated with any true targets in the drop n. - Mn' is the total number of detected objects in the drop n. - FFS: - Option 1: K is number of drops (N) - Option 2: K is number of drops with at least one detected object Note: the number of targets should be reported by companies when providing False alarm probability Type 2

Observation  5. The current false-alarm definition does not capture the case where a target is detected on two different instances (e.g., delay taps) due to multi-path or shadow effect of one target, where one (or multiple) target is present

Proposal 7. 6GR to consider definition of false alarm metrics both conditioned to no target being present and in the multi-target scenarios.

Sensing resolution has been defined in 22.137 as the capability to differentiate two concurrently present sensing objects:

- Sensing Resolution describes the minimum difference in the measured magnitude of target objects (e.g., range, velocity) to be allowed to detect objects in different magnitude.

As such, it is an informative capability of a sensing system and may be needed when sensing is performed in the presence of multiple closely located sensing targets, e.g., pedestrians in a pathway, or a group of UAVs. Moreover, while certain resolution metrics can be directly related to system parameters (e.g., range resolution to bandwidth for a link, assuming the signal processing is not utilizing super-resolution methods), such statement is not true for a multi-link sensing system considering a combination of position, velocity as sensing target metrics. As such, it is motivated to not rule-out sensing resolution as an indication of sensing capability/KPI.

Observation  6. Resolution is an informative KPI of a sensing operation and needed when sensing is performed in the presence of multiple closely located sensing targets, e.g., pedestrians in a pathway. Moreover, sensing resolution is impacted by the algorithm complexity and multiple aggregated sensing links and may not be trivially inferred from the system parameters.

Proposal 8. Consider sensing resolution as a metric defining sensing KPIs.

As part of the discussions in RAN#109, it was proposed to continue discussion around the following capability metrics:

## Proposal 1-9: Further discuss the following metrics for 3GPP internal sensing KPIs, also take the conclusion of sensing use cases into consideration - Micro-Doppler accuracy - sensing capacity - sensing service latency - refreshing rate - reconstruction accuracy - Others are not precluded

Regarding the sensing capacity, this our understanding that the number of sensing targets does not effectively define the sensing capacity, as the sensing overhead is not linearly dependent on the number of the present or potential targets. For instance, when the sensing transmission is performed over the potential sensing area, any target within the coverage of the said sensing signal is illuminated and subsequently measured by a sensing entity monitoring the said area of interest. As such, the sensing signal overhead necessary for a successful sensing operation is not directly dependent on the number of targets, but the monitored/supported sensing area. From another perspective, when the environment clutter is removed for the purpose of target detection within a specific area, the same clutter-removal step is valid for all targets within the said sensing area. The remaining overhead, e.g., reporting of additional target-related measurements due to an increment in the number of targets within the same area is negligible compared to the total sensing operation complexity & overhead. As such, it is motivated to either remove the sensing capacity as a sensing capability/KPI or define capacity in terms of the supported sensing areas over which a sensing KPI is expected to be delivered.

Observation  7. The number of sensing targets does not sufficiently define the sensing overhead or needed complexity.

Proposal 9. Remove the sensing capacity as a sensing capability/KPI or re-define sensing capacity in terms of the supported sensing areas over which a sensing KPI is expected to be delivered.

Regarding the sensing latency and refreshing rate, both these metrics are informative KPIs of the 6GR sensing from two aspects. Firstly, they both describe timing requirements for delivery of sensing results, as also captured in the 22.137 sensing service KPI definitions. Although the sensing latency is further dependent in the system operation beyond RAN, it is envisioned that the sensing measurement process to occupy a long time window, which may effectively contribute to the sensing latency (e.g., coherent processing interval of 20-100 msecs are currently considered in the NR ISAC evaluations study). Secondly, the sensing service latency and refresh rate effectively impact the sensing resource overhead, i.e., whether each simulated sensing drop shall deliver the sensing results in 10 or 100 msecs, define the ratio of resources that are consumed by the sensing operation, when sensing operation is present. As such, it is motivated to capture the above metrics as description of the 6GR sensing capability.

Observation  8. Sensing latency and refreshing rate the are both informative metrics of the sensing capability, both in terms of the service KPI, as well as the resulting measurements overhead when sensing operation is present.

Proposal 10. RAN to consider Sensing latency and refreshing rate as metrics defining sensing capability/KPI.

A collection of representative assumptions for the 6G radio evaluations study is provided in the below Table 1, including various use-cases, sensing types and sensing methods. Sensing performance can be quantified at least in terms of estimation accuracy of target position and velocity, sensing time/latency, reliability/service availability, false-alarm (FA) and missed detection (MD) rate of the sensing results over a single sensing target or group of sensing targets. Further categories/levels and target KPI requirements can be considered based on iterations of 6GR evaluations and considering the developed 6G use-cases and system requirements [TR 22.870].

Table 1. Collection of representative assumptions for the 6G radio evaluations study

| Target Types | Sensing type/Use-case | Sensing mode | Deployment Scenario | Target KPIs |
| --- | --- | --- | --- | --- |
| UAV, human outdoor/indoor, vehicles outdoor/indoor, objects creating hazards on roads Human breathing/heart activity | Object detection and tracking (e.g., Proximity sensing Collision avoidance Intrusion detection) Joint sensing and positioning Sensing assisted communication | All modes, including cooperative & multi-static sensing | UMa, UMi (Outdoor) InF (indoor) | Missed detection (<5%) False Alarm Rate (<5%) Reliability/confidence (>95%) Positioning Accuracy [H/V] (<10m) Velocity Accuracy [H/V] (<10m/sec) Range Resolution (<1m) Velocity Resolution (<1m/sec) Latency (< 1 sec) Refreshing rate (< 1 sec) |

Proposal 11. As part of the sensing evaluations, the use-cases of object detection and tracking, sensing-assisted positioning enhancements (sensing of a UE as a passive object), as well as sensing-assisted communication enhancements should be considered.

Proposal 12. RAN to consider adopting the values for the target KPIs as presented in Table 1 for the initial minimum sensing target KPIs. Further categories and target KPI requirements can be considered based on iterations of 6GR evaluations, and the progress of the study of 6G use-cases and system requirements [TR 22.870].

# Conclusion

The main observations and conclusions of this paper is summarized in the following:

Observation  1. Cooperative and multi-static sensing operation facilitates re-using of the same sensing signal transmission for multiple measurements, leading to increased energy efficiency and measurement diversity/accuracy

Observation  2. While sensing primarily considers target as a physical object without a UE identity/function, sensing operation may also be applied to the physical objects associated (e.g., attached to) a UE device, with mutual benefit between sensing, positioning and communications.

Proposal 1. 6GR to study cooperative and multi-static sensing modes

Proposal 2. 6GR to study both sensing scenarios where sensing target is not associated with a UE entity as well as the case where sensing target is associated (e.g., attached) to a UE.

Observation  3. It is motivated to re-use the parameters defined for the deployment scenarios of the underlying communication scenarios for defining deployment scenarios for sensing, both to avoid sensing-specific deployments as well as to evaluate mutual impact (interference or constructive use) between sensing and communication transmissions

Proposal 3. Re-use the same scenario assumptions & parameters to define sensing deployment scenarios, including at least carrier frequencies, system bandwidth, (cell/sector) layout, ISD, antenna configurations (UE and BS), UE dropping, communication traffic model

Observation  4. Additional sensing-related definitions are required to be added on top of each deployment per-sensing use-case/scenario and/or per-target type to define an evaluation scenario for sensing.

Proposal 4. The evaluation of the sensing performance needs to consider the radio resource multiplexing scheme and any potential interference or mutual impact with communication evaluation.

Proposal 5. RAN to consider sensing Tx/Rx node definition, target related parameters/dropping, known environment objects, when available (e.g., EO type 1/2), and necessary channel features, as additional parameters for defining a sensing scenario.

Proposal 6. Given the applicability and relevance of sensing capability and use-cases for urban scenarios, the scenarios of UMa and UMi to be considered as the evaluation benchmark for outdoor targets.

Observation  5. The current false-alarm definition does not capture the case where a target is detected on two different instances (e.g., delay taps) due to multi-path or shadow effect of one target, where one (or multiple) target is present

Proposal 7. 6GR to consider definition of false alarm metrics both conditioned to no target being present and in the multi-target scenarios.

Observation  6. Resolution is an informative KPI of a sensing operation and needed when sensing is performed in the presence of multiple closely located sensing targets, e.g., pedestrians in a pathway. Moreover, sensing resolution is impacted by the algorithm complexity and multiple aggregated sensing links and may not be trivially inferred from the system parameters.

Proposal 8. Consider sensing resolution as a metric defining sensing KPIs.

Observation  7. The number of sensing targets does not sufficiently define the sensing overhead or needed complexity.

Proposal 9. Remove the sensing capacity as a sensing capability/KPI or re-define sensing capacity in terms of the supported sensing areas over which a sensing KPI is expected to be delivered.

Observation  8. Sensing latency and refreshing rate the are both informative metrics of the sensing capability, both in terms of the service KPI, as well as the resulting measurements overhead when sensing operation is present.

Proposal 10. RAN to consider Sensing latency and refreshing rate as metrics defining sensing capability/KPI.

Proposal 11. As part of the sensing evaluations, the use-cases of object detection and tracking, sensing-assisted positioning enhancements (sensing of a UE as a passive object), as well as sensing-assisted communication enhancements should be considered.

Proposal 12. RAN to consider adopting the values for the target KPIs as presented in Table 1 for the initial minimum sensing target KPIs. Further categories and target KPI requirements can be considered based on iterations of 6GR evaluations, and the progress of the study of 6G use-cases and system requirements [TR 22.870].

# Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator).

- RP-252802, 3GPP TSG RAN Meeting #109, Moderator's summary for RAN led 6G SI: Key performance indicators

- RP-252943, 3GPP TSG RAN Meeting #109, Moderator's summary for RAN led 6G SI: Requirements of new and existing services

- TR 22.837: "Study on Integrated Sensing and Communication".

- TS 22.137: "Integrated Sensing and Communication".

- TR 22.870: "Study on 6G Use Cases and Service Requirements"