# 3GPP TSG RAN Meeting #110																		RP-253329

# Baltimore, USA, December 8-11, 2025

Source:	Qualcomm

Title:	Views on Deployment Scenarios

Document for:	Approval

Agenda item:	8.2.1.0

### 1 Introduction

This document includes our views on specific aspects not yet agreed related to RAN deployment Scenarios, namely on UE antenna elements assumptions.

### 2 Discussion and Proposals

Based on the discussion and progress in RAN#109 [2], one pending assumption for most deployment scenarios is related to UE Tx/Rx antenna elements.

The table below captures our proposal, showing UE Tx/Rx assumptions per evaluation carrier frequency (i.e. applicable to all deployment scenarios including that carrier frequency).

Table 1. Proposal on UE Antenna elements assumptions

| Evaluation Frequency | UE (Tx/Rx) | Note/Comment |
| --- | --- | --- |
| 700 MHz | Up to 4/4 | Same as IMT-2020 [1] |
| 4 GHz | Up to 8/8 | Same as IMT-2020 [1] |
| 7HGz | Up to 8/16 | 16 Rx may apply to certain devices, e.g. CPE, Vehicles etc. |
| 30 GHz | Up to 32/32 | Same as IMT-2020 [1] |

The assumptions above represent max Tx/Rx values and should be considered strictly for evaluation purposes, not intended to reflect mandatory or typical UE antenna implementations. Evaluation targets maybe achieved with lower Tx/Rx values.

It should be noted that the above assumptions apply to eMBB/IC and HRLLC scenarios. For massive communication, UE Tx/Rx maybe further limited, e.g. up to 2 Tx and Rx.

### 3 Summary and Conclusions

It is proposed to consider the assumptions above for the Deployment scenarios tables in TR 38.914.

### 4 References

[1] TR 38.913: Study on Scenarios and Requirements for Next Generation Access Technologies

[2] RP-252915: pCR on Deployment scenarios for TR 38.914 (for RAN#110 discussion continuation)