3GPP TSG RAN Meeting #110	             RP-253332

Baltimore, US, December 8-11, 2025

Agenda Item:	8.2.1.1

Source:	Lenovo

Title:	6GR Coverage Target

Document for:	Discussion

# 1	Introduction

In RAN#108, a new SI “Study on 6G Radio” was approved [1]. The study aims to develop a new 6G Radio (6GR) access technology to meet a broad range of use cases, requirements and deployment scenarios as defined in [2]. An overview of the new 6GR interface is provided in [3]. This contribution provides coverage evaluations for device types supporting 5 MHz and 20 MHz bandwidths, as well as for co-sited FR1–FR3 deployments.

# 2	Discussion

In RAN1#122bis, it was agreed to provide coverage analysis to RAN#110.

Agreement

- RAN1 provides methodology and corresponding initial analysis of potentially achievable coverage to RAN#110 to determine the coverage target(s)

The following evaluations assume ideal channel estimation under a TDL-C channel model with a 300 ns delay spread(unless stated otherwise) and a stationary user. The simulations use 5G LDPC coding with a 24-bit CRC and QPSK modulation. The TBoMS technique is included in the analysis, while additional mechanisms—such as inter-slot frequency hopping—may further enhance performance.

## 6GR Device with maximum 5 MHz BW

Cell-edge throughput results for DL and UL at 154 dB MCL are provided in Table 1. It is observed that for 154 dB MCL coverage,

- in UL, to transmit ~ 500 bits payload, ~500 slots needed, and

- in DL, to transmit ~0.5 Mbits-packet, ~ 10000 slots needed.

Table 1:Throughput of 6GR device with 5MHz device BW @ 154 dB MCL, 700 MHz*

| Direction | BW [MHz] | SNR | RB# | Rep# | Thp [kbps] | MCS | TBS | slot# per TB (TBoMS) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL | 5 | -19.57 | 4 | 128 | 0.876 | 0 | 128 | 1 |
| DL | 5 | -14.35 | 24 | 20 | 43 | 2 | 3848 | 4 |

*additional mechanisms—such as inter-slot frequency hopping—may further enhance performance

**further reducing # of PRBs (e.g., from 4 to 1) may not achieve noticeable throughput improvement (see appendix2, Table 6).

Observation 1: For <1 GHz spectrum with 5 MHz device BW at 154 dB MCL, ~0.9 kbps can be achieved with 128 repetitions, and ~43 kbps can be achieved in DL with 20 repetitions.

## 6GR Device with maximum 20 MHz BW

Cell-edge throughput results for DL and UL at 154 dB, and 144 dB MCL are provided in Table 2, and Table 3, respectively.

Table 2:Throughput of 6GR device with 20MHz device BW @ 154 dB MCL, 2 GHz

| Direction | BW [MHz] | SNR | RB# | Rep# | Thp [kbps] | MCS | TBS | slot# per TB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL | 20 | -27.35 | 24 | 512 | 1.4 | 0 | 1480 | 2 |
| DL | 20 | -14.55 | 100 | 16 | 193.6 | 0 | 3368 | 1 |

Table 3:Throughput of 6GR device with 20MHz device BW @ 144 dB MCL, 2 GHz

| Direction | BW [MHz] | SNR | RB# | Rep# | Thp [kbps] | MCS | TBS | slot# per TB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL | 20 | -17.35 | 24 | 64 | 14.8 | 1 | 2024 | 2 |
| DL | 20 | -4.55 | 100 | 2 | 1670 | 0 | 3368 | 1 |

Observation 2: For ~ 2GHz spectrum with 20 MHz device BW,

- at 154 dB MCL, ~1.4 kbps can be achieved with 512 repetitions, and ~194 kbps can be achieved in DL with 16 repetitions, and

- at 144 dB MCL, ~15 kbps can be achieved with 64 repetitions, and ~1.7 Mbps can be achieved in DL with 2 repetitions

Proposal 1: Bottleneck is the UL throughput at 154dB for 5MHz device bandwidth, check whether ~1kbps@154 dB MCL satisfies 6G low-tier IoT requirements before setting up the target.

## 3.5/7.5 GHz co-sited deployment

6GR coverage aspects were discussed during the previous RAN1 and RAN plenary meetings, where it was clarified that the final decision regarding coverage targets will be made by the RAN plenary. One of the objectives of 6GR study is to achieve coverage at least comparable to that of 5G to enable reuse of the existing 5G site grid for 6GR deployments, e.g., 6GR system operating at 7.5 GHz can achieve a comparable coverage of 5G at ~3.5 GHz. For this, coverage evaluation should consider the required updates to system configuration to achieve similar or comparable coverage for both uplink and downlink. This configuration may involve the required number of antennas, number of antenna elements, max Tx power, number of required repetitions, etc.

In this section, we provide some evaluation analysis for both 3.5GHz and 7.5GHz for Msg3 considering the following LLS assumptions.

Table 4: Link level simulation assumptions

| Carrier frequency | SCS | BW | #RBs | MCS | #Tx chains in LLS | Channel | #Repetitions | Payload | Required SNR@10% from LLS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.5GHz | 15kHz | 20MHz | 6 | 0 | 1 | TDL-C@500 | 1,2,4,8,16 | 50bits | 0, -3, -6, -9, -12 |
| 7.5GHz | 30kHz | 100MHz | 6 | 0 | 1 | TDL-C@500 | 1,2,4,8,16 | 50bits | 0, -3, -6, -9, -12 |

The achieved coverage performance—expressed in terms of Maximum Path Loss (MPL), Maximum Coupling Loss (MCL), and Maximum Isotropic Loss (MIL)—for various antenna configurations and Msg3 repetition factors is summarized in Table 3 (based on R1-2509615, see Appendix1). Figure 1 illustrates how MPL scales with the number of repetitions across different antenna array sizes.

Table 5: MPL, MCL, and MIL calculation for different Msg3 repetitions and antenna configurations

| Carrier frequency | BS antenna elements | MPL (dB) @1, 2, 4, 8, 16 repetitions | MCL (dB) @1, 2, 4, 8, 16 repetitions | MIL (dB) @ 1, 2, 4, 8, 16 repetitions |
| --- | --- | --- | --- | --- |
| 3.5GHz | 192 | (144.2, 147.2, 150.2, 153.2, 156.2) | (131.2, 134.2, 137.2, 140.2, 143.2) | (161.05, 164.05, 167.05, 170.05, 173.05) |
| 3.5GHz | 256 | (145.5, 148.5, 151.5, 154.5, 157.5) | (131.2, 134.2, 137.2, 140.2, 143.2) | (162.30, 165.30, 168.30, 171.30, 174.30) |
| 3.5GHz | 512 | (148.5, 151.5, 154.5, 157.5, 160.5) | (131.2, 134.2, 137.2, 140.2, 143.2) | (165.31, 168.31, 171.31, 174.31, 177.31) |
| 7.5GHz | 768 | (147.2, 150.2, 153.2, 156.2, 159.2) | (128.2, 131.2, 134.2, 137.2, 140.2) | (164.06, 167.06, 170.06, 173.06 , 176.06) |
| 7.5GHz | 1024 | (148.5, 151.5, 154.5, 157.5, 160.5) | (128.2, 131.2, 134.2, 137.2, 140.2) | (165.31, 168.31, 171.31, 174.31, 177.31) |
| 7.5GHz | 1536 | (150.2, 153.2, 156.2, 159.2, 162.2) | (128.2, 131.2, 134.2, 137.2, 140.2) | (167.07, 170.07, 173.07, 176.07, 179.07) |

Figure 1: impact of antenna scaling gain on MPL

Observation 3: Msg3 coverage for a typical 3.5 against 7.5 GHz co-sited deployment is achievable.

- Msg3 repetitions not considered in typical 3.5 GHz deployments; Msg3 coverage enhancements achieve ~10 dB improvement.

Proposal 2: Consider +10 dB coverage improvement in 6GR day1.

# 3	Conclusion

This contribution evaluates coverage target for 6G Release 20.

Observation 1: For <1 GHz spectrum with 5 MHz device BW at 154 dB MCL, ~0.9 kbps can be achieved with 128 repetitions, and ~43 kbps can be achieved in DL with 20 repetitions.

Observation 2: For ~ 2GHz spectrum with 20 MHz device BW,

- at 154 dB MCL, ~1.4 kbps can be achieved with 512 repetitions, and ~194 kbps can be achieved in DL with 16 repetitions, and

- at 144 dB MCL, ~15 kbps can be achieved with 64 repetitions, and ~1.7 Mbps can be achieved in DL with 2 repetitions

Observation 3: Msg3 coverage for a typical 3.5 against 7.5 GHz co-sited deployment is achievable.

- Msg3 repetitions not considered in typical 3.5 GHz deployments; Msg3 coverage enhancements achieve ~10 dB improvement.

Proposal 1: Bottleneck is the UL throughput at 154dB for 5MHz device bandwidth; check whether ~1kbps@154 dB MCL satisfies 6G low-tier IoT requirements before setting up the target.

Proposal 2: Consider +10 dB coverage improvement in 6GR day1.

# References

[1] RP-251881, “New SID: Study on 6G”, NTT DOCOMO.

[2] 3GPP TR 38.914, “Study on 6G Scenarios and Requirements”.

[3] R1-2505790, “Overview of 6GR radio interface”, Lenovo.

# Appendix 1: System configuration

The table below contains assumed values for the coverage comparison of 3.5 vs. 7.5 GHz.

| System configuration |  |
| --- | --- |
| Channel for evaluation |  |
| Scenarios and Carrier frequency (GHz) | 3.5 vs. 7.5 |
| BS antenna heights (m) | 25 |
| UT antenna heights (m) | 1.5 |
| Cell area reliability (%) | 90% |
| Lognormal shadow fading std deviation (dB) | 1.5 |
| Tx Diversity |  |
| Number of SSB |  |
| Transmitter |  |
| (1) Number of transmit antenna elements | 4 |
| (2) Number of transmit TxRUs Note: this row is void (left empty) for uplink |  |
| (2a) Number of transmit chains modelled in LLS | 1 |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth | 29 |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | 6 RBs @ (15 KHz for 3.5 GHz) or (30 KHz for 7.5 GHz) |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) Note: no PSD constraint for uplink | 23 |
| (3c) Bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. For uplink, (3a) = (3c) |  |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ((3c) /1000000) (dBm) |  |
| (4) Total antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4a) – (4b) (dB) | 0 |
| (4a) Antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4c) + 10 log ((1) / (2)) (dB) for downlink, and = (4c) + 10 log ((1) / (2a)) (dB) for uplink |  |
| (4b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB) |  |
| (4c) Gain of antenna element (dBi) | 0 |
| (5) Total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink |  |
| (5a) Antenna gain at antenna gain component 2 of transmitter = 10 log((2)/(2a)) (dB) Note: zero for uplink |  |
| (5b) Antenna gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink |  |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | 1 |
| (9) EIRP = (3bis) + (4) + (5) – (8) dBm | 31 |
| Receiver |  |
| (10) Number of receive antenna elements | Based on the agreement: (192, 256, 512) @ 3.5 GHz, and (768, 1024, 1536) @ 7.5 GHz |
| (10a) Number of receive TxRUs Note: this row is void (empty) for downlink | 1 |
| (10b) Number of receive chains modelled in LLS | 1 |
| (11) Total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11a) - (11b) (dB) |  |
| (11a) Antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11c) + 10 log ((10)/(10a)) (dB) for uplink = (11c) + 10 log ((10)/(10b)) (dB) for downlink |  |
| (11b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB) |  |
| (11c) Gain of antenna element (dBi) | 8 |
| (11bis) Total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink |  |
| (11bis-a) Antenna gain at antenna gain component 2 of receiver = 10 log((10a)/(10b)) (dB) Note: zero for downlink | 0 |
| (11bis-b) Antenna gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | 0 |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | 3 |
| (13) Receiver noise figure (dB) | 5 |
| (14) Thermal noise density (dBm/Hz) | -174 |
| (15) Receiver interference density (dBm/Hz) | -165.7 |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) | -164.03 |
| (18) Effective noise power = (16) + 10 log ((3c)) (dBm) |  |
| (19) Required SNR (dB) | Determined based on LLS achieving 10% BLER |
| (20) Receiver implementation margin (dB) | 2 |
| (21) H-ARQ gain (dB) Note: Only applicable if HARQ is not considered in LLS | 0.5 |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) |  |
| (22bis) MCL = (3bis) – (22) + (5) + (11bis) (dB) |  |
| (23) Hardware link budget, a.k.a. MIL = (9) + (11) + (11bis) − (12) − (22) (dB) Note: MIL can also be derived by (22bis) + (4) – (8) + (11) − (12) |  |
| Calculation of available pathloss |  |
| (25) Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation) (dB) | 4.85 |
| (26) BS selection/macro-diversity gain (dB) | 0 |
| (27) Penetration margin (dB) | 9 |
| (28) Other gains (dB) (if any please specify) | 0 |
| (29) Available path loss = (23) – (25) + (26) – (27) + (28) (dB) |  |
| Range/coverage efficiency calculation |  |
| FFS: (30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m) |  |

# Appendix 2: Additional Simulation Results

Table 6: Throughput of 6GR device with 5 MHz BW @ 154 dB MCL, 700 MHz

| Direction | BW [MHz] | SNR | RB# | Rep# | Thp [kbps] | MCS | TBS | slot# per TB (TBoMS) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL | 5 | -13.55 | 1 | 50 | 0.527 | 0 | 120 | 4 |
| UL | 5 | -19.57 | 4 | 128 | 0.876 | 0 | 128 | 1 |
| DL | 5 | -14.35 | 24 | 20 | 43 | 2 | 3848 | 4 |
| DL | 5 | -14.35 | 24 | 35 | 35.8 | 2 | 1320 | 1 |

Table 7: Throughput of 6GR device with 20 MHz BW @ 154 dB MCL, 2 GHz

| Direction | BW [MHz] | SNR | RB# | Rep# | Thp [kbps] | MCS | TBS | slot# per TB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL | 20 | -14.55 | 100 | 16 | 193.6 | 0 | 3368 | 1 |
| UL | 20 | -13.55 | 1 | 50 | 0.531 | 0 | 120 | 4 |
| UL | 20 | -13.55 | 1 | 50 | 0.423 | 0 | 24 | 1 |
| UL | 20 | -13.55 | 1 | 50 | 0.513 | 0 | 56 | 2 |
| UL | 20 | -19.57 | 4 | 128 | 0.916 | 0 | 120 | 1 |
| UL | 20 | -22.58 | 8 | 256 | 1.1 | 1 | 640 | 2 |
| UL | 20 | -27.35 | 24 | 512 | 1.4 | 0 | 1480 | 2 |

Table 8: Throughput of 6GR device with 5 MHz BW @ 144 dB MCL, 2 GHz

| Direction | BW [MHz] | SNR | RB# | Rep# | Thp [kbps] | MCS | TBS | slot# per TB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL | 20 | -4.55 | 100 | 2 | 1670 | 0 | 3368 | 1 |
| DL | 20 | -4.55 | 100 | 3 | 1672 | 2 | 5384 | 1 |
| UL | 20 | -3.55 | 1 | 4 | 6.768 | 0 | 120 | 8 |
| UL | 20 | -9.57 | 4 | 16 | 7.4 | 0 | 120 | 1 |
| UL | 20 | -17.35 | 24 | 64 | 14.8 | 1 | 2024 | 2 |