3GPP TSG RAN Meeting #110							RP-253335

Baltimore, USA, December 8-11, 2025

## Status Report to TSG

Agenda item:			9.5.1.5

| WI / SI Name | Enhancements of network energy savings for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | Netw_Energy_NR_enh-Perf |  |  |  |  |
| Unique ID | 1022095 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252332 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: n/a | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: n/a | Performance Part: 80% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Ajit Nimbalker |
|  | Company | Ericsson |
|  | Email | Ajit.Nimbalker@ericsson.com |
| Rapporteur (RAN2) | Name | Peng Cheng |
|  | Company | Apple |
|  | Email | pcheng24@apple.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

n/a

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

n/a

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.3	RAN3

n/a

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

Core Part

n/a

Performance Part

In RAN4#116bis, the following agreements were made.

Topic #1: On-demand SSB(OD-SSB) performance part

Issue 1-1-1: General – Case 1 and Case 2

Agreement:

- The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

  - Case #1: scenario #2, scenario #2A.

  - Case #2: scenario #2, scenario #2A and scenario #3B.

Issue 1-1-3: General – Different OD-SSB and AO-SSB relation in Case 2

Agreement:

RAN4 to define TCs for Alt Time-C1. Meanwhile,

- Option 1: Not define TC for Alt Time-C2

- Option 2: Select one test for Alt Time-C2. For UE supporting Alt Time-C2, the corresponding test case for Alt Time-C1 can be skipped.

Further discuss in one test case or separate test cases.

Issue 1-1-4: Scenario – Scell activation scope

Agreement:

RAN4 to only define the test cases for single Scell activation, Direct SCell activation.

Issue 1-1-6: T2 for deactivated SCell measurement

Agreement:

Define one test case in case 1 to verify the UE measurement behaviour when the OD-SSB is reactivated relevant to T2.

Issue 1-2-1: Test case list

Agreement

- Interruption Test Case

The interruption test for OD-SSB activation/deactivation and during measurement on deactivated NR SCC are merged to SCell activation and deactivated SCell measurement test cases.

Issue 1-3-1: General principle

Agreement:

RAN4 to apply the following principles for OD-SSB test

- Both FR1 and FR2

- Test non-DRX mode, and select one test case with DRX configuration for deactivated SCell measurement.

- The test without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

- Only NCD-SSB for OD-SSB test case

Topic #2: Adaptation of common signal/channel transmission

Issue 1-1: TC(s) to be specified besides L3 intra-frequency measurement

Agreement:

- Besides L3 intra-frequency measurement, specify the following test case for SSB adaptation

  - L1-RSRP measurement

  - L1-SINR measurement

Issue 1-2: General principle

Agreement:

- RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only

- Both FR1 and FR2

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement

- Only test non-DRX mode

- without SSB time index detection

- RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9.

- RAN4 to reuse legacy trigger events for measurement report after SSB adaptation.

Issue 1-3: SMTC and SSB configuration

Agreement (in AH):

- For L1-RSRP measurement test case:

  - SMTC periodicity from 20ms to 80ms, and from 80ms to 40ms.

  - SSB periodicity from 20ms to 80ms, and from 80ms to 40ms.

- For other test cases:

  - SMTC periodicity from 80ms to 20ms.

  - SSB periodicity from 80ms to 20ms.

Issue 1-4: How to verify SSB adaption

Agreement:

  - For L3 Intra-frequency serving Cell measurement:

- UE is configured with Event A1 or A2.

- Condition of serving Cell is changed to satisfy the Event threshold at the instance after DCI 2-9 received + the transition period.

- It is verified whether UE can trigger event within the delay based on the mapped additional SMTC.

Topic #3: OD-SIB1

Issue 3-1: Whether to define TC for OD-SIB1

Agreement:

- RAN4 to define cell reselection TC with longer TSI-NR for OD-SIB1.

Issue 3-3: TC scope

Agreement:

- Cell reselection FR1 to FR1 target Cell

- Cell reselection FR2 to FR2 target Cell

In RAN4#117, the following agreements were made.

Topic #1: On-demand SSB(OD-SSB) performance part

Issue 1-1-1: SCell activation of known SCell Test Case

Agreement:

No need to define the following test case

| Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- |
| SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation FR1 |

Issue 1-1-2: Directed SCell activation Test Case

Agreement:

Define the following test case

| Test category | Test scenario | Test configuration | Test purpose | Company |
| --- | --- | --- | --- | --- |
| Direct SCell activation at SCell addition based on OD-SSB | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a FR1 | ZTE |

Issue 1-1-3: Deactivated SCell measurement Test Case

Agreement:

Define the following test case:

| Test category | Test scenario | Test configuration | Test purpose | Company |
| --- | --- | --- | --- | --- |
| Intra-frequency measurement (deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW FR1 | Huawei |

Issue 1-1-4: Test case for Alt Time-C2

Agreement:

Not define TC for Alt Time-C2.

Topic #2: Adaptation of common signal/channel transmission

Issue 1-1: Whether to verify DCI processing time in L1 based SSB adaptation test

Agreement:

  - DCI processing time is to be verified in L1 based SSB adaptation test

Issue 1-2: How many SSB index in L1 based SSB adaptation test

Agreement:

  - Only SSB#0 is configured and observed in L1 based SSB adaptation test

Issue 1-3: Details in test procedure for L1 based SSB adaptation test

Agreement:

  - For TC of L1-RSRP/SINR measurement of SSB adaptation, the details in test procedure are proposed as follows

    - Treport=20ms (Periodic L1-RSRP reporting) for FR1

    - timeRestrictionForChannelMeasurements is configured

    - DCI 2-9 indicates SSB adaptation:

L1-RSRP TC: SSB periodicity from 20ms to 80ms, and from 80ms to 40ms

L1-SINR TC: SSB periodicity from 80ms to 20ms

    - Condition of serving Cell is changed at the same time as DCI 2-9 indication

    - UE shall report L1-RSRP result with new value based on indicated SSB from the first SSB after DCI decoding plus OD-SSB post processing time

The below figure (from R4-2520928) which illustrates one-way SSB adaption (from 80ms to 20ms) is for information

Figure cited from R4-2520928

Topic #3: OD-SIB1

Issue 3-1: Ending point for cell reselection TC with longer TSI-NR for OD-SIB1Agreement:

Agreement:

  - The ending point of the TC is defined as legacy (i.e., starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2)

Issue 3-3: Test requirement

Agreement:

- The cell reselection delay shall additionally consider the time for requesting and acquiring OD-SIB1 (i.e., Tevaluate, NR_Intra + TSI-NR + TOD-SIB1), where

  - The additional cell reselection delay due to OD-SIB1 is named TOD-SIB1. TOD-SIB1 comprises below three components:

    - UE transmits UL-WUS transmission

    - UE receives and decodes RAR

    - UE receives OD-SIB1 within OD-SIB1 window

  - TOD-SIB1 set to [1]s.

Issue 3-4: One direction or two directions for cell reselection TC with longer TSI-NR for OD-SIB1

Agreement:

- Only verify the case that UE reselect from Cell 1 to Cell2 (NES Cell), and no need to verify the case the UE reselect back from Cell2 (NES Cell) to Cell 1 as designed in legacy Cell reselection TC.

Issue 3-5: Verify known target cell case for cell reselection TC

Agreement:

- Specify the new test case for known target cell only.

#### 2.4.2	Remaining Open issues

- Performance requirements and test cases for the network energy saving techniques.

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

#### 4.1	List of RAN WG1 contributions

n/a

#### 4.2	List of RAN WG2 contributions

n/a

#### 4.3	List of RAN WG3 contributions

n/a

#### 4.4	List of RAN WG4 contributions

#### 4.4.1	RAN4#116-bis

R4-2513105	On RRM performance requirements of OD-SSB SCell operation	OPPO

R4-2513106	On RRM performance requirements of SSB adaptation	OPPO

R4-2513107	On RRM performance requirements of OD-SIB1	OPPO

R4-2513192	Discussion on RRM performance requirements for on-demand SSB SCell operation	CATT

R4-2513193	Discussion on RRM performance requirements for adaptation of common signalchannel transmission	CATT

R4-2513194	Discussion on RRM performance requirements for On-demand SIB1	CATT

R4-2513368	Discussion on RRM performance requirements for On-demand SSB SCell operation	China Telecom

R4-2513369	Discussion on RRM performance requirements for Adaptation of common signal/channel transmission	China Telecom

R4-2513370	Discussion on RRM performance requirements for On-demand SIB1	China Telecom

R4-2513445	Discussion on the performance part for NES	Xiaomi

R4-2513484	Discussion on performance requirements for On-demand SSB SCell operation	Huawei, HiSilicon

R4-2513485	Discussion on performance requirements for adaptation RS	Huawei, HiSilicon

R4-2513486	Discussion on performance requirements for On-demand SIB1	Huawei, HiSilicon

R4-2513514	Topic summary for [116bis][208] Netw_Energy_NR_enh_RRM_Part1	Moderator (Ericsson)

R4-2513515	Topic summary for [116bis][209] Netw_Energy_NR_enh_RRM_Part2	Moderator (Huawei)

R4-2513548	On performance requirements for on-demand SSB SCell operation	Qualcomm Technologies Ireland

R4-2513549	On performance requirements for SSB adaptation	Qualcomm Technologies Ireland

R4-2513550	On performance requirements for on-demand SIB1	Qualcomm Technologies Ireland

R4-2513555	Discussion on RRM performance requirements of OD-SSB for Rel-19 NES	LG Electronics Inc.

R4-2513556	Discussion on RRM performance requirements of SSB adaptation for Rel-19 NES	LG Electronics Inc.

R4-2513619	Discussion on RRM performance requirements for On-demand SSB SCell operation	Apple

R4-2513620	Discussion on RRM performance requirements for Adaptation of common signal/channel transmission	Apple

R4-2513621	Discussion on RRM performance requirements for On-demand SIB1	Apple

R4-2513706	Discussion on RRM performance for Rel-19 NES OD-SSB	vivo

R4-2513707	Discussion on RRM performance for Rel-19 NES adaptation	vivo

R4-2513708	Discussion on RRM performance for Rel-19 NES OD-SIB1	vivo

R4-2513723	Performance requirements for OD-SSB operation	Nokia, Nokia Shanghai Bell

R4-2513886	Discussion on Perf part of on-demand SSB for R19 NES	ZTE Corporation, Sanechips

R4-2513887	Discussion on Perf part of SSB adaptation for R19 NES	ZTE Corporation, Sanechips

R4-2513982	Discussion on performance requirements for OD-SSB	Ericsson

R4-2513983	Discussion on performance requirements for SSB adaptation	Ericsson

R4-2513984	Discussion on performance requirements for OD-SIB1	Ericsson

R4-2514208	Discussion on OD-SSB SCell operation performance	Samsung

R4-2514209	Discussion on SSB adaptation SCell operation performance	Samsung

R4-2514487	RRM performance requirements for SSB adaptation	Nokia, Nokia Shanghai Bell

R4-2514488	RRM performance requirements for On-demand SIB1	Nokia, Nokia Shanghai Bell

R4-2514807	Ad-hoc minutes for Netw_Energy_NR_enh_RRM	Ericsson

R4-2514808	WF on Netw_Energy_NR_enh_RRM_Part1	Ericsson

R4-2514809	WF on Netw_Energy_NR_enh_RRM_Part2	Huawei

#### 4.4.2	RAN4#117

R4-2520110	Draft CR on test case for SA event triggered reporting tests without gap under non DRX based on OD-SSB (TC10)	CATT

R4-2520111	Discussion on RRM performance requirements for on-demand SSB SCell operation	CATT

R4-2520479	Draft CR for NES TC5 of OD-SSB based known SCell activation in FR2 (Case 2, Alt-Time C1)	Xiaomi

R4-2520803	Discussion on RRM performance requirements for On-demand SSB SCell operation	China Telecom

R4-2520814	TC1 OD-SSB based SCell activation and deactivation in FR1	Nokia, Nokia Shanghai Bell

R4-2520837	Topic summary for [117][212] Netw_Energy_NR_enh_RRM_Part1	Moderator (Ericsson)

R4-2520838	Topic summary for [117][213] Netw_Energy_NR_enh_RRM_Part2	Moderator (Huawei)

R4-2520908	Draft CR for NES TC9 Intra-frequency measurement(Scenario 3B, Case 1)	LG Electronics Inc.

R4-2520926	Discussion on performance requirements for On-demand SSB SCell operation	Huawei, HiSilicon

R4-2520927	TC for SCell Activation and deactivation of unknown SCell in DRX based on OD-SSB	Huawei, HiSilicon

R4-2520928	Discussion on performance requirements for adaptation RS	Huawei, HiSilicon

R4-2520929	TC for L1-SINR measurement with SSB based CMR and dedicated IMR for SSB adaptation	Huawei, HiSilicon, China Telecom

R4-2520993	Draft CR on NES TC6 of OD-SSB based Direct SCell activation in FR1 (Case 2, Alt-Time C1)	CMCC

R4-2521099	Discussion on RRM performance for Rel-19 NES OD-SSB	vivo

R4-2521100	DraftCR on introducing TC for intra-frequency measurement (deactivated SCell) based on OD-SSB TC8	vivo

R4-2521101	Discussion on RRM performance for Rel-19 NES OD-SIB1	vivo

R4-2521102	DraftCR on introducing TC for OD-SIB1 based Cell reselection FR1 to FR1 target Cell	vivo

R4-2521245	RRM performance requirements for On-demand SIB1	Nokia, Nokia Shanghai Bell

R4-2521246	Draft CR 38.133 TC2: OD-SIB1 based Cell reselection FR2 to FR2 target Cell	Nokia, Nokia Shanghai Bell

R4-2521328	Draft CR for TC of OD-SSB based deactivated SCell measurement - case 1	OPPO

R4-2521683	Draft CR on test case for SCell activation and deactivation of known SCell in non-DRX based on OD-SSB	Samsung

R4-2521684	Draft CR on L1-RSRP test case for SSB adaptation	Samsung

R4-2521685	Discussion on L1-RSRP test case on adaption SSB	Samsung

R4-2521777	Discussion on remaining issues for OD-SSB performance requirements	Ericsson

R4-2521778	Discussion on remaining issues for SSB adaptation performance requirements	Ericsson

R4-2521779	Discussion on remaining issues for OD-SIB1 performance requirements	Ericsson

R4-2521782	Draft CR for NES TC3 unknown SCell activation(Case 2, Alt-Time C1)	Ericsson

R4-2521956	Draft CR on test case of L3 intra-frequency measurement with SSB adaptation_R19 NES	ZTE Corporation, Sanechips

R4-2521957	Draft CR on test case of OD-SSB based L1-RSRP measurement no DRX _R19 NES	ZTE Corporation, Sanechips

R4-2522642	WF on Netw_Energy_NR_enh_RRM_Part1	Ericsson

R4-2522643	Ad-hoc minutes for Netw_Energy_NR_enh_RRM 	Ericsson

R4-2522644	WF on Netw_Energy_NR_enh_RRM_Part2	Huawei, HiSilicon