3GPP TSG-RAN WG3 Meeting #130	R3-258019

# Dallas, US, 17 – 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1975 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the MME Name IE in the S1 REMOVAL RESPONSE message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Xiaomi, Qualcomm, Ericsson, CATT, Airbus, Jio Platforms, ZTE, LG Electronics, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a mis-slignment between tabular and ASN.1 for the presence of the MME Name IE in the S1 REMOVAL RESPONSE message. The presence is “Mandatory” in tabular, but “Optional” in ASN.1. The Assigned Criticality of the MME Name IE is also inconsistent between tabular and ASN.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make following changes to S1 REMOVAL RESPONSE message: - Change the Presence of MME Name IE to “O”, and add behavior text. - Change the Assigned Criticality of the MME Name IE to “ignore”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between tabular and ASN.1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.8.2, 9.1.8.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: change the Assigned Criticality of the MME Name IE to “ignore”. Rev 2: resubmit to RAN3#130. |  |  |  |  |  |  |  |  |

-----------------Start of the Changes-------------------

### 8.7.8	S1 Removal

#### 8.7.8.1	General

The purpose of the S1 Removal procedure is to remove the interface between the eNB and the MME in a controlled manner. If successful, this procedure erases any existing application level configuration data in the two nodes.

The procedure uses non-UE associated signaling.

#### 8.7.8.2	Successful Operation

Figure 8.7.8.2-1: S1 Removal, successful operation

The eNB initiates the procedure by sending the S1 REMOVAL REQUEST message to the MME. Upon reception of the S1 REMOVAL REQUEST message, the MME shall reply with the S1 REMOVAL RESPONSE message. After receiving the S1 REMOVAL RESPONSE message, the eNB shall initiate removal of the TNL association towards the MME and may remove all resources associated with that interface. The MME may then remove all resources associated with the eNB.

If the S1 REMOVAL RESPONSE message contains the MME Name IE, the eNB may use this IE as a human readable name of the MME.

-----------------Next Change-------------------

#### 9.1.8.16	S1 REMOVAL RESPONSE

This message is sent by the MME to acknowledge the initiation of removal the interface.

Direction: MME ® eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

-----------------End of Change-------------------