3GPP TSG RAN Meeting #110						           RP-253386

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.4

Source:	Apple, MediaTek, Xiaomi

Title:	TP to TR 38.914 on ISAC

Document for:	Discussion/Decision

# Introduction

In RAN#109, we’ve agreed to study the following ISAC modes:

- TRP monostatic,

- TRP-TRP bistatic,

- TRP-UE DL,

- UE-TRP UL,

- UE-UE bistatic and

- UE monostatic.

To recap, this decision was taken in the context of the discussion on requirements of new and existing services (see RAN#109 meeting report in [1]):

conclusion: proposal 1 and 2, moderator's proposals 1, 2, 3 (without [ ]) and first bullet of moderator's proposal 4 are endorsed

Where the “proposal 4” referred in the agreement can be found in [2] (Moderator's summary for RAN led 6G SI: Requirements of new and existing services):

Moderator proposal 4: - 6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic. - 6GR should study cooperative sensing/multi-static sensing mechanism involving multiple UEs and TRPs.

As one can see, this agreement isn’t easy to find, which causes confusion in WGs and other TSGs. It would be better to document it clearly, to prevent unnecessary confusion in other groups.

Observation 1: RAN#109 agreed to study all sensing modes (TRP and UE), but the agreement was not explicitly nor clearly documented.

Furthermore, it is important to discuss the ISAC key principles of operation and RAN architecture, as those would be quite new and for which there is no 5G baseline. We believe it is important define a clean and usable ISAC architecture, unburdened from the 5G positioning architecture (which saw almost no deployments). To this end, the following ISAC modes of operations should be considered:

- Network-based,

- UE-based, and

- UE-centric.

Finally, there are the following (among many others) use cases agreed in TR 22.870 of relevance [4]:

- Public Safety Use cases for both the target and subscriber e.g. R19 SA1 ISAC Use Case 5.27

- Intruder Detection use cases for home e.g. R19 SA1 Use Case 5.1 and 5.6

Observation 2: use cases agreed in TR 22.837 include both TRP and UE as sensing entities.

We propose to agree the TP to TR 38.914 provided below.

Proposal 1: agree the TP for TR 38.914 provided below.

# Text Proposal for TR 38.914

* * * Start of Changes * *

### 5.4.4	Sensing

Editor note:	More sensing use cases can be included depending on further discussion.

The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including the following targets: UAVs, human (indoor and outdoor), vehicles and AGVs.

The 6GR and 6G RAN architecture shall support sensing by TRPs and UEs.

* * * End of Changes * *

# References

[1] RAN#109 	Meeting report

[2] RP-252943	Moderator's summary for RAN led 6G SI: Requirements of new and existing services, CMCC

[3] TR 38.914	Study on 6G Scenarios and Requirements; (Release 20)

[4] TR 22.870	Study on 6G Use Cases and Service Requirements; Stage 1