3GPP TSG RAN Meeting #110	RP-253387

# Baltimore, U.S.A., December 8-11, 2025

Source:			Skylo Technologies, Lockheed Martin, EchoStar, Sateliot, ViaSat, ESA, Aalyria, Novamint, Terrestar, Google

Title:			Discussion on Rel-20 IoT NTN Ph4 Scope Update

Agenda item:	10.3.1

Document for:	Discussion and Approval

# Introduction

The support for Satellite Switch with Resynchronization in IoT-NTN was discussed in RAN2#131 (R2-2505933 [1]) and RAN2#132 (R2-2509101 [5]). Based on feedback documented in R2-2506203 [2] and R2-2509113 [6], RAN2 recommended that Satellite Switch with Resynchronization for IoT-NTN be further considered in Rel-20, specifically as an update to the existing Rel-20 IoT NTN WID.

Accordingly, this document discusses and propose necessary update to the RAN2 Rel-20 IoT NTN WID [3].

# Discussion

In an NGSO constellation, the satellites are constantly moving across the sky relative to the user on the ground. For a voice call to last longer than the serving satellite's visibility (which can be short, depending on the orbit altitude), the device must transition to an upcoming target satellite. Without an efficient switch procedure, the call will drop at every satellite change.

It is thus proposed to update the WID to study a resource-efficient quick switchover to avoid a time-consuming and power-intensive process cell reselection procedure, especially over high-latency satellite links. This seamless switch is important for maintaining the QoS required for voice, even low-bitrate Voice over NB-IoT-NTN.

Specified in TS 38.331, TS 38.321 and TS 38.300, the satellite switch with re-synchronization procedure is currently supported for NR-NTN. A UE capable of hard satellite switch with resynchronization in RRC_CONNECTED initiates the procedure when SatSwitchWithReSync and t-Service are included in SIB19. The procedure is executed by the UE in the RRC_CONNECTED state based on network-provided information in SIB19, which contains parameters for a target satellite required (e.g., t-ServiceStart) to perform satellite switch with resynchronization. One of the essential requirements of this satellite switch with resynchronization is that a currently serving cell (i.e., operated by a source satellite) and a target cell (i.e., operated by a target satellite) use the same PCI for their respective cells targeting the same geographical area. Accordingly, with a help of some essential parameters that the source satellite provides in its SIB19, namely ssb-timeoffset and t-ServiceStart pertaining to the target (cell) satellite, UEs know when to start the synchronization process. For (quasi-)earth-fixed deployments in NB-IoT NTN, the rapid movement of satellites across the sky can benefit from a similar procedure in order to facilitate seamless mobility between different satellites. The support of a "satellite switch with re-synchronization" procedure is preferred. This procedure enables a UE in the RRC_CONNECTED state to transition from a serving satellite to a new target satellite seamlessly without having to perform a blind search and synchronization with a new satellite on the horizon. In addition, such a switchover can avoid L3-based handover procedure that is not well supported by NB-IoT.

For NB-IoT, a new or modified SIB would be used to broadcast the satSwitchwithReSync parameters, which contain orbital parameters and configurations for upcoming target satellites. A UE in the RRC_CONNECTED state reads and processes this SIB. Based on its internal clock, position, and the provided satellite orbital parameters, the UE's RRC layer can pre-determine the precise time when it needs to switch to a new satellite.

At the pre-determined time, the UE initiates the switch. It applies the resynchronization parameters from the SIB to adjust its timing and frequency, enabling it to quickly re-synchronize with the new satellite. Since the PCI is the same, the physical layer transition is simplified.

Observation 1: The Satellite Switch with Re-synchronization procedure should be supported for NB-IoT NTN.

# Proposal

Based on the preceding discussion, we propose to update WID in RP-252473 [3] to include the scope of studying the support of satellite switch with resynchronization for NB-IoT-NTN as outlined in [4].

Proposal 1: It is proposed to update the IoT NTN Ph4 WID to Support of Satellite Switch with Resynchronization for NB-IoT-NTN (hard switch only) as outlined in RP-253388 [4].

# Reference

[1] R2-2505933, [TEI19] Introduction of Satellite Switch with Resynchronization support for IoT NTN, RAN2#131.

[2] R2-2506203, Report from session on NR NTN and IoT NTN, RAN2#131.

[3] RP-252473, New WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4, RAN#109.

[4] RP-253388, Rel-20 New WID Scope Update: NTN for IoT Ph4, RAN#110.

[5] R2-2509101, Discussion on Rel-20 IoT NTN Ph4 Scope Update, RAN2#132.

[6] R2-2509113, Report from Break-out session on NR-NTN and IoT-NTN, RAN2#132.