3GPP TSG RAN meeting #110								     		RP-253404

Baltimore, USA, December 8th - 11th, 2025

## Status Report to TSG

Agenda item:			9.6.1.1

| WI / SI Name | Introduction of Ku Bands for NR NTN |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_Ku_bands |  |  |  |  |
| Unique ID | 1040135 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP‑252911 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: 75% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

- requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Manook Soghomonian |
|  | Company | SES |
|  | Email | Manook.soghomonian@ses.com |

## 2.1	RAN4

Summary

After the group completed the core part of the NR_NTN_Ku_bands work item during Sep. 2025, the group had started the performance part of this WI.

The group had discussed the test configurations and identified the specification impact of SAN conformance testing for the NTN Ku bands. However, the measurement uncertainties (MUs) will still require further input from the TE vendors.

Then regarding the RRM performance requirement and test cases for the NTN Ku band, the group agreed to reuse the legacy measurement accuracy requirements from the NTN Ka band, reuse the existing test cases of FR2-NTN (from R18 for Ka band) for the FR2-NTN Ku bands, and reuse the existing test cases of FR2-NTN for Ku band with FR1-NTN numerology with the necessary modifications for Ku bands for FR1-NTN Ku bands. In addition, the group will introduce NGSO configuration for transmit timing test case of a mobile VSAT.

Regarding the SAN demodulation requirement, the group agreed to reuse existing SAN demodulation requirements for SAN type 1-O for FR1-NTN Ku-bands, and reuse existing SAN demodulation requirements for SAN type 2-O for FR2-NTN Ku-bands.

For the UE demodulation requirements, the group agreed on the test parameters for the FR1-NTN Ku-bands with 15 kHz and 30 kHz SCS, and will continue working on finalizing the corresponding requirements. For the FR2-NTN Ku-bands, the group agreed to reuse the performance criteria with 120 kHz SCS defined for the Ka-bands.

#### 2.1.1	Agreements

##### Performance part:

###### RAN WG4 Meeting #116bis, Prague, Czech Republic, 13th – 17th October, 2025:

The following agreements for SAN conformance testing for Ku-band documented in the Way Forward for [116bis][308] Rel-19_BS_SAN_conformance_Part2 in [18] were approved:

Way forward:

Sub-topic 1-1

Issue 1-1-1: MU is an Open issue that needs further discussion

For SAN Type 1-O above 10 GHz, it is not a straightforward choice what MUs to use, since the range of 10-15 GHz was not studied before for TN and cannot be reused for NTN.

Agreement:

Inputs from TE vendors would be appreciated, with respect to Measurement Uncertainties (MUs) for:

13.75GHz-14.5GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests

10.7GHz-12.75GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests.

Companies to further discuss if current FR2-NTN MUs could be reused.

Issue 1-1-2: Test Signal for SAN 1-H and 1-O test configurations

In TS 38.181, the test signal used to build Test configurations for SAN type 1-H and SAN type 1-O is defined in Table 4.7.2-1.

Agreement:

Part 1: The table 4.7.2-1 to be titled to reflect operation below 10 GHz.

Table 4.7.2-1: Signal to be used to build NR TCs for SAN type 1-H and SAN type 1-O operating below 10 GHz

| Operating Band characteristics |  | FDL_high – FDL_low <100 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note 1, 2 and 3) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. NOTE 2: For NRTC1, if NB-IoT is not supported, the signal's channel bandwidth shall be the narrowest supported one. NOTE 3: It should use 3MHz channel bandwidth, if supported. |  |  |

Part 2: For FR1-NTN Ku-band, a new table needs to be added to support new operating band characteristics, and the table 4.7.2-1a title should reflect operation above 10 GHz.

Table 4.7.2-1a: Signal to be used to build NR TCs for SAN type 1-O operating above 10 GHz

| Operating Band characteristics |  | FDL_high – FDL_low ≤2050 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 10 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |

Sub-topic 1-2 Manufacturer declaration Ku-band SAN

Issue 1-2-1: Manufacturer declaration for 1-O

Agreement: Define manufacturer declaration for SAN type 1-O operating below 10GHz and SAN type 1-O operating above 10GHz as following example table, with two columns for SAN type 1-O, to differentiate below and above 10 GHz.

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SAN type 1-H (Note 2) | SAN type 1-O operating below 10GHz | SAN type 1-O operating above 10GHz | SAN type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the SAN enclosure. | x | x | x | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the SAN enclosure. | x | x | x | x |
| … | … | … | .. | … | … | … |
| D.19 | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a | n/a |
| D.20 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD (D.19). (Note 5) | x | x | n/a | n/a |
| … | … | … | .. | … | … | … |

Sub-topic 1-3: Applicability of requirements for Ku-band SAN conformance testing

Issue 1-3-1: General requirements in TS 38.181 for NTN Ku-band

Agreement: General requirements in TS 38.181 for NTN Ku-band as per table below.

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| 4 General test conditions and declarations |  |
| 4.1 Measurement uncertainties and test requirements | Need to update the test system uncertainty for carrier frequency f of Ku band in Table 4.1.2.2-2, 4.1.2.2-3, 4.1.2.3-2, 4.1.2.3-3. |
| 4.2 Requirement reference points | No specification impact. |
| 4.3 Satellite Access Node classes | No specification impact. |
| 4.4 Regional requirements | No specification impact. |
| 4.5 SAN configurations | No specification impact. |
| 4.6 Manufacturer declarations | No specification impact. |
| 4.7 Test configurations | Need to define the signal to be used to build NR TCs for SAN type 1-O for Ku band. |
| 4.8 Applicability of requirements | No specification impact. |
| 4.9 RF channels and test models | No specification impact. |
| 4.10 Void |  |
| 4.11 Reference coordinate system | No specification impact. |
| 4.12 Format and interpretation of tests | No specification impact. |

Issue 1-3-2: Tx requirements in TS 38.181 for NTN Ku-band

Agreement: Tx requirements for NTN Ku band to be defined as below:

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| 9 Radiated transmitter characteristics |  |
| 9.1 General | No specification impact. |
| 9.2 Radiated transmit power | Need to update test requirement for Ku band in Table 9.2.5-1. |
| 9.3 OTA SAN output power | The TRP accuracy in section 9.3.5 needs to be updated for Ku band. |
| 9.4 OTA output dynamics |  |
| 9.4.1 General | No specification impact. |
| 9.4.2 OTA RE power control dynamic range | No specification impact. |
| 9.4.3 OTA total power dynamic range | OTA total power dynamic range requirement in 9.4.3.5 needs to be updated for Ku bands. |
| 9.4.4 OTA NB-IoT RB power dynamic range for in-band operation | No specification impact. |
| 9.5 OTA transmit ON/OFF power | No specification impact. |
| 9.6 OTA transmitted signal quality |  |
| 9.6.1 General | No specification impact. |
| 9.6.2 OTA frequency error | No specification impact. |
| 9.6.3 OTA modulation quality | EVM window length in section 9.6.3.5.1 for FR1-NTN needs to be updated. |
| 9.7 OTA unwanted emissions |  |
| 9.7.1 General | No specification impact. |
| 9.7.2 OTA occupied bandwidth | Span and number of measurement points for OBW measurements in Table 9.7.2.4.2-1 needs to be updated for FR1-NTN Ku bands. |
| 9.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | SAN channel bandwidths in Table 9.7.3.5.1-1 and 9.7.3.5.1-2 need to be updated. |
| 9.7.4 OTA out-of-band emissions | No specification impact. |
| 9.7.5 OTA transmitter spurious emissions | OTA transmitter spurious emission limits for Ku bands needs to be defined. |
| 9.8 OTA transmitter intermodulation | No specification impact. |

Issue 1-3-3: Rx requirements in TS 38.181 for NTN Ku-band

Agreement: Rx requirements for NTN Ku band to be defined as below:

| Requirements in TS 38.181 | Expected impact |
| --- | --- |
| 10 Radiated receiver characteristics |  |
| 10.1 General | OTA REFSENS RoAoA absolute requirement values for FR1-NTN operating above 10GHz need to be updated. |
| 10.2 OTA sensitivity | EIS levels needs to be updated for Ku bands. |
| 10.3 OTA reference sensitivity level | OTA reference sensitivity level needs to be updated for Ku bands. |
| 10.4 OTA dynamic range | Not applicable. |
| 10.5 OTA in-band selectivity and blocking |  |
| 10.5.1 OTA adjacent channel selectivity | OTA ACS requirement for Ku bands needs to be updated. |
| 10.5.2 OTA in-band blocking | Not applicable. |
| 10.6 OTA out-of-band blocking | OTA out-of-band blocking for SAN type 1-O operating above 10GHz needs to be updated. |
| 10.7 OTA receiver spurious emissions | Not applicable. |
| 10.8 OTA receiver intermodulation | Not applicable. |
| 10.9 OTA in-channel selectivity | OTA ICS for SAN type 1-O operating above 10GHz needs to be updated. |

The following agreements for RRM performance requirements for NR NTN Ku band documented in the Way Forward for [116bis][310] Rel-19 RRM performance_Part2 in [17] were approved:

Issue 1-1-1: Measurement accuracy with new band

Agreement

The legacy measurement accuracy requirements for Ka band NTN (FR2-NTN) shall be reused for Ku band with FR1 numerology and FR2 numerology.

Introduce FR1 numerology in FR2-NTN accuracy requirements table.

The following measurement accuracy requirements shall be considered:

Intra-frequency RSRP accuracy requirements

Inter-frequency RSRP accuracy requirements

Intra-frequency RSRQ accuracy requirements

Inter-frequency RSRQ accuracy requirements

Intra-frequency SINR accuracy requirements

Inter-frequency SINR accuracy requirements

L1-RSRP accuracy requirements

FFS the band group to include Ku band

Issue 1-2-1: test cases with new requirement

Agreement

For Ku band with FR1 and FR2 numerology, introduce new test cases for hard and soft satellite switching for VSAT UEs

Issue 1-2-2: UE transmit timing test for NGSO VSAT

Agreement

Introduce NGSO configuration for transmit timing test case of a mobile VSAT.

Further discuss the need to emulate the mobility of the mobile NGSO VSAT UE.

Issue 1-2-3: Test cases for Ku bands in FR2-NTN

Agreement

For Ku bands in FR2-NTN, the existing test cases defined for FR2-NTN (in R18 for Ka band) are re-used, unless otherwise specified.

No explicit clarification is needed in the specification; by default, FR2-NTN test cases apply unless otherwise specified.

Issue 1-2-4: Test cases for Ku bands in FR1-NTN

WayForward

Further discuss the following options

Option 1: reuse exiting test cases defined for SAN FR1

Add a table clarifying test applicability for the corresponding test cases for VSAT UEs.

Add OTA related parameters

Option 2: reuse existing test cases defined for FR2-NTN, unless otherwise specified

Introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

Issue 1-2-5: Test applicability and principle

Agreement

The testing related to satellite access defined in A.3.36 with FR2 can be a reference.

If mobile VSAT UE supports both GSO and NGSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

RAN4 to discuss the test applicability for VSAT supporting Ku bands in both FR1-NTN and FR2-NTN, if such VSAT exists.

Option 1: it is at least to pass two test configurations: one with FR1 numerology and one with FR2 numerology.

Other options are not precluded.

The following agreements for demodulation requirements for NR NTN Ku band documented in the Way Forward for [116bis][312] Rel-19 Demodulation_Part2 in [19] were approved:

Sub-Topic 1-1: Ku Band SAN Demodulation

Whether to define requirements

Agreement

Do not define SAN demodulation requirements

Sub-Topic 1-2: Ku Band UE Demodulation

Whether to define requirements for FR1

Agreement

RAN4 could consider both PDCCH and PDSCH requirement could be defined for both 15KHz and 30KHz SCS for Ku-band

PDSCH Parameters:

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | [TBD] | 50 / 15 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 |  |
| 3-2 | [TBD] | 50 / 30 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 |  |
| Note1: The Maximum throughput is based upon HARQ feedback disabled. |  |  |  |  |  |  |  |

PDCCH Parameters:

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| [2-1] | [50] | [48] | [1] | [8] | [TBD] | NTN-TDLC5-1200 | 1x1 | 1 | TBD |

Whether to define requirements for FR2

Agreement

Do not introduce new UE demodulation requirements for FR2-NTN Ku band

Sub-Topic 1-3: Ku Band CSI Requirements

Whether to define CSI Requirements

Agreement

No CSI reporting requirements will be defined for NTN with Ku-band.

Work Split

Specification – TS 38.101-5

| Big CR | Nokia |
| --- | --- |
| PDSCH Requirements (including Reference Channel Generation) | Ericsson |
| PDCCH Requirements (including Reference Channel Generation) | SES |

Endorsed draft CRs in RAN4#116bis:

None

###### RAN WG4 Meeting #117, Dallas, USA, 17th – 21th December, 2025:

The following agreements for SAN conformance testing for Ku-band documented in the Way Forward for [117][308] Rel-19_BS_SAN_conf_Part2_NTN in [42] were approved:

Issue 1-1-1: MUs for Ku-band in 10.7GHz-12.75GHz range (DL)

Issue 1-1-2: MUs for Ku-band in 13.75GHz-14.5GHz range (UL)

Agreement: Inputs from TE vendors would be required with respect to Measurement Uncertainties (MUs) at the February 2026 RAN4#118 meeting for:

13.75GHz-14.5GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA receiver tests

10.7GHz-12.75GHz for FR1-NTN Ku-band and FR2-NTN Ku-band OTA transmitter tests.

Issue 1-1-3: Naming of SAN Type 1-O

Agreement: FFS if to use SAN Type 1-OL and 1-OH, (from Low and High part of the band) together with a proper definition.

Issue 1-1-4: Work split for TS 38.181

Agreement: The following work-split will be used for implementation of changes related to Ku-Band in the SAN Conformance Testing Specifications:

| Clause # | Title | Company |
| --- | --- | --- |
| 4.1 | Measurement uncertainties and test requirements | R&S |
| 4.6 | Manufacturer declaration | Ericsson |
| 4.7 | Test configurations | Ericsson |
| 9.2 | Radiated transmitted power | THALES |
| 9.3 | OTA SAN output | THALES |
| 9.4.3 | OTA total power dynamic range | THALES |
| 9.6.3 | OTA modulation quality | THALES |
| 9.7.2 | OTA occupied bandwidth | ZTE |
| 9.7.5 | OTA Tx spurious emissions | ZTE |
| 10.1 | General | CATT |
| 10.2 | OTA sensitivity | CATT |
| 10.3 | OTA reference sensitivity level | CATT |
| 10.4 | OTA dynamic range | ZTE |
| 10.5 | OTA in-band selectivity and blocking | ZTE |
| 10.6 | OTA out-of-band blocking | ZTE |
| 10.9 | OTA in-channel selectivity | ZTE |

Agreement: Draft CR R4-2521058 (ZTE) can be used as starting point for February 2026 RAN4#118 meeting.

The following agreements for RRM performance requirements for NR NTN Ku band documented in the Way Forward for [117][310] Rel-19 RRM perf_Part2_NTN in [41] were approved:

Issue 1-1-1: Measurement Accuracy

Agreement

Not add the new band group for FR1-NTN Ku band accuracy requirement.

Extend the current FR2-NTN measurement accuracy requirement to cover FR1-NTN Ku band VSAT.

Issue 1-2-1: Test cases for Ku bands in FR1-NTN

Agreement

Reuse the existing test cases defined for FR2-NTN for Ku band with FR1-NTN numerology with the necessary modifications

Introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

Specify the applicability of test cases for FR1-NTN in the specification.

Issue 1-2-2: Reference Channel for Ku band with FR1-NTN numerology

Agreement

Use 30 kHz SCS reference channels for testing for Ku band with FR1-NTN numerology.

Issue 1-2-3: Test principle for VSAT supporting both FR1 and FR2 numerology

Agreement

The UE shall pass two test configurations: one with FR1 numerology and one with FR2 numerology.

Issue 1-2-4: UE transmit timing test for NGSO VSAT

Not pursue

The following agreements for demodulation requirements for NR NTN Ku band documented in the Way Forward for [117][312] Rel-19 Demod_Part2_NTN in [43] were approved:

1.1 Ku Band SAN Demodulation Requirements

Reuse of existing requirements

Agreement:

Do not define new SAN demodulation requirement, reuse existing SAN demodulation requirements for SAN type 1-O for FR1-NTN Ku-band, and reuse existing SAN demodulation requirements for SAN type 2-O for FR2-NTN Ku-band.

AWGN power level at the SAN input for PUSCH demodulation requirement for SAN type 1-O operating above 10GHz

Agreement:

Add AWGN power level at the SAN input for PUSCH demodulation requirement for SAN type 1-O operating above 10GHz in Table 11.2.1.4.2-2 of TS 38.181 as below

AWGN power level at the SAN input for PUCCH demodulation requirement for SAN type 1-O operating above 10GHz

Agreement:

Add the AWGN power level at the SAN input for PUCCH demodulation requirement for SAN type 1-O operating above 10GHz in Table 11.3.1.4.2-2 of TS 38.181 as below:

AWGN power level at the SAN input for PRACH demodulation requirement for SAN type 1-O operating above 10GHz

Agreement:

Add the AWGN power level at the SAN input for PRACH demodulation requirement for SAN type 1-O operating above 10GHz in Table 11.4.1.4.2-1 of TS 38.181 as below

1.2 Ku Band UE Demodulation

Test Parameters for PDSCH Requirements (for FR1 Configuration)

Agreement:

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  |  | Transmission scheme 1 |
| Carrier configuration |  | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  |  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 |  | Cyclic prefix |  |  | Normal |
|  |  | RB offset |  | RBs | 0 |
|  |  | Number of contiguous PRB |  | PRBs | 50MHz/15kHz SCS: 270 50MHz/30kHz SCS: 133 |
| Common serving cell parameters |  | Physical Cell ID |  |  | 0 |
|  |  | SSB position in burst |  |  | First SSB in Slot #0 |
|  |  | SSB periodicity |  | ms | 20 |
| PDCCH configuration |  | Slots for PDCCH monitoring |  |  | Each slot |
|  |  | Symbols with PDCCH |  | Symbols | 0, 1 |
|  |  | Number of PRBs in CORESET |  |  | Table 5.2-2 of 38.101-4 for tested channel bandwidth and subcarrier spacing |
|  |  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |
|  |  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  |  | DCI format |  |  | 1_1 |
|  |  | TCI state |  |  | TCI state #1 |
|  |  | PDCCH & PDCCH DMRS Precoding configuration |  |  | Single Panel Type I, Random per slot with equal probability of each applicable i1, i2 combination, and with REG bundling granularity for number of Tx larger than 1 |
| Cross carrier scheduling |  |  |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  |  |  | k0=0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  |  | 'No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  |  |  | 3 for 2 CSI-RS ports and 5 for 4 CSI-RS ports |
|  | First subcarrier index in the PRB used for CSI-RS |  |  |  | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  |  | 'No CDM' for 1 transmit antenna 'FD-CDM2' for 2 and 4 transmit antenna |
|  | Density (ρ) |  |  |  | 1 |
|  | CSI-RS periodicity |  |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | Slots | 0 |
|  | Frequency Occupation |  |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  |  |  | 5 |
|  | First subcarrier index in the PRB used for CSI-RS |  |  |  | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |
|  | CDM Type |  |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  |  | 1 |
|  | CSI-RS periodicity |  |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | Slots | 0 |
|  | Frequency Occupation |  |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
| PDSCH DMRS configuration | Antenna ports indexes |  |  |  | {1000} for Rank 1 tests |
|  | Position of the first DMRS for PDSCH mapping type A |  |  |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  |  | 1 for Rank 1 |
| TCI state #0 | Type 1 QCL information |  | SSB index |  | SSB #0 |
|  |  |  | QCL Type |  | Type C |
|  | Type 2 QCL information |  | SSB index |  | N/A |
|  |  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  |  | QCL Type |  | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A |
|  |  |  | QCL Type |  | N/A |
| PT-RS configuration |  |  |  |  | PT-RS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Symbols for all unused REs |  |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 of 38.101-4 |
| Physical signals, channels mapping and precoding |  |  |  |  | As specified in Annex B.4.1 of 38.101-4 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. Note 3: Refer to Table 7.4.1.5.3-1 in [9] |  |  |  |  |  |

Channel Model for PDSCH Requirements

Agreement:

NTN-TDLC5-600.

FR2 configuration for Ku Band

Agreement:

Reuse existing Ka Band Performance criteria with 120 kHz SCS, no new requirements in specification. Applicability rule to be included referring that Ku Band FR2 configuration will use Ka band requirements.

Test Cases for PDCCH Requirements

Agreement:

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 2-1 | 50 | 270 | 1 | 8 | R.PDCCH.A-1.1 FDD | NTN-TDLC5-600 | 1x1 | 1 | TBD |
| 2-2 | 50 | 270 | 1 | 16 | R.PDCCH.A-1.2 FDD | NTN-TDLC5-600 | 1x1 | 1 | TBD |
| 2-3 | 50 | 132 | 1 | 8 | R.PDCCH.A-1.3 FDD | NTN-TDLC5-600 | 1x1 | 1 | TBD |
| 2-4 | 50 | 132 | 1 | 16 | R.PDCCH.A-1.4 FDD | NTN-TDLC5-600 | 1x1 | 1 | TBD |

Channel Model for PDCCH Requirements

Agreement:

NTN-TDLC5-600

FRC for PDCCH Requirements

Agreement:

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.A-1.1 FDD | R.PDCCH.A-1.2 FDD | R.PDCCH.A-1.3 FDD | R.PDCCH.A-1.4 FDD |
| Subcarrier spacing | kHz | 15 | 15 | 30 | 30 |
| CORESET frequency domain allocation |  | 270 | 270 | 132 | 132 |
| CORESET time domain allocation |  | 1 | 2 | 1 | 2 |
| Aggregation level |  | 8 | 16 | 8 | 16 |
| DCI Format |  | 1_0 | 1_1 | 1_0 | 1_1 |
| Payload (without CRC) | Bits | 44 | 56 | 42 | 56 |

Endorsed draft CRs in RAN4#117:

| Ref | Tdoc | Subject | Source | Status |
| --- | --- | --- | --- | --- |
| [29] | R4-2521560 | Test case on satellite switching on Ku band | ZTECorporation,Sanechips | endorsed |
| [31] | R4-2521736 | DraftCR to add FR1-NTN configuration for test cases involving VSAT UEs in Ku Band | Nokia | endorsed |

#### 2.1.2	Remaining Open issues

##### Core part:

None

##### Performance part:

Remaining RRM performance requirements and test cases

UE demodulation and CSI reporting requirements

Satellite access node demodulation requirements

Satellite access node conformance tests

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

#### Not applicable.

## 4.	References

Contributions submitted to RAN4#116bis:

| Ref | Tdoc | Subject | Source | Status |
| --- | --- | --- | --- | --- |
| [1] | R4-2513154 | Discussion on SAN demodulation performance requirements for NR NTN Ku-band | CATT | noted |
| [2] | R4-2513155 | Discussion on SAN conformance tests for NR NTN Ku-band | CATT | noted |
| [3] | R4-2513407 | View on demoduation requirement for NR_NTN_Ku_bands | Samsung | noted |
| [4] | R4-2513581 | Ku Band Performance Part: An overview of the simulation work | SES S.A. | noted |
| [5] | R4-2513635 | Discussion on RRM performance requirements for NR NTN Ku bands | MediaTek inc. | noted |
| [6] | R4-2514028 | Discussion on SAN conformance testing for NR NTN Ku band | ZTE Corporation, Sanechips | noted |
| [7] | R4-2514064 | Discussion on performance requirements for NTN Ku band | Ericsson | noted |
| [8] | R4-2514095 | Discussion on RRM performance requirements on Ku band NTN | ZTECorporation,Sanechips | noted |
| [9] | R4-2514113 | SAN RF Conformance testing for Ku-bands | Ericsson, Thales | noted |
| [10] | R4-2514140 | Discussion on demodulation requirements for NR NTN on Ku band | Ericsson | noted |
| [11] | R4-2514161 | Discussion on performance requirements for NTN in Ku band | Huawei, HiSilicon | noted |
| [12] | R4-2514348 | Performance Requirements for NTN UEs on Ku band operation | Nokia | noted |
| [13] | R4-2514431 | Topic summary for [116bis][308] Rel-19_BS_SAN_conformance_Part2 | Moderator (Iridium) | noted |
| [14] | R4-2514433 | Topic summary for [116bis][310] Rel-19 RRM performance_Part2 | Moderator (MediaTek) | noted |
| [15] | R4-2514435 | Topic summary for [116bis][312] Rel-19 Demodulation_Part2 | Moderator (Nokia) | noted |
| [16] | R4-2515041 | Way Forward for [116bis][312] Rel-19 Demodulation_Part2 | Nokia | revised |
| [17] | R4-2515055 | Way Forward for [116bis][310] Rel-19 RRM performance_Part2 | MediaTek | approved |
| [18] | R4-2515065 | Way Forward for [116bis][308] Rel-19_BS_SAN_conformance_Part2 | Iridium | approved |
| [19] | R4-2515160 | Way Forward for [116bis][312] Rel-19 Demodulation_Part2 | Nokia | approved |

Contributions submitted to RAN4#117:

| Ref |  | Tdoc |  | Subject | Source | Status |
| --- | --- | --- | --- | --- | --- | --- |
| [20] | R4-2520018 |  | Ku Band Simulation Summary Collection |  | Nokia | noted |
| [21] | R4-2520034 |  | Discussion on RRM performance requirements for NR NTN Ku bands |  | MediaTek inc. | noted |
| [22] | R4-2520061 |  | Discussion on SAN demodulation performance requirements for NR NTN Ku-band |  | CATT | noted |
| [23] | R4-2520062 |  | Discussion on SAN conformance tests for NR NTN Ku-band |  | CATT | noted |
| [24] | R4-2520379 |  | Ku Band Performance Part: Simulation Results of Downlink and Uplink Channels |  | SES S.A. | noted |
| [25] | R4-2521058 |  | DraftCR to TS38.181 Introduction of Ku bands |  | ZTE Corporation, Sanechips | noted |
| [26] | R4-2521216 |  | Discussion on performance requirements for NTN Ku band |  | Ericsson | noted |
| [27] | R4-2521355 |  | Discussion on NR NTN Ku band demodulation |  | Huawei,HiSilicon | noted |
| [28] | R4-2521543 |  | Discussion on RRM performance requirements on Ku band NTN |  | ZTECorporation,Sanechips | noted |
| [29] | R4-2521560 |  | Test case on satellite switching on Ku band |  | ZTECorporation,Sanechips | endorsed |
| [30] | R4-2521633 |  | Discussion on performance requirements for NTN in Ku band |  | Huawei, HiSilicon | noted |
| [31] | R4-2521736 |  | DraftCR to add FR1-NTN configuration for test cases involving VSAT UEs in Ku Band |  | Nokia | endorsed |
| [32] | R4-2521737 |  | Discussing the test configuration using FR1-NTN numerology in Ku band |  | Nokia | noted |
| [33] | R4-2521769 |  | Discussion on NR NTN Ku band demodulation requirements |  | Ericsson | noted |
| [34] | R4-2521770 |  | Simulation results on NR NTN Ku band demodulation requirements |  | Ericsson | noted |
| [35] | R4-2522078 |  | SAN RF Conformance testing for Ku-bands |  | Ericsson | noted |
| [36] | R4-2522216 |  | Topic summary for [117][308] Rel-19_BS_SAN_conf_Part2_NTN |  | Moderator (Thales) | noted |
| [37] | R4-2522219 |  | Topic summary for [117][310] Rel-19 RRM perf_Part2_NTN |  | Moderator (MediaTek) | noted |
| [38] | R4-2522222 |  | Topic summary for [117][312] Rel-19 Demod_Part2_NTN |  | Moderator (Nokia) | noted |
| [39] | R4-2522932 |  | Way Forward for [117][312] Rel-19 Demod_Part2_NTN |  | Nokia | revised |
| [40] | R4-2522933 |  | Offline meeting minutes for [117][312] Rel-19 Demod_Part2_NTN |  | Nokia | noted |
| [41] | R4-2522936 |  | Way Forward for [117][310] Rel-19 RRM perf_Part2_NTN |  | MediaTek | approved |
| [42] | R4-2522949 |  | Way Forward for [117][308] Rel-19_BS_SAN_conf_Part2_NTN |  | Thales | approved |
| [43] | R4-2523085 |  | Way Forward for [117][312] Rel-19 Demod_Part2_NTN |  | Nokia | approved |

12.08.2024		minor adaptations for RAN #105

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v01	13.11.2009		First version of the template