3GPP TSG-RAN Meeting #110			RP-253414

Baltimore, US, December 08-11, 2025

Source:	Reliance Jio, IIT-M, WiSig

Title:	Device Type Requirements for 6G

Agenda Item:	8.2.1.3.2

Document for:	Discussion and Decision

# 1	Background

In RAN#109, the following proposals in were endorsed [1]:

Proposal 3: To investigate further:

  - Motivations/justifications behind the proposed diverse device types, which should be a limited set

  - Whether/how to have one or more device types for eMBB or 6G IoT

  - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc.

  - Whether/how to explicitly standardize device types

  - Ensuring forward compatibility

  - Minimizing/avoiding potential market fragmentation

Note: the terminology “device type” is subject to further discussion and possible refinement.

Proposal 4: In terms of diverse device types, study further:

- Possible parameters/factors, e.g.:

  - Number of Tx antennas/chains

  - Number of Rx antennas/chains

  - Power classes

  - Maximum UE bandwidth (DL/UL)

  - Peak data rate (DL/UL)

  - Maximum MIMO layers (DL/UL)

  - Duplex mode

  - Max modulation order (DL/UL)

  - CA/spectrum aggregation (DL/UL)

  - UE processing capabilities

  - Coverage

  - Energy efficiency

  - Mobility/speed

  - Sensing

  - AI

A limited set of device types are proposed for 6GR motivated by practical considerations and as learning from 5G deployments.

# 2	Device Types

2.1 Classification of device types for 6G

The 5G Reduced Capability (RedCap) was defined as a new device type to fill a gap between high-speed 5G and low-power IoT technologies like NB-IoT. Its adoption has been slow and analysts suggest the current market is not where it was predicted to be. There are multiple reasons for this, some being device ecosystem maturity, slow 5G standalone (SA) network rollouts, and the continued viability of existing LTE networks. The premise RedCap was to replace LTE modems in a 5G only scenario and it does not look like LTE networks are going anywhere anytime soon.

Wearables such as Smartwatches, fitness trackers, and e-health medical monitoring devices today use LTE devices and could subsequently use 5G RedCap modems. Even Point-of-Sale (POS) terminals, Payment systems and smart meters are sufficiently addressed by existing LTE and subsequently (when that happens) 5G RedCap device types.

Observation 1: RedCap as a category has not succeeded in the market for 5G. There should be no additional efforts to define another such category in 6G.

AI devices are proliferating, AI devices are typically categorized by their capabilities (Narrow, General, Superintelligent) and by their functionality (Reactive, Limited Memory, Theory of Mind, Self-Aware) all of which are a function of the compute power available to run the algorithms and memory for data.

AI device compute and memory requirements vary significantly by use case, with training large models demanding high-end GPUs, extensive RAM (64GB+), and fast storage, while smaller or inference tasks can be handled by CPUs, integrated NPUs, and less powerful GPUs with lower memory needs. For enterprise and high-performance tasks, multiple powerful GPUs, 128GB+ RAM, and large storage are common, whereas AI-powered laptops or PCs for personal use may only require 16GB+ RAM and an integrated NPU. It has to be understood that connectivity has limited scope in an AI device as either most compute are local or when connectivity is necessary, the 6G radio connectivity should be sufficient for the purpose.

Observation 2: AI devices have more to do with available compute power / memory in a device and have limited bearing on connectivity or will anyway be covered as part of 6G eMBB type. Commercial factors will dictate the use of AI in devices and as such 3GPP shall avoid defining any AI type device.

XR and VR devices have limited mobility usage scenarios, and such devices are typically used in limited mobility scenarios.  Most of these device types are for at-home use and often focus on maximum immersion, high graphical fidelity (often via PC tethering), and room-scale tracking. Tethered & High Performance are still the norm in such devices, many systems (like older Oculus Rift or HTC Vive models) connect via cable to a high-end PC, which provides superior processing power, leading to higher resolution, refresh rates, and more realistic simulations.

Most use cases such as High-Fidelity Gaming & Entertainment (immersive gaming experiences and virtual cinemas), Content Creation & Design (architecture visualization, or design work where powerful rendering is crucial), Intensive Training & Simulation and Virtual Shopping & Interior Design are all scenarios with limited mobility considerations.

Even for XR Glasses which find use in mobile scenarios, it’s unclear if there is a need to define a new 3GPP device type. 6G eMBB or 6G IOT type (or even LTE or 5G modems) should be sufficient to provide connectivity.

Observation 3: Typical XR and VR device types have limited mobility considerations, and it’s best avoided as a new device type.

It is proposed to consider the following device types as a starting point and further discuss whether more device types would be needed.

- Massive IoT device (e.g., tracker, wearable devices, meters)

  - Reduced device complexity and power consumption as a focus.

- Mobile broadband device type (eMBB)

  - Mobility (speed)

- Ultra Broadband devices (FWA/CPE type)

  - Less constraints due to device form factors (Max BW, Max MIMO Layers etc)

- Sensing Device Type

Proposal 1: Considering ONLY 4 BROAD CATAGORIES of devices for 6GR

1) Massive IoT device, 2) Broadband device 3) Ultra Broadband devices (Specifically FWA/CPE/XR) 4) Sensing Device Category as a starting point.

Proposal 2: Devices shall not be defined primarily by their hardware capabilities or performance requirements (No of TX/RX chains, Power class, Modulation order, energy efficiency etc), these shall be features of the above defined 4 categories.

# 3	Conclusion

In this contribution we made the following observations and proposals:

Observation 1: RedCap as a category has not succeeded in the market for 5G. There should be no additional efforts to define another such category in 6G.

Observation 2: AI devices have more to do with available compute power / memory in a device and have limited bearing on connectivity or will anyway be covered as part of 6G eMBB type. Commercial factors will dictate the use of AI in devices and as such 3GPP shall avoid defining any AI type device.

Observation 3: Typical XR and VR device types have limited mobility considerations, and it’s best avoided as a new device type.

Proposal 1: Considering ONLY 4 BROAD CATAGORIES of devices for 6GR; Massive IoT device, Broadband device type, Ultra Broadband devices (Specifically FWA/CPE/XR), Sensing Device Category as a starting point.

Proposal 2: Devices (categories) shall not be defined primarily by their hardware capabilities or performance requirements (No of TX/RX chains, Power class, Modulation order, energy efficiency etc), these shall be features within the above defined 4 categories.

# References

- RP-252873. “Moderator’s summary on 6G diverse device types (incl. minimum UE bandwidth). Apple (moderator). RANP#109. Beijing. September 2025.