3GPP TSG RAN Meeting #110					    RP-253452

Baltimore, U.S.A., December 8th – 11th, 2025

Agenda Item:	9.7

Source:            CATT

Title:	Discussion on TN PDCCH repetition

Document for:	Decision

1	Introduction

In the RAN #108[1], it was agreed that common PDCCH repetition is applied in TN network. Based on this agreement, RAN1 has finished the specification modification corresponding to TN PDCCH repetition. However, for NTN, the PDCCH repetition is linked to SIB1 PDSCH repetition. For initial access stage, before read the SIB1 and SIB19, UE can’t identify serving cell is NTN cell or TN cell. It means even if one TN UE can receive the type0 PDCCH repetition, but this UE can’t determine how to treat SIB1 PDSCH before this UE knows this cell is NTN cell or TN cell, because NTN UE will automatically detect repeated SIB1, while TN UE may have two choices. It will cause two different UE behaviors and related specification changes.

This contribution presents our views of common PDCCH repetition and SIB1 repetition for TN UE.

- Discussion

In Rel-19, there is 1 bit in PBCH to indicate the repetition of Type0 PDCCH, which is applicable for both TN and NTN following the agreements of RAN #108. For NTN UE, SIB1 PDSCH transmission is not required with independent indication, and it will assume SIB1 repetition is associated with Type0 PDCCH repetition. But RAN #108 didn’t clearly say the SIB1 repetition will follow PDCCH repetition. Now it causes one technical issue.

If the TN UE doesn’t support SIB1 PDSCH repetition, it will have different behavior with NTN UE. Actually the motivation to support PDCCH repetition for TN UE is to improve the coverage of TN cell. Due to big SIB1 Packet size with more than 1000 bytes, the coverage issue is worse than PDCCH based on R19 NTN discussion. Hence, technically it has no reason to support PDCCH repetition but not support SIB1 repetition. This is same reason to support SIB1 enhancement for NTN UE.

Observation 1：SIB1 coverage is worse than that of Type0 PDCCH for TN cell.

Furthermore, if one TN cell supports PDCCH repetition but not supports SIB1 repetition, it means that the UE needs to determine whether SIB1 PDSCH is repeatedly transmitted or not. If one UE is able to support the access to NTN cell and TN cell, before UE reads SIB19, UE can’t identify current cell is NTN cell or TN cell. Network should have another indication to tell UE if current cell transmits the SIB1 repeated or not before SIB19 reception, for example via PBCH signalling. Then it will require additional signaling design and related specification change. PBCH change should be avoided as much as possible. Another issue is the ambiguity issue in the timing relationship between the repeated PDCCH and the associated PDSCH carrying SIB1 because there are two PDSCHs but just one PDCCH. In order to support legacy UE, it is better to support that the first PDCCH will schedule one SIB1 PDSCH and the second PDCCH will not schedule the SIB1 PDSCH.

Observation 2: Significant specification change is needed if TN cell only supports type0 PDCCH repetition but not supports the SIB1 PDSCH repetition.

Based on above analysis, it is nature to support SIB1 repetition for TN UE without complicating network implementation and specification modification.

Therefore, SIB1 PDSCH repetition enhancement should also be applied to TN networks to ensure that TN UE can have proper operation.

Proposal 1:  SIB1 PDSCH repetition is applicable to both TN UE and NTN UE, and same association with Type0 PDCCH repetition is applied as well.

- Conclusion

In order to support effective coverage and simplify UE behavior, it is proposed to support SIB1 PDSCH repetition for TN UE when Type0 PDCCH repetition is configured.

Observation 1：SIB1 coverage is worse than that of Type0 PDCCH for TN cell.

Observation 2:  Significant specification change is needed if TN cell only supports type0 PDCCH repetition but not supports the SIB1 PDSCH repetition.

Proposal 1:  SIB1 PDSCH repetition is applicable to both TN UE and NTN UE, and same association with Type0 PDCCH repetition is applied as well.

- References

- RP-252864, Meeting Report for TSG RAN meeting #108