3GPP TSG-RAN WG Meeting #110	RP-253512

# Baltimore, USA, December 8-11, 2025

Title: 	Discussion on release independence for 6Rx

Source: 	Huawei, HiSilicon

Agenda item:	9.5.4.1

Document for:	Discussion/Decision

Background

6Rx with 4 MIMO layers, 6 MIMO layers and 3T6R SRS antenna switching are introduced in Rel-19. New UE capability signalling for max DL 6 MIMO layers for PDSCH that is only applicable for FWA is defined. Now whether to introduce 6Rx with 4 MIMO layers and 6 MIMO layers from earlier release than Rel-19 is still under discussion. In this contribution, we share our views on the 6Rx release independence issue.

Discussion

Principle of 3GPP release independence

The original motivation is to apply a new band introduced in a later release(s) to an earlier/frozen release(s) to meet an operator’s demand whose band is introduced in a later release. In fact, the title of the release-independent specifications reflects the original intent across the generations.

TS 25/36/38.307; Requirements on User Equipments (UEs) supporting a release-independent frequency band

Observation 1: The original motivation of TS 25/36/38.307 is to apply a new band introduced in a later release(s) to an earlier/frozen release(s) to meet an operator’s demand whose band is introduced in a later release.

Further, the prerequisite of the release-independent is that all the essential functionalities across WGs for a band are ready in the earliest release that the release-independent applies to.

Observation 2: The prerequisite is that all the essential functionalities across WGs for a band are ready in the earliest release that the release-independent applies to.

Currently, RAN4 TS 38.307 applies release-independent to features other than band/band combination. In principle, release-independent for a feature applies to the release down to the release where associated RAN1 and/or RAN2 requirements are introduced.

Observation 3: In principle, TS 38.307 applies release-independent to a feature other than band/band combination to the release down to the release where RAN1 with or without RAN2 requirements associated with the feature are introduced.

Although there are exceptional cases that release-independent applies to an earlier release than the release that the feature is introduced, most of the exceptional cases impact only RAN2 specification(s) on top of RAN4 specification(s), e.g., FR2 PC5 introduced in Rel-17 can be release-independent from Rel-15.

Observation 4: TS 38.307 has some exceptional cases, while most of the cases are no RAN1 impact, but only RAN2 impact, e.g., FR2 PC5 introduced in Rel-17 with RAN2 specifications impact, and the release-independent applies from Rel-15.

In order to make the discussion more specific to 6Rx, we take a look at 8Rx whose situation is similar to 6Rx. RAN4 requirements for 8Rx were introduced in Rel-18 [1], while the release-independent applies from Rel-17. In fact, Rel-18 RAN4 8Rx requirements contain features, e.g., SRS Antenna Switching (AS) that were introduced in Rel-17 as shown in Figure 1 and the conclusion of “Rel-17” follows Observation 3.

Figure 1: relationship b/w Rel-18 RAN4 8Rx, RAN1/2 specifications and its release-independent

Observation 5: RAN4 8Rx requirements were introduced in Rel-18 whose requirements contain features like SRS Antenna Switching (AS) e.g., t1r8 etc., that were introduced into RAN1/2 specifications in Rel-17. The release-independent for 8Rx applies from Rel-17 which follows Observation 3.

RAN4 discussion on 6Rx release independence

The last RAN4 discussed the 6Rx release-independent and RAN4 chairman provided a following clear guidance together with two Cases shown in Figure 2, though RAN4 couldn’t reach a consensus [3].

Figure 2: The latest guidance from RAN4 chairman

Observation 6: In the latest RAN4, RAN4 chairman provided two options (Case 1 and 3) to further consider as guidance.

- Case 1: 6Rx with up to 4 MIMO layers for handheld/FWA UE from Rel-18

  - There is no signaling impact

  - New requirements based on 6Rx need to be introduced

- Case 3: 6Rx with up to 6 MIMO layer for FWA UE only from Rel-19

  - There is signaling impact

  - New requirements based on 6Rx need to be introduced.

Though Case 1 says that it is from Rel-18, similar to Rel-18 RAN4 8Rx, actually, Rel-19 RAN4 6Rx contains features introduced in Rel-19 RAN1 and RAN2 specifications. For more specific, the agreed UE RF CR [2] to introduce 6Rx includes 3T6R SRS AS that is also introduced in Rel-19.

Figure 3: relationship b/w Rel-19 RAN4 6Rx, RAN1/2 specifications and its release-independent

Though we acknowledge that there are some exceptional cases than the principle for release-independent as observation 4, RAN4 6Rx are very similar to RAN4 8Rx and if we apply the same principle for 8Rx to 6Rx, it is natural to conclude that release-independent for RAN4 6Rx is from Rel-19, given that associated RAN1/2 specifications are introduced in Rel-19.

Observation 7: If the same logic for RAN4 8Rx is applied to RAN4 6Rx, it is natural to conclude that release-independent for RAN4 6Rx is from Rel-19, given that both cases have impact on associated RAN1/2 specifications introduced in Rel-19.

With the above analysis and observations, our proposal is as follows.

Proposal: Not introduce release independent for 6Rx to a release earlier than Rel-19 regardless of the number of MIMO layers.

Conclusion

In this contribution we provide our views on 6Rx release independence, our observations and proposal are:

Observation 1: The original motivation of TS 25/36/38.307 is to apply a new band introduced in a later release(s) to an earlier/frozen release(s) to meet an operator’s demand whose band is introduced in a later release.

Observation 2: The prerequisite is that all the essential functionalities across WGs for a band are ready in the earliest release that the release-independent applies to.

Observation 3: In principle, TS 38.307 applies release-independent to a feature other than band/band combination to the release down to the release where RAN1 with or without RAN2 requirements associated with the feature are introduced.

Observation 4: TS 38.307 has some exceptional cases, while most of the cases are no RAN1 impact, but only RAN2 impact, e.g., FR2 PC5 introduced in Rel-17 with RAN2 specifications impact, and the release-independent applies from Rel-15.

Observation 5: RAN4 8Rx requirements were introduced in Rel-18 whose requirements contain features like SRS Antenna Switching (AS) e.g., t1r8 etc., that were introduced into RAN1/2 specifications in Rel-17. The release-independent for 8Rx applies from Rel-17 which follows Observation 3.

Observation 6: In the latest RAN4, RAN4 chairman provided two options (Case 1 and 3) to further consider as guidance.

- Case 1: 6Rx with up to 4 MIMO layers for handheld/FWA UE from Rel-18

  - There is no signaling impact

  - New requirements based on 6Rx need to be introduced

- Case 3: 6Rx with up to 6 MIMO layer for FWA UE only from Rel-19

  - There is signaling impact

  - New requirements based on 6Rx need to be introduced.

Observation 7: If the same logic for RAN4 8Rx is applied to RAN4 6Rx, it is natural to conclude that release-independent for RAN4 6Rx is from Rel-19, given that both cases have impact on associated RAN1/2 specifications introduced in Rel-19.

Proposal: Not introduce release independent for 6Rx to a release earlier than Rel-19 regardless of the number of MIMO layers.

Reference

R4-2321791 draft CR for introduction of 8Rx UE RF requirements for TS 38.101-1, NTT DOCOMO, INC., et al., RAN4#109

R4-2511318 Big CR to TS38.101-1: Introduction of requirements for 6Rx, ZTE Corporation et al, RAN4#116

2.Revision of R4-2522279_after NWM.docx

Annex

- B.4.17	Common UE RF requirements for 8Rx

The requirements and test cases listed in Table B.4.17-1 are specified in REL-P version of TS 38.101-1 [2].

Table B.4.17-1: Common UE RF requirements for 8Rx for single band in FR1

| Clause | Description |
| --- | --- |
| 6.2.4 | Configured output power |
| 7.3 | Reference sensitivity |
| 7.4 | Maximum input level |
| 7.5 | Adjacent Channel Selectivity |
| 7.6 | Blocking characteristics |
| 7.7 | Spurious response |
| 7.8 | Intermodulation characteristics |
| 7.9 | Spurious emissions |