3GPP TSG RAN Meeting #110						           RP-253515

Baltimore, USA, December 15-18th, 2025

Agenda Item:	8.2.1.0

Source:	Apple

Title:	Views on deployment scenarios

Document for:	Discussion/Decision

# Introduction

The latest RAN 6G SID is approved in RP-251395 [1], was approved. The SID covers mainly two aspects of the 6G study, i.e., (1) study on ITU IMT-2030 TPR (Technical Performance Requirement), (2) study of 6G in general. Regarding the study of 6G in general, the objectives from RP-251395 can be found below:

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework

At RAN #109, it was noted that RP-252915 (“pCR for 38.914 on Deployment scenarios for 6G (Moderator: Deutsche Telekom) captures aspects that be used as starting point for further discussion at RAN #110.

In this contribution, we provide our views on deployment scenarios for 6G.

# Deployment scenarios for 5G and 6G

From RP-252915, the open issues include

- Network layout and ISD

- UE antenna elements

- Multi-TRP operation

For 1), using Urban macro related information from RP-252915 as an example:

| Layout | Single layer: - Hex. Grid Two layers - Macro layer: Hex. Grid - Micro layer: Random drop |
| --- | --- |
| ISD | Single layer: 500m Macro layer: 500m 500m Micro layer: At least 144.8m 250m, 6 [3] micro TRxPs per macro TRxP NOTE4, All micro TRxPs are all outdoor All micro TRxPs are all outdoor |

It will be good to clarify the carrier frequency combinations for the deployments, e.g., (Macro layer with the hexagon layout at 4 GHz, and micro layer with random drop at 4 GHz), (Macro layer with the hexagon layout at 4 GHz, and micro layer with random drop at 7 GHz), … Also for micro layer dropping, purely from radio performance point of view, micro cells can be dropped to places where coverage from macro cells is challenging, yet from cell site acquisition point of view, it may not work so conveniently in the real world. Thus, operators are encouraged to weigh different factors and provide a recommendation. We

Proposal 1: Operators should provide a selected combinations of network layout for 6G design.

For 2), using Urban macro related information from RP-252915 as an example:

| UE antenna elements NOTE3 | Around 700 MHz: 1T2R Up to 2 4 Tx and Rx antenna elements Around 2 GHz: 2T4R Around 4 GHz: 2T4R Around 7 GHz: up to 4T8R Around 2 GHz, around 4 GHz and around 7 GHz: Up to 8 Tx and Rx antenna elements Note Note: Possible device types include handheld device, CPE, vehicle, sensing dedicated device etc. Configurations of antenna element more than 2Tx and 4Rx are not necessarily applicable to smart phone. Around 2 GHz: Up to 4 8 Tx and Rx antenna elements Around 4 GHz: Up to 8 8 8 Tx and Rx antenna elements Around 7 GHz and around 15 GHz: Up to [8] 8 16 Tx and Rx antenna elements Around 30 GHz: Up to 32 32 32 Tx and Rx antenna elements |
| --- | --- |
|  | FWA: 4Tx and 8 Rx (Around 2GHz / 4GHz / 7GHz) eMBB UE “handheld” device (used in eMBB, HRLLC, Positioning): • Around 2 GHz: Up to 2 Tx and 4 Rx antenna elements • Around 4 GHz: Up to 4 Tx and 4 Rx antenna elements • Around 7 GHz: Up to 4 Tx and 8 Rx antenna elements • Around 30 GHz: Up to 8 Tx and 8 Rx antenna elements Sensing TxRx “non-handheld” device (used in Sensing): • Around 4 GHz: Up to 8 Tx and 8 Rx antenna elements • Around 7 GHz: Up to 8 Tx and 8 Rx antenna elements • Around 30 GHz: Up to 32 Tx and 32 Rx antenna elements MTC device (used in MTC): • Around 2 GHz: 1 Tx/Rx antenna elements • Around 4 GHz: 1 Tx and up to 2 Rx antenna elements • Around 7 GHz: 1 Tx and up to 2 Rx antenna elements |

NOTE3:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

As the typical antenna configurations for handheld UEs and CPEs can be rather different, then number of Tx/Rx assumed should be designated with the intended usage un-equivocally, e.g., handheld UEs or CPEs. First, we need to identify the typical antenna configurations at various carrier frequencies. Beyond that, even though we don’t expect 6Rx to be utilized for all handheld UEs at 4 GHz or 7 GHz, 6Rx can enable higher user perceived throughput and better user experience with a handheld UE; so evaluation on 6Rx can considered at relevant frequencies for handheld UEs. The same logic applies to 4Rx for 700 MHz and 2 GHz, thus we are open to its evaluation study.

We have

Proposal 2: For handheld UEs, the following antenna configurations are considered for performance evaluations:

- 700 MHz, 1Tx2Rx, optional: 1Tx4Tx

- 2 GHz, 1Tx2Rx, 2Tx2Rx, optional: 1Tx4Rx, 2Tx4Rx, 1Tx6Rx, 2Tx6Rx

- 4 GHz, 1Tx4Rx, 2Tx4Rx, optional: 1Tx6Rx, 2Tx6Rx

- 7 GHz, 1Tx4Rx, 2Tx4Rx, optional: 1Tx6Rx, 2Tx6Rx

- 15 GHz, 1Tx2Rx

- 30 GHz, 1Tx2Rx

mTRP technology can be an important enabler of ubiquitous coverage for desired services; however, there are different variants of mTRP technology, such as coherent joint transmission, non-coherent joint transmission, and dynamic point switching. Practical considerations may influence the relative usefulness of each, so decisions on whether one or multiple technologies should be supported in 6G from day one need to be well justified by feasible deployment scenarios.

In our view, the network backhaul assumptions should be clearly stated for each scenario—for example, whether ideal backhaul can be assumed in dense urban or urban macro environments. Similarly, timing, frequency, and phase synchronization assumptions—which may be loosely referred to as fronthaul assumptions—also have a major impact on mTRP design.

We understand that different mobile operators may face varying practical conditions, such as access to dark fiber. Even so, enumerating the prevailing scenarios would be more valuable than leaving these assumptions entirely to a working group. While providing information for scenarios with little or no current deployment can be challenging, offering practical or at least plausible assumptions—particularly for well-studied environments like dense urban and urban macro—can be extremely useful. Thus, we have

Proposal 3: In TR 38.914, capture network backhaul assumptions (e.g., ideal, non-ideal with 10 ms latency) and TRP fronthaul synchronization assumptions such as timing/frequency/phase at least for some scenarios including dense urban and urban macro.

# Conclusion

In this contribution, we provide our views on 6G deployment scenarios, with the following proposals regarding

Proposal 1: Operators should provide a selected combinations of network layout for 6G design.

Proposal 2: For handheld UEs, the following antenna configurations are considered for performance evaluations:

- 700 MHz, 1Tx2Rx, optional: 1Tx4Tx

- 2 GHz, 1Tx2Rx, 2Tx2Rx, optional: 1Tx4Rx, 2Tx4Rx, 1Tx6Rx, 2Tx6Rx

- 4 GHz, 1Tx4Rx, 2Tx4Rx, optional: 1Tx6Rx, 2Tx6Rx

- 7 GHz, 1Tx4Rx, 2Tx4Rx, optional: 1Tx6Rx, 2Tx6Rx

- 15 GHz, 1Tx2Rx

- 30 GHz, 1Tx2Rx

Proposal 3: In TR 38.914, capture network backhaul assumptions (e.g., ideal, non-ideal with 10 ms latency) and TRP synchronization assumptions such as timing/frequency/phase at least for some scenarios including dense macro and urban macro.

# Reference

[1] RP-251395, Revised SID: Study on 6G Scenarios and requirements, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2] 3GPP TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies (Release 14)