3GPP TSG RAN Meeting #110	RP-253523

Baltimore, USA, December 8-11, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: |  | CR for TS 38.214 on clarification on SIB1 PDSCH repetition |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  | vivo |  |  |  |  |  |  |  |
| Work item code: |  | NR_NTN_Ph3-core |  |  |  |  | Date: |  | 2025-11-26 |
| Category: |  | F |  |  |  |  | Release: |  | Rel-19 |
|  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: | SIB1 PDSCH repetition is only applicable to FR1-NTN. However, the current RAN1 spec may lead to confusion that inter-slot SIB1 PDSCH repetition is also applicable to TN and FR2-NTN. |  |  |  |  |  |  |  |  |
| Summary of change: | In the section of 5.1 of TS 38.214, adding relevant descriptions of SIB1 PDSCH repetition is applicable to only NTN in FR1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: | It can lead to confusion that the repetition of SIB1 PDSCH is supported by the TN cell and FR2-NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: | 5.1 |  |  |  |  |  |  |  |  |
|  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

### 5.1	UE procedure for receiving the physical downlink shared channel

============== unchanged parts are omitted ===============

A UE capable of PDSCH repetitions for broadcast channels for a NTN serving cell in FR1, which assumed the DCI format 1_0 in the Type0 PDCCH CSS of searchSpaceZero transmitted with two inter-slot repetitions may assume that PDSCHs scheduled by the DCI format 1_0 have also been transmitted with inter-slot repetitions in the same slots as the Type0 PDCCH CSS, with the same RV as indicated by the DCI format 1_0.

============== unchanged parts are omitted ===============