3GPP TSG RAN Meeting #110                                                       RP-253523

Baltimore, USA, December 8-11, 2025

Source:	vivo

Title:	Clarification on applicability of Rel-19 NR NTN common PDCCH repetition for TN

Agenda Item:	9.9.2.2

Document for:	Discussion and Decision

Introduction

In the RAN#108 meeting [1], the following proposal in [2] was endorsed:

Proposal：Common PDCCH repetition introduced in Rel-19 NR NTN WI is also applicable for TN for FR1 only

Proposal：Common PDCCH repetition introduced in Rel-19 NR NTN WI is also applicable for TN for FR1 only

However, during the CR drafting in WGs, companies had different views on the meaning of the above proposal, and consequently, different proposals on capturing the agreements in the specifications. In the contribution, the current situation in RAN1 WG is provided.

Discussion

In the RAN#108 meeting, the proposal in [2] was endorsed, i.e., Common PDCCH repetition introduced in Rel-19 NR NTN WI is also applicable for TN for FR1 only. Moreover, in the RAN1#122 meeting, the RAN1 chairman also provided the following guidance for NTN in R1-2505101:

Note: This proposal is taken as an exception. According to RANP clarification, solutions achieved in items targeting to NTN can’t be adopted by TN by default in WGs. TEI/WI should be proposed for applying such solutions to TN additionally.

However, during the CR drafting in RAN1 WG, it appeared that companies had different views on the meaning of the RANP agreement [3], as cited below.

Companies are invited to comment on the two proposed options: WF1; clarify in the specifications that SIB1 PDSCH repetition does not apply to TN or FR2-NTN (by amending 38.213); and WF2; support SIB1 repetition for TN in line with NTN, with no additional specification changes required. Based on the above discussion the following initial proposal is made Proposal 1-v0 WF1: Clarify in specs that SIB1 PDSCH repetition does not apply to TN/FR2-NTN (amend 38.213 for explicit statement) WF2: Support SIB1 repetition for TN, same implementation as for NTN; no additional specification changes required..

Some companies understand that, based on the RAN plenary’s decision, only the feature of “common PDCCH repetition” has been agreed to be extended to TN. On the other hand, given that enabling of Type-0 CSS PDCCH repetitions for SIB1 and PDSCH repetitions for SIB1 are coupled as agreed in the RAN1#121 meeting, some other companies believe that, the RAN plenary’s decision implicitly also applies to SIB1 PDSCH repetition, i.e., not only common PDCCH repetition, but also SIB1 PDSCH repetition introduced in Rel-19 NR NTN are also applicable for TN for FR1.

Agreement (in the RAN1#121 meeting, Malta, May 2025) The enabling/disabling of SIB1 PDSCH repetition is implicitly indicated by the enabling/disabling of Type-0 CSS PDCCH repetition.

The current RAN1 specification leads to the confusion of SIB1 PDSCH repetition introduced in Rel-19 NR NTN are also applicable for TN for both FR1 and FR2. The details of the ambiguity is provided in the Annex.

In the last quarter, several rounds of online and offline discussions happened to address this issue, but no common understanding was achieved. According to the Chair’s guidance, a clarification in RAN plenary is required in order to progress RAN1’s work.

Therefore, it is proposed that RAN plenary clarifies the intention of the endorsement of RP-251857, i.e., whether SIB1 PDSCH repetition introduced in Rel-19 NR NTN are also applicable for TN for FR1.

Proposal 1: RAN plenary is requested to clarify the intention of the endorsement of “Applicability of Rel-19 common PDCCH repetition for terrestrial networks” in RP-251857, specifically, whether SIB1 PDSCH repetition introduced in Rel-19 NR NTN are also applicable for TN for FR1.

In case that SIB1 PDSCH repetition is only applicable to FR1-NTN, a company CR is attached to address this issue.

Proposal 2: In case that SIB1 PDSCH repetition is only applicable to FR1-NTN, agree the attached company CR to address this issue.

Conclusions

In this contribution, we present the background information and the current situation in the RAN1 WG regarding the how to capture the RAN plenary’s decision in specification, and have the following proposal:

Proposal 1: RAN plenary is requested to clarify the intention of the endorsement of “Applicability of Rel-19 common PDCCH repetition for terrestrial networks” in RP-251857, specifically, whether SIB1 PDSCH repetition introduced in Rel-19 NR NTN are also applicable for TN for FR1.

Proposal 2: In case that SIB1 PDSCH repetition is only applicable to FR1-NTN, agree the attached company CR to address this issue.

References

- RP-252864, “Meeting Report for TSG RAN meeting #108”, MCC.

- RP-251857, “Applicability of Rel-19 common PDCCH repetition for terrestrial networks”, Huawei, HiSilicon, China telecom, Apple, OPPO, Lenovo.

- R1-2509490, “FL Summary #1 - Maintenance on NR-NTN downlink coverage enhancements”, Moderator (Thales).

- Annex: The ambiguity of the RAN1 specification

Specifically, the current RAN1 specification (highlighted below from 38.214), which is for the Rel-19 SIB1 PDSCH repetition for NTN, would lead to confusion that SIB1 PDSCH repetition is also supported for TN.

A UE capable of PDSCH repetitions for broadcast channels, which assumed the DCI format 1_0 in the Type0 PDCCH CSS of searchSpaceZero transmitted with two inter-slot repetitions may assume that PDSCHs scheduled by the DCI format 1_0 have also been transmitted with inter-slot repetitions in the same slots as the Type0 PDCCH CSS, with the same RV as indicated by the DCI format 1_0.

The confusing point is that, starting from Rel-15, the UE already supports broadcast channel (for example, SIB1) PDSCH repetition, as shown below from the TS38.331 v15.29.0, i.e., the latest Rel-15 spec.

- the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13).

Consequently, combining the above specifications, it may be misunderstood that every NR UE operating in FR1 or FR2 TN, as well as NR UEs operating in FR2-NTN, should support the inter-slot PDSCH repetitions, which is violating both the RAN plenary’s decision and the RAN1 chair guidance.