3GPP TSG-RAN #110	RP-253525

Baltimore, USA, 8-11 December 2025

Agenda Item:	8.2.2

Source:	Ericsson

Title:	6G Positioning Architecture

Document for:	Discussion

# Introduction

In a separate contribution [1] we discuss and propose that 6GR and 6G RAN shall support A-GNSS and E-CID positioning methods.

This contribution details the possible 6G positioning architecture for the above proposal. We also propose the corresponding updates to the approved 6G SID [2].

# Discussion

As already mentioned, [1] E-CID and A-GNSS are good candidates to meet the applicable positioning requirements and to ensure positioning is available in 6G for Rel-21. Both methods have the key benefits of minimizing UE impact; furthermore, E-CID is based on reporting serving cell ID from the RAN node, reusing existing RRM measurements and has been improved over the past NR releases to report additional TA/AoA/Multi-TRPs AoA measurements from the serving RAN node. [3][7]

In addition, given that both methods rely on tried and tested functionality and are based on a consolidated architecture since many releases, it seems feasible to natively support both in 6G RAN.

Observation 1: E-CID and A-GNSS are good candidates to ensure 6G positioning is available in Rel-21.

Proposal 1: Due to their limited impact and tried-and-tested characteristics, it seems feasible to natively support both E-CID and A-GNSS in 6G RAN in Rel-21.

We aim to make the first iteration of 6G as complete as possible, including satisfying IMT-2030 positioning requirements, without undue burden in the study phase. Inclusion of additional positioning methods in the 6GR framework, if justified by satisfying IMT-2030 positioning requirements, should not be precluded, but it can be done by RAN WGs directly in the WI phase.

Proposal 2: It should not be precluded to select and if needed specify any additional positioning methods, if justified, directly in the WI phase.

## Potential 6G Positioning Architecture

In order to support E-CID and A-GNSS in 6G RAN, the suitable positioning architecture needs to be assumed.

Figure 1 below shows the 6G RAN architecture figure, agreed for inclusion [6] in TR 38.760-3.

Figure 1 6G RAN Architecture as currently agreed for inclusion [6] in TR 38.760-3.

In order to support E-CID and A-GNSS in 6G, we need to assume at least 3 elements in 6G architecture:

- An appropriate logical node in the CN for 6G (in 5GC, this is the LMF).

- The 6G RAN Node terminates an appropriate protocol toward such logical node (needed for E-CID - in NG-RAN, this is the NRPPa protocol).

- The UE terminates an appropriate protocol toward such logical node (needed for A-GNSS –in NG-RAN, this is the LPP protocol)

Such assumptions will have to be eventually checked between RAN3, RAN2 and SA2; they result in Figure 2 below. The figure shows NG-RAN nomenclature in quotes for ease of reference.

Figure 2 Assumed 6G logical architecture for positioning.

Notice that Figure 2 shows the logical architecture; the transport for “NRPPa” and “LPP” between the respective endpoints is not shown here.

Observation 2: In order to support E-CID and A-GNSS in 6G, at a minimum we should assume an appropriate logical node in the CN for 6G, terminating the appropriate protocols to the 6G RAN Node (for E-CID) and to the UE (for A-GNSS), respectively.

In NG-RAN, NRPPa is transported between the NG-RAN Node and the LMF through the AMF, and LPP is transported between the UE and the LMF through the NG-RAN node and the AMF via NAS. This results in the current overall NG-RAN positioning architecture [7].

In principle, similar assumptions on transporting the positioning protocols could be made for 6G RAN (also to be eventually checked between RAN3, RAN2 and SA2); if so, the resulting positioning architecture would be similar.

Observation 3: If the same assumptions on positioning protocol transport involving UE, 6G RAN Node, and CN for 6G are made as for NG-RAN, the resulting positioning architecture is very similar.

It seems then feasible to include the 6G positioning architecture, with the above understanding, in the scope of the ongoing 6G study. This does not require a dedicated study for 6G positioning and will ensure that the functionality needed can be specified in Rel-21. The minimum set of assumptions is to be eventually checked between RAN3, RAN2 and SA2 during the course of the study (as for all other aspects, as already stated in the SID [2]).

Proposal 3: Include the 6G positioning architecture in the scope of the ongoing 6G study, supporting E-CID and A-GNSS, with the minimum set of assumptions.

## Considerations on Periodic Positioning

Given the general trends towards network densification and penetration, some concerns on scalability of positioning protocol transport may arise, especially when considering periodic positioning (part of current E-CID functionality). In order to address such concerns, it seems appropriate to consider periodic positioning measurements as an additional data type within the 6G study aspects on “data transfer design to support various types of data” [2]. In other words, it seems appropriate to study the possibility of transferring e.g. periodic positioning measurements directly from the 6G RAN Node to the appropriate CN positioning node, instead of relying on CP signaling.

Proposal 4: For better scalability, periodic positioning measurements should be considered as an additional data type that may be transferred directly from 6G RAN to CN instead of relying on CP signaling.

## Additional Considerations

We should note that E-CID was added to NG-RAN as a “copy/paste” from LTE, then improved to support gNBs based on NR signals and NR measurements. Here we propose to study native 6G support for E-CID and A-GNSS relying on suitable 6G protocols and RRM measurements. Additional methods, if justified, are not to be precluded in the WI phase, as per our proposal in Sec. 2 above.

Proposal 5: Study native 6G support for E-CID and A-GNSS relying on suitable 6G protocols and RRM measurements.

Proposal 6: The relevant updates to the approved 6G SID are proposed in [9].

# Conclusions and Proposals

Our observations and proposals are summarized below.

Observation 1: E-CID and A-GNSS are good candidates to ensure 6G positioning is available in Rel-21.

Proposal 1: Due to their limited impact and tried-and-tested characteristics, it seems feasible to natively support both E-CID and A-GNSS in 6G RAN in Rel-21.

Proposal 2: It should not be precluded to select and if needed specify any additional positioning methods, if justified, directly in the WI phase.

Observation 2: In order to support E-CID and A-GNSS in 6G, at a minimum we should assume an appropriate logical node in the CN for 6G, terminating the appropriate protocols to the 6G RAN Node (for E-CID) and to the UE (for A-GNSS), respectively.

Observation 3: If the same assumptions on positioning protocol transport involving UE, 6G RAN Node, and CN for 6G are made as for NG-RAN, the resulting positioning architecture is very similar.

Proposal 3: Include the 6G positioning architecture in the scope of the ongoing 6G study, supporting E-CID and A-GNSS, with the minimum set of assumptions.

Proposal 4: For better scalability, periodic positioning measurements should be considered as an additional data type that may be transferred directly from 6G RAN to CN instead of relying on CP signaling.

Proposal 5: Study native 6G support for E-CID and A-GNSS relying on suitable 6G protocols and RRM measurements.

Proposal 6: The relevant updates to the approved 6G SID are proposed in [9].

# References

RP-253093	Requirements for regulatory services and positioning, Ericsson, Airbus, AT&T, BDBOS, BMW, Deutsche Telekom, FirstNet, Erillisverkot, KT Corp., Motorola Solutions, Nokia, NTT DOCOMO, Softil, SyncTechno Inc, T-Mobile USA, Telstra.

RP-252912	Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone.

X. Lin, N. Lee, ed., 5G and Beyond – Fundamentals and Standards, Springer, 2021.

TS 36.305 v. 9.10.0.

TS 38.305 v. 15.10.0.

R3-258870	(pCR for 38.760-3) RAN Architecture, InterDigital, Samsung, Qualcomm, Ericsson, Nokia, LG Electronics, Huawei, Vodafone, NEC, NTT Docomo, Rakuten, CMCC, Ofinno, Lenovo, BT, Google, Verizon, Tejas Networks, China Telecom, FiberCop, Xiaomi, Deutsche Telekom, ZTE, Jio Platforms, CATT, OPPO, Samsung, T-Mobile USA.

TS 38.305 v. 19.0.0.

TS 36.305 v. 19.0.0.

RP-253526	Revised SID: Study on 6G Radio, Ericsson.

E-CID and A-GNSS were first introduced in Rel-9 for E-UTRAN [4] and in Rel-15 for NG-RAN [5].

The following assumptions are also part of the foundation for the NG-RAN and E-UTRAN positioning architectures. [7][8]