3GPP TSG-WG5 Meeting #109	R5-256375

# Dallas, USA, Nov 17 – 21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3667 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding test frequency for CA_n78C BCS1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The configuration of CA_n78C BSC1 is introduced into TS 38.101-1 but the test frequencies are still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the test frequency information CA_n78C BSC1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test frequency information remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.3.78.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

4.3.1.1.3.78.1	CA_n78C

Editor's note: Test frequencies for CA_n78C with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.78.1-1: NR Intra-Band contiguous CA configuration CA_n78C (PCC=CC1 and SCC=CC2), SCS 30 kHz

| CBW combination (BCS) | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10+100 | CC1 | 10 | 30 | 24 | Downlink | Low | 3305.01 | 620334 | 3300.69 | 620046 | 0 | 30 |  | 7711 | 620352 |  | 18 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3500.01 | 633334 | 3458.97 | 630598 | 102 |  |  | 7846 | 633312 |  | 2 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3695.4 | 646360 | 3509.64 | 633976 | 504 |  |  | 7982 | 646368 |  | 8 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=54.6 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3359.61 | 623974 | 3310.47 | 620698 | 0 | 30 |  | 7718 | 621024 |  | 14 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3554.61 | 636974 | 3468.75 | 631250 | 102 |  |  | 7853 | 633984 |  | 22 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 | 0 | 3 (3) | 1014 |
| 15+90 | CC1 | 15 | 30 | 38 | Downlink | Low | 3307.5 | 620500 | 3300.66 | 620044 | 0 | 30 |  | 7711 | 620352 |  | 20 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3504.99 | 633666 | 3461.43 | 630762 | 102 |  |  | 7848 | 633504 |  | 6 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3702.9 | 646860 | 3514.62 | 634308 | 504 |  |  | 7985 | 646656 |  | 12 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=52.08 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3359.58 | 623972 | 3315.48 | 621032 | 0 | 30 |  | 7721 | 621312 |  | 16 | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3557.07 | 637138 | 3476.25 | 631750 | 102 |  |  | 7858 | 634464 |  | 2 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 |  |  | 7996 | 647712 |  | 8 | 0 | 3 (3) | 1014 |
| 15+100 | CC1 | 15 | 30 | 38 | Downlink | Low | 3307.5 | 620500 | 3300.66 | 620044 | 0 | 30 |  | 7711 | 620352 |  | 20 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3500.01 | 633334 | 3456.45 | 630430 | 102 |  |  | 7844 | 633120 |  | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3692.88 | 646192 | 3504.6 | 633640 | 504 |  |  | 7978 | 645984 |  | 8 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=57.12 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3364.62 | 624308 | 3315.48 | 621032 | 0 | 30 |  | 7721 | 621312 |  | 16 | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3557.13 | 637142 | 3471.27 | 631418 | 102 |  |  | 7855 | 634176 |  | 22 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 | 0 | 3 (3) | 1014 |
| 20+90 | CC1 | 20 | 30 | 51 | Downlink | Low | 3310.02 | 620668 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3504.99 | 633666 | 3459.09 | 630606 | 102 |  |  | 7846 | 633312 |  | 18 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3700.08 | 646672 | 3509.46 | 633964 | 504 |  |  | 7982 | 646368 |  | 20 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=54.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3364.92 | 624328 | 3320.82 | 621388 | 0 | 30 |  | 7725 | 621696 |  | 20 | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3559.89 | 637326 | 3479.07 | 631938 | 102 |  |  | 7860 | 634656 |  | 6 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 |  |  | 7996 | 647712 |  | 8 | 0 | 3 (3) | 1014 |
| 20+100 | CC1 | 20 | 30 | 51 | Downlink | Low | 3310.02 | 620668 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3500.01 | 633334 | 3454.11 | 630274 | 102 |  |  | 7843 | 633024 |  | 14 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3690.06 | 646004 | 3499.44 | 633296 | 504 |  |  | 7975 | 645696 |  | 16 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=59.94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3369.96 | 624664 | 3320.82 | 621388 | 0 | 30 |  | 7725 | 621696 |  | 20 | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3559.95 | 637330 | 3474.09 | 631606 | 102 |  |  | 7857 | 634368 |  | 2 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 | 0 | 3 (3) | 1014 |
| 25+80 | CC1 | 25 | 30 | 65 | Downlink | Low | 3312.51 | 620834 | 3300.81 | 620054 | 0 | 30 |  | 7711 | 620352 |  | 10 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3510 | 634000 | 3461.58 | 630772 | 102 |  |  | 7848 | 633504 |  | 20 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3707.67 | 647178 | 3514.53 | 634302 | 504 |  |  | 7985 | 646656 |  | 18 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=52.32 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3364.83 | 624322 | 3325.77 | 621718 | 0 | 30 |  | 7728 | 621984 |  | 2 | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3562.32 | 637488 | 3486.54 | 632436 | 102 |  |  | 7865 | 635136 |  | 12 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 | 0 | 3 (3) | 1014 |
| 25+90 | CC1 | 25 | 30 | 65 | Downlink | Low | 3312.51 | 620834 | 3300.81 | 620054 | 0 | 30 |  | 7711 | 620352 |  | 10 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3504.99 | 633666 | 3456.57 | 630438 | 102 |  |  | 7845 | 633216 |  | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3697.62 | 646508 | 3504.48 | 633632 | 504 |  |  | 7978 | 645984 |  | 16 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=57.36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3369.87 | 624658 | 3325.77 | 621718 | 0 | 30 |  | 7728 | 621984 |  | 2 | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3562.35 | 637490 | 3481.53 | 632102 | 102 |  |  | 7862 | 634848 |  | 10 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 |  |  | 7996 | 647712 |  | 8 | 0 | 3 (3) | 1014 |
| 25+100 | CC1 | 25 | 30 | 65 | Downlink | Low | 3312.51 | 620834 | 3300.81 | 620054 | 0 | 30 |  | 7711 | 620352 |  | 10 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3500.01 | 633334 | 3451.59 | 630106 | 102 |  |  | 7841 | 632832 |  | 14 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3687.6 | 645840 | 3494.46 | 632964 | 504 |  |  | 7971 | 645312 |  | 12 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=62.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3374.91 | 624994 | 3325.77 | 621718 | 0 | 30 |  | 7728 | 621984 |  | 2 | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3562.41 | 637494 | 3476.55 | 631770 | 102 |  |  | 7858 | 634464 |  | 6 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 | 0 | 3 (3) | 1014 |
| 30+80 | CC1 | 30 | 30 | 78 | Downlink | Low | 3315 | 621000 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3510 | 634000 | 3459.24 | 630616 | 102 |  |  | 7846 | 633312 |  | 8 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3705.15 | 647010 | 3509.67 | 633978 | 504 |  |  | 7982 | 646368 |  | 6 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=54.84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3369.84 | 624656 | 3330.78 | 622052 | 0 | 30 |  | 7732 | 622368 |  | 4 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3564.84 | 637656 | 3489.06 | 632604 | 102 |  |  | 7867 | 635328 |  | 12 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 | 0 | 3 (3) | 1014 |
| 30+90 | CC1 | 30 | 30 | 78 | Downlink | Low | 3315 | 621000 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3504.99 | 633666 | 3454.23 | 630282 | 102 |  |  | 7843 | 633024 |  | 6 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3695.1 | 646340 | 3499.62 | 633308 | 504 |  |  | 7975 | 645696 |  | 4 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=59.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3374.88 | 624992 | 3330.78 | 622052 | 0 | 30 |  | 7732 | 622368 |  | 4 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3564.87 | 637658 | 3484.05 | 632270 | 102 |  |  | 7864 | 635040 |  | 10 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 |  |  | 7996 | 647712 |  | 8 | 0 | 3 (3) | 1014 |
| 30+100 | CC1 | 30 | 30 | 78 | Downlink | Low | 3315 | 621000 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3500.01 | 633334 | 3449.25 | 629950 | 102 |  |  | 7839 | 632640 |  | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3685.08 | 645672 | 3489.6 | 632640 | 504 |  |  | 7968 | 645024 |  | 0 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3379.92 | 625328 | 3330.78 | 622052 | 0 | 30 |  | 7732 | 622368 |  | 4 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3564.93 | 637662 | 3479.07 | 631938 | 102 | 30 |  | 7860 | 634656 |  | 6 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 | 30 |  | 7989 | 647040 |  | 4 | 0 | 3 (3) | 1014 |
| 40+70 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3515.01 | 634334 | 3459.21 | 630614 | 102 | 30 |  | 7846 | 633312 |  | 10 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3710.16 | 647344 | 3509.64 | 633976 | 504 | 30 |  | 7982 | 646368 |  | 8 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=54.84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3374.85 | 624990 | 3340.83 | 622722 | 0 | 30 |  | 7739 | 623040 |  | 6 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3569.85 | 637990 | 3499.11 | 633274 | 102 | 30 |  | 7874 | 636000 |  | 14 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3765 | 651000 | 3549.54 | 636636 | 504 | 30 |  | 8010 | 649056 |  | 12 | 0 | 3 (3) | 1014 |
| 40+80 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3510 | 634000 | 3454.2 | 630280 | 102 | 30 |  | 7843 | 633024 |  | 8 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3700.17 | 646678 | 3499.65 | 633310 | 504 | 30 |  | 7975 | 645696 |  | 2 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=59.82 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3379.83 | 625322 | 3340.77 | 622718 | 0 | 30 |  | 7739 | 623040 |  | 10 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3569.82 | 637988 | 3494.04 | 632936 | 102 | 30 |  | 7871 | 635712 |  | 16 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 | 30 |  | 8003 | 648384 |  | 10 | 0 | 3 (3) | 1014 |
| 40+90 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3504.99 | 633666 | 3449.19 | 629946 | 102 | 30 |  | 7839 | 632640 |  | 6 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3690.18 | 646012 | 3489.66 | 632644 | 504 | 30 |  | 7968 | 645024 |  | 20 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=64.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3384.81 | 625654 | 3340.71 | 622714 | 0 | 30 |  | 7739 | 623040 |  | 14 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3569.79 | 637986 | 3488.97 | 632598 | 102 | 30 |  | 7867 | 635328 |  | 18 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 | 30 |  | 7996 | 647712 |  | 8 | 0 | 3 (3) | 1014 |
| 40+100 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3500.01 | 633334 | 3444.21 | 629614 | 102 | 30 |  | 7836 | 632352 |  | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3680.28 | 645352 | 3479.76 | 631984 | 504 | 30 |  | 7961 | 644352 |  | 8 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=69.72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3389.73 | 625982 | 3340.59 | 622706 | 0 | 30 |  | 7739 | 623040 |  | 22 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3569.73 | 637982 | 3483.87 | 632258 | 102 | 30 |  | 7864 | 635040 |  | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 | 30 |  | 7989 | 647040 |  | 4 | 0 | 3 (3) | 1014 |
| 50+60 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 | 0 | 1 (1) | 2 |
| (0,1) |  |  |  |  | & | Mid | 3519.99 | 634666 | 3459.33 | 630622 | 102 |  |  | 7846 | 633312 |  | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3715.02 | 647668 | 3509.64 | 633976 | 504 |  |  | 7982 | 646368 |  | 8 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3379.98 | 625332 | 3350.82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3574.95 | 638330 | 3509.07 | 633938 | 102 |  |  | 7881 | 636672 |  | 22 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 | 0 | 0 (0) | 1008 |
| 50+70 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3515.01 | 634334 | 3454.35 | 630290 | 102 | 30 |  | 7843 | 633024 |  | 22 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3705.12 | 647008 | 3499.74 | 633316 | 504 | 30 |  | 7975 | 645696 |  | 20 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=59.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3384.9 | 625660 | 3350.88 | 623392 | 0 | 30 |  | 7746 | 623712 |  | 8 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3574.89 | 638326 | 3504.15 | 633610 | 102 | 30 |  | 7878 | 636384 |  | 14 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3765 | 651000 | 3549.54 | 636636 | 504 | 30 |  | 8010 | 649056 |  | 12 | 0 | 3 (3) | 1014 |
| 50+80 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 | 0 | 1 (1) | 2 |
| (0,1) |  |  |  |  | & | Mid | 3510 | 634000 | 3449.34 | 629956 | 102 |  |  | 7840 | 632736 |  | 20 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3695.13 | 646342 | 3489.75 | 632650 | 504 |  |  | 7968 | 645024 |  | 14 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3389.88 | 625992 | 3350.82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3574.86 | 638324 | 3499.08 | 633272 | 102 |  |  | 7874 | 636000 |  | 16 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 | 0 | 3 (3) | 1014 |
| 50+90 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3504.99 | 633666 | 3444.33 | 629622 | 102 | 30 |  | 7836 | 632352 |  | 18 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3685.14 | 645676 | 3479.76 | 631984 | 504 | 30 |  | 7961 | 644352 |  | 8 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=69.84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3394.86 | 626324 | 3350.76 | 623384 | 0 | 30 |  | 7746 | 623712 |  | 16 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3574.83 | 638322 | 3494.01 | 632934 | 102 | 30 |  | 7871 | 635712 |  | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 | 30 |  | 7996 | 647712 |  | 8 | 0 | 3 (3) | 1014 |
| 50+100 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 | 0 | 1 (1) | 2 |
| (0,1) |  |  |  |  | & | Mid | 3500.01 | 633334 | 3439.35 | 629290 | 102 |  |  | 7833 | 632064 |  | 14 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3675.24 | 645016 | 3469.86 | 631324 | 504 |  |  | 7954 | 643680 |  | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3399.78 | 626652 | 3350.64 | 623376 | 0 | 30 |  | 7745 | 623616 |  | 0 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3574.77 | 638318 | 3488.91 | 632594 | 102 |  |  | 7867 | 635328 |  | 22 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 | 0 | 3 (3) | 1014 |
| 60+50 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3525 | 635000 | 3459.12 | 630608 | 102 |  |  | 7846 | 633312 |  | 16 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3720.03 | 648002 | 3509.43 | 633962 | 504 |  |  | 7982 | 646368 |  | 22 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3384.96 | 625664 | 3361.02 | 624068 | 0 | 30 |  | 7753 | 624384 |  | 4 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.96 | 638664 | 3519.3 | 634620 | 102 |  |  | 7888 | 637344 |  | 12 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 | 0 | 3 (3) | 1014 |
| 60+60 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3519.99 | 634666 | 3454.11 | 630274 | 102 |  |  | 7843 | 633024 |  | 14 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3709.98 | 647332 | 3499.38 | 633292 | 504 |  |  | 7975 | 645696 |  | 20 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=60 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3390 | 626000 | 3360.84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.99 | 638666 | 3514.11 | 634274 | 102 |  |  | 7885 | 637056 |  | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 | 0 | 0 (0) | 1008 |
| 60+70 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3515.01 | 634334 | 3449.13 | 629942 | 102 | 30 |  | 7839 | 632640 |  | 10 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3700.08 | 646672 | 3489.48 | 632632 | 504 | 30 |  | 7968 | 645024 |  | 8 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3394.92 | 626328 | 3360.9 | 624060 | 0 | 30 |  | 7753 | 624384 |  | 12 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.93 | 638662 | 3509.19 | 633946 | 102 | 30 |  | 7881 | 636672 |  | 14 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3765 | 651000 | 3549.54 | 636636 | 504 | 30 |  | 8010 | 649056 |  | 12 | 0 | 3 (3) | 1014 |
| 60+80 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3510 | 634000 | 3444.12 | 629608 | 102 |  |  | 7836 | 632352 |  | 8 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3690.09 | 646006 | 3479.49 | 631966 | 504 |  |  | 7961 | 644352 |  | 2 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3399.9 | 626660 | 3360.84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.9 | 638660 | 3504.12 | 633608 | 102 |  |  | 7878 | 636384 |  | 16 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 | 0 | 3 (3) | 1014 |
| 60+90 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3504.99 | 633666 | 3439.11 | 629274 | 102 | 30 |  | 7832 | 631968 |  | 6 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3680.1 | 645340 | 3469.5 | 631300 | 504 | 30 |  | 7954 | 643680 |  | 20 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3404.88 | 626992 | 3360.78 | 624052 | 0 | 30 |  | 7753 | 624384 |  | 20 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.87 | 638658 | 3499.05 | 633270 | 102 | 30 |  | 7874 | 636000 |  | 18 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 | 30 |  | 7996 | 647712 |  | 8 | 0 | 3 (3) | 1014 |
| 60+100 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3500.01 | 633334 | 3434.13 | 628942 | 102 |  |  | 7829 | 631680 |  | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3670.2 | 644680 | 3459.6 | 630640 | 504 |  |  | 7947 | 643008 |  | 8 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3409.8 | 627320 | 3360.66 | 624044 | 0 | 30 |  | 7752 | 624288 |  | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3579.81 | 638654 | 3493.95 | 632930 | 102 |  |  | 7871 | 635712 |  | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 | 0 | 3 (3) | 1014 |
| 70+60 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3519.99 | 634666 | 3449.25 | 629950 | 102 | 30 |  | 7839 | 632640 |  | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3705.06 | 647004 | 3489.6 | 632640 | 504 | 30 |  | 7968 | 645024 |  | 0 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3399.93 | 626662 | 3370.77 | 624718 | 0 | 30 |  | 7759 | 624960 |  | 2 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3584.91 | 638994 | 3519.03 | 634602 | 102 | 30 |  | 7888 | 637344 |  | 6 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 | 30 |  | 8016 | 649632 |  | 4 | 0 | 0 (0) | 1008 |
| 70+70 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3515.01 | 634334 | 3444.27 | 629618 | 102 | 30 |  | 7836 | 632352 |  | 22 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3695.04 | 646336 | 3479.58 | 631972 | 504 | 30 |  | 7961 | 644352 |  | 20 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=69.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3404.97 | 626998 | 3370.95 | 624730 | 0 | 30 |  | 7760 | 625056 |  | 14 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3584.97 | 638998 | 3514.23 | 634282 | 102 | 30 |  | 7885 | 637056 |  | 14 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3765 | 651000 | 3549.54 | 636636 | 504 | 30 |  | 8010 | 649056 |  | 12 | 0 | 3 (3) | 1014 |
| 70+80 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3510 | 634000 | 3439.26 | 629284 | 102 | 30 |  | 7833 | 632064 |  | 20 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3685.05 | 645670 | 3469.59 | 631306 | 504 | 30 |  | 7954 | 643680 |  | 14 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3409.95 | 627330 | 3370.89 | 624726 | 0 | 30 |  | 7760 | 625056 |  | 18 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3584.94 | 638996 | 3509.16 | 633944 | 102 | 30 |  | 7881 | 636672 |  | 16 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 | 30 |  | 8003 | 648384 |  | 10 | 0 | 3 (3) | 1014 |
| 70+90 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3504.99 | 633666 | 3434.25 | 628950 | 102 | 30 |  | 7829 | 631680 |  | 18 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3675.06 | 645004 | 3459.6 | 630640 | 504 | 30 |  | 7947 | 643008 |  | 8 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3414.93 | 627662 | 3370.83 | 624722 | 0 | 30 |  | 7760 | 625056 |  | 22 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3584.91 | 638994 | 3504.09 | 633606 | 102 | 30 |  | 7878 | 636384 |  | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 | 30 |  | 7996 | 647712 |  | 8 | 0 | 3 (3) | 1014 |
| 70+100 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3500.01 | 633334 | 3429.27 | 628618 | 102 | 30 |  | 7826 | 631392 |  | 14 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3665.16 | 644344 | 3449.7 | 629980 | 504 | 30 |  | 7940 | 642336 |  | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=84.84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3419.85 | 627990 | 3370.71 | 624714 | 0 | 30 |  | 7759 | 624960 |  | 6 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3584.85 | 638990 | 3498.99 | 633266 | 102 | 30 |  | 7874 | 636000 |  | 22 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 | 30 |  | 7989 | 647040 |  | 4 | 0 | 3 (3) | 1014 |
| 80+50 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3525 | 635000 | 3449.22 | 629948 | 102 |  |  | 7839 | 632640 |  | 4 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3710.13 | 647342 | 3489.63 | 632642 | 504 |  |  | 7968 | 645024 |  | 22 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3404.88 | 626992 | 3380.94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3589.86 | 639324 | 3529.2 | 635280 | 102 |  |  | 7895 | 638016 |  | 0 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 | 0 | 3 (3) | 1014 |
| 80+60 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3519.99 | 634666 | 3444.21 | 629614 | 102 |  |  | 7836 | 632352 |  | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3700.08 | 646672 | 3479.58 | 631972 | 504 |  |  | 7961 | 644352 |  | 20 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3409.92 | 627328 | 3380.76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3589.89 | 639326 | 3524.01 | 634934 | 102 |  |  | 7891 | 637632 |  | 10 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 | 0 | 0 (0) | 1008 |
| 80+70 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3515.01 | 634334 | 3439.23 | 629282 | 102 | 30 |  | 7833 | 632064 |  | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3690.06 | 646004 | 3469.56 | 631304 | 504 | 30 |  | 7954 | 643680 |  | 16 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3414.96 | 627664 | 3380.94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3589.95 | 639330 | 3519.21 | 634614 | 102 | 30 |  | 7888 | 637344 |  | 18 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3765 | 651000 | 3549.54 | 636636 | 504 | 30 |  | 8010 | 649056 |  | 12 | 0 | 3 (3) | 1014 |
| 80+80 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3510 | 634000 | 3434.22 | 628948 | 102 |  |  | 7829 | 631680 |  | 20 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3680.01 | 645334 | 3459.51 | 630634 | 504 |  |  | 7947 | 643008 |  | 14 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3420 | 628000 | 3380.94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3589.98 | 639332 | 3514.2 | 634280 | 102 |  |  | 7885 | 637056 |  | 16 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 | 0 | 3 (3) | 1014 |
| 80+90 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3504.99 | 633666 | 3429.21 | 628614 | 102 | 30 |  | 7826 | 631392 |  | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3670.02 | 644668 | 3449.52 | 629968 | 504 | 30 |  | 7940 | 642336 |  | 8 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=84.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3424.98 | 628332 | 3380.88 | 625392 | 0 | 30 |  | 7766 | 625632 |  | 0 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3589.95 | 639330 | 3509.13 | 633942 | 102 | 30 |  | 7881 | 636672 |  | 18 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 | 30 |  | 7996 | 647712 |  | 8 | 0 | 3 (3) | 1014 |
| 80+100 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3500.01 | 633334 | 3424.23 | 628282 | 102 |  |  | 7822 | 631008 |  | 14 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3660.12 | 644008 | 3439.62 | 629308 | 504 |  |  | 7933 | 641664 |  | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3429.9 | 628660 | 3380.76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3589.89 | 639326 | 3504.03 | 633602 | 102 |  |  | 7878 | 636384 |  | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 | 0 | 3 (3) | 1014 |
| 90+60 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300.9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3519.99 | 634666 | 3439.17 | 629278 | 102 | 30 |  | 7832 | 631968 |  | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3695.1 | 646340 | 3469.56 | 631304 | 504 | 30 |  | 7954 | 643680 |  | 16 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3419.88 | 627992 | 3390.72 | 626048 | 0 | 30 |  | 7773 | 626304 |  | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3594.87 | 639658 | 3528.99 | 635266 | 102 | 30 |  | 7895 | 638016 |  | 14 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 | 30 |  | 8016 | 649632 |  | 4 | 0 | 0 (0) | 1008 |
| 90+70 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300.9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3515.01 | 634334 | 3434.19 | 628946 | 102 | 30 |  | 7829 | 631680 |  | 22 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3685.08 | 645672 | 3459.54 | 630636 | 504 | 30 |  | 7947 | 643008 |  | 12 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3424.92 | 628328 | 3390.9 | 626060 | 0 | 30 |  | 7773 | 626304 |  | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3594.93 | 639662 | 3524.19 | 634946 | 102 | 30 |  | 7892 | 637728 |  | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3765 | 651000 | 3549.54 | 636636 | 504 | 30 |  | 8010 | 649056 |  | 12 | 0 | 3 (3) | 1014 |
| 90+80 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300.9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3510 | 634000 | 3429.18 | 628612 | 102 | 30 |  | 7826 | 631392 |  | 20 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3675.03 | 645002 | 3449.49 | 629966 | 504 | 30 |  | 7940 | 642336 |  | 10 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=84.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3429.96 | 628664 | 3390.9 | 626060 | 0 | 30 |  | 7773 | 626304 |  | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3594.96 | 639664 | 3519.18 | 634612 | 102 | 30 |  | 7888 | 637344 |  | 20 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 | 30 |  | 8003 | 648384 |  | 10 | 0 | 3 (3) | 1014 |
| 90+90 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300.9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3504.99 | 633666 | 3424.17 | 628278 | 102 | 30 |  | 7822 | 631008 |  | 18 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3664.98 | 644332 | 3439.44 | 629296 | 504 | 30 |  | 7933 | 641664 |  | 8 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=90.0 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3435 | 629000 | 3390.9 | 626060 | 0 | 30 |  | 7773 | 626304 |  | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3594.99 | 639666 | 3514.17 | 634278 | 102 | 30 |  | 7885 | 637056 |  | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 | 30 |  | 7996 | 647712 |  | 8 | 0 | 3 (3) | 1014 |
| 90+100 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300.9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3500.01 | 633334 | 3419.19 | 627946 | 102 | 30 |  | 7819 | 630720 |  | 14 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3655.08 | 643672 | 3429.54 | 628636 | 504 | 30 |  | 7926 | 640992 |  | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=94.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3439.92 | 629328 | 3390.78 | 626052 | 0 | 30 |  | 7773 | 626304 |  | 12 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3594.93 | 639662 | 3509.07 | 633938 | 102 | 30 |  | 7881 | 636672 |  | 22 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 | 30 |  | 7989 | 647040 |  | 4 | 0 | 3 (3) | 1014 |
| 100+50 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3525 | 635000 | 3439.14 | 629276 | 102 |  |  | 7832 | 631968 |  | 4 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3700.23 | 646682 | 3469.65 | 631310 | 504 |  |  | 7954 | 643680 |  | 10 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3424.77 | 628318 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.76 | 639984 | 3539.1 | 635940 | 102 |  |  | 7902 | 638688 |  | 12 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 | 0 | 3 (3) | 1014 |
| 100+60 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3519.99 | 634666 | 3434.13 | 628942 | 102 |  |  | 7829 | 631680 |  | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3690.18 | 646012 | 3459.6 | 630640 | 504 |  |  | 7947 | 643008 |  | 8 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3429.81 | 628654 | 3400.65 | 626710 | 0 | 30 |  | 7780 | 626976 |  | 2 | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3599.79 | 639986 | 3533.91 | 635594 | 102 |  |  | 7898 | 638304 |  | 22 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 | 0 | 0 (0) | 1008 |
| 100+70 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3515.01 | 634334 | 3429.15 | 628610 | 102 | 30 |  | 7826 | 631392 |  | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3680.16 | 645344 | 3449.58 | 629972 | 504 | 30 |  | 7940 | 642336 |  | 4 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=84.84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3434.85 | 628990 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.85 | 639990 | 3529.11 | 635274 | 102 | 30 |  | 7895 | 638016 |  | 6 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3765 | 651000 | 3549.54 | 636636 | 504 | 30 |  | 8010 | 649056 |  | 12 | 0 | 3 (3) | 1014 |
| 100+80 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3510 | 634000 | 3424.14 | 628276 | 102 |  |  | 7822 | 631008 |  | 20 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3670.11 | 644674 | 3439.53 | 629302 | 504 |  |  | 7933 | 641664 |  | 2 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3439.89 | 629326 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.88 | 639992 | 3524.1 | 634940 | 102 |  |  | 7891 | 637632 |  | 4 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 | 0 | 3 (3) | 1014 |
| 100+90 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3504.99 | 633666 | 3419.13 | 627942 | 102 | 30 |  | 7819 | 630720 |  | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3660.06 | 644004 | 3429.48 | 628632 | 504 | 30 |  | 7926 | 640992 |  | 0 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=94.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3444.93 | 629662 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.91 | 639994 | 3519.09 | 634606 | 102 | 30 |  | 7888 | 637344 |  | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 | 30 |  | 7996 | 647712 |  | 8 | 0 | 3 (3) | 1014 |
| 100 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 | 0 | 2 (2) | 4 |
| +100 |  |  |  |  | & | Mid | 3500.01 | 633334 | 3414.15 | 627610 | 102 |  |  | 7815 | 630336 |  | 14 | 0 | 1 (1) | 206 |
| (0,1) |  |  |  |  | Uplink | High | 3650.04 | 643336 | 3419.46 | 627964 | 504 |  |  | 7919 | 640320 |  | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3449.97 | 629998 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.97 | 639998 | 3514.11 | 634274 | 102 |  |  | 7885 | 637056 |  | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 | 0 | 3 (3) | 1014 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

==============End of change==============