3GPP TSG-RAN5 Meeting #109	R5-255594

# Dallas, Texas, USA, 17 – 21 Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 1088 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE co-existence spurious emission test point analysis for CA_n5A-n25A and CA_n5A-n77A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing UE co-existence spurious emission test point analysis for CA_n5A-n25A and CA_n5A-n77A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added UE co-existence spurious emission test point analysis for CA_n5A-n25A into Table 4.1.3.2-1. Corrected UE co-existence spurious emission test point analysis for CA_n5A-n77A in Table 4.1.3.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE co-existence spurious emission test point analysis for CA_n5A-n25A and CA_n5A-n77A will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 CR 3529 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Requirement CR in R5-255597 (CR 3529). Removed file: 38.521-1_TpAnalysisSpur(CA_n5A_n77A)_v3.zip Added file: 38.521-1_TpAnalysisSpur(CA_n5A_n77A)_v4.zip Added file: 38.521-1_TpAnalysisSpur(CA_n5A_n25A).zip |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

#### 4.1.3.2	Spurious emissions test cases for FR1 UL CA

In this case, it is sufficient to verify the minimum requirements in frequency ranges affected by 2nd and 3rd order intermodulation products. The frequency ranges and UL RB allocations used in the test are calculated here.

The analyses are performed per CA configuration and are stored as zip-files as defined in annex A.

NOTE:	Compared to the test points in test configuration Table 6.5A.3.2.1.4.1-1 in TS 38.521-1[2], additional test points in the test point analysis for Spurious emissions for UE co-existence in "6.5A.3.2.3 co-existence" tab of the zip-files shown in Table 4.1.3.2-1 introduced before RAN5#102 shall be ignored.

Table 4.1.3.2-1: Frequency range analysis availability per CA configuration

| CA config | Number of test point | Justification | Comments |
| --- | --- | --- | --- |
| CA_n1A-n3A | 0 | 38.521-1_TpAnalysisSpur(CA_n1A-n3A_v1).zip | RAN5#108 |
| CA_n1A-n5A | 0 | 38.521-1_TpAnalysisSpur(CA_n1A-n5A)_v1.zip | RAN5#108 |
| CA_n1A-n8A | 0 | 38.521-1_TpAnalysisSpur(CA_n1A-n8A)_v1.zip | RAN5#108 |
| CA_n1A-n28A | 1 | 38.521-1_TpAnalysisSpur(CA_n1A-n28A).zip | RAN5#102 |
| CA_n1A-n78A | 0 | 38.521-1_TpAnalysisSpur(CA_n1A-n78A)_v4.zip | RAN5#108 |
| CA_n1A-n79A | 0 | 38.521-1_TpAnalysisSpur(CA_n1A-n79A)_v1.zip | RAN5#108 |
| CA_n2A-n5A | 0 | 38.521-1_TpAnalysisSpur(CA_n2A-n5A)_v2.zip | RAN5#108 |
| CA_n2A-n14A | 2 | 38.521-1_TpAnalysisSpur(CA_n2A-n14A).zip | RAN5#101 |
| CA_n2A-n30A | 0 | 38.521-1_TpAnalysisSpur(CA_n2A-n30A)_v1.zip | RAN5#108 |
| CA_n2A-n48A | 0 | 38.521-1_TpAnalysisSpur(CA_n2A-n48A)_v2.zip | RAN5#108 |
| CA_n2A-n77A | 0 | 38.521-1_TpAnalysisSpur(CA_n2A-n77A)_v4.zip | RAN5#108 |
| CA_n3A-n5A | 0 | 38.521-1_TpAnalysisSpur(CA_n3A-n5A).zip | RAN5#107 |
| CA_n3A-n8A | 0 | 38.521-1_TpAnalysisSpur(CA_n3A-n8A)_v1.zip | RAN5#108 |
| CA_n3A-n28A | 0 | 38.521-1_TpAnalysisSpur(CA_n3A-n28A)_v1.zip | RAN5#108 |
| CA_n3A-n40A | 0 | 38.521-1_TpAnalysisSpur(CA_n3A-n40A).zip | RAN5#105 |
| CA_n3A-n41A | 0 | 38.521-1_TpAnalysisSpur(CA_n3A-n41A)_v1.zip | RAN5#108 |
| CA_n3A-n77A | 1 | 38.521-1_TpAnalysisSpur(CA_n3A-n77A).zip | RAN5#100 |
| CA_n3A-n78A | 4 | 38.521-1_TpAnalysisSpur(CA_n3A-n78A) v1.zip | RAN5#97 |
| CA_n5A-n25A | 0 | 38.521-1_TpAnalysisSpur(CA_n5A-n25A).zip | RAN5#109 |
| CA_n5A-n30A | 0 | 38.521-1_TpAnalysisSpur(CA_n5A-n30A)_v1.zip | RAN5#108 |
| CA_n5A-n48A | 7 | 38.521-1_TpAnalysisSpur(CA_n5A-n48A)_v1.zip | RAN5#100 |
| CA_n5A-n66A | 0 | 38.521-1_TpAnalysisSpur(CA_n5A-n66A)_v1.zip | RAN5#108 |
| CA_n5A-n77A | 0 | 38.521-1_TpAnalysisSpur(CA_n5A-n77A)_v4.zip | RAN5#109 |
| CA_n5A-n78A | 8 | 38.521-1_TpAnalysisSpur(CA_n5A-n78A).zip | RAN5#105 |
| CA_n8A-n77A | 1 | 38.521-1_TpAnalysisSpur(CA_n8A-n77A).zip | RAN5#108 |
| CA_n8A-n78A | 5 | 38.521-1_TpAnalysisSpur(CA_n8A-n78A) v1.zip | RAN5#97 |
| CA_n14A-n30A | 0 | 38.521-1_TpAnalysisSpur(CA_n14A-n30A)_v1.zip | RAN5#108 |
| CA_n14A-n66A | 0 | 38.521-1_TpAnalysisSpur (CA_n14A-n66A)_v1.zip | RAN5#108 |
| CA_n14A-n77A | 0 | 38.521-1_TpAnalysisSpur(CA_n14A-n77A)_v1.zip | RAN5#108 |
| CA_n20A-n78A | 0 | 38.521-1_TpAnalysisSpur(CA_n20A-n78A)_v1.zip | RAN5#108 |
| CA_n24A-n41A | 0 | 38.521-1_TpAnalysisSpur(CA_n24A-n41A)_v1.zip | RAN5#108 |
| CA_n24A-n48A | 0 | 38.521-1_TpAnalysisSpur(CA_n24A-n48A)_v1.zip | RAN5#108 |
| CA_n24A-n77A | 0 | 38.521-1_TpAnalysisSpur(CA_n24A-n77A)_v1.zip | RAN5#108 |
| CA_n25A-n66A | 0 | 38.521-1_TpAnalysisSpur(CA_n25A-n66A)_v1.zip | RAN5#108 |
| CA_n25A-n77A | 0 | 38.521-1_TpAnalysisSpur(CA_n25A-n77A)_v1.zip | RAN5#108 |
| CA_n25A-n78A | 0 | 38.521-1_TpAnalysisSpur(CA_n25A-n78A)_v1.zip | RAN5#108 |
| CA_n26A-n66A | 0 | 38.521-1_TpAnalysisSpur(CA_n26A-n66A)_v1.zip | RAN5#108 |
| CA_n26A-n70A | 0 | 38.521-1_TpAnalysisSpur(CA_n26A-n70A)_v1.zip | RAN5#108 |
| CA_n28A-n40A | 0 | 38.521-1_TpAnalysisSpur(CA_n28A-n40A).zip | RAN5#105 |
| CA_n28A-n41A | 4 | 38.521-1_TpAnalysisSpur(CA_n28A-n41A) v1.zip | RAN5#97 |
| CA_n28A-n77A | 1 | 38.521-1_TpAnalysisSpur(CA_n28A-n77A).zip | RAN5#100 |
| CA_n28A-n78A | 6 | 38.521-1_TpAnalysisSpur(CA_n28A-n78A).zip | RAN5#99 |
| CA_n28A-n79A | 0 | 38.521-1_TpAnalysisSpur(CA_n28A-n79A)_v1.zip | RAN5#108 |
| CA_n30A-n66A | 0 | 38.521-1_TpAnalysisSpur(CA_n30A-n66A)_v1.zip | RAN5#108 |
| CA_n30A-n77A | 0 | 38.521-1_TpAnalysisSpur(CA_n30A-n77A)_v1.zip | RAN5#108 |
| CA_n39A-n41A | 0 | 38.521-1_TpAnalysisSpur(CA_n39A-n41A)_v1.zip | RAN5#108 |
| CA_n40A-n41A | 1 | 38.521-1_TpAnalysisSpur(CA_n40A-n41A).zip | RAN5#108 |
| CA_n40A-n77A | 1 | 38.521-1_TpAnalysisSpur(CA_n40A-n77A).zip | RAN5#105 |
| CA_n41A-n77A | 1 | 38.521-1_TpAnalysisSpur(CA_n41A-n77A).zip | RAN5#102 |
| CA_n41A-n79A | 8 | 38.521-1_TpAnalysisSpur(CA_n41A-n79A) v1.zip | RAN5#97 |
| CA_n48A-n66A | 0 | 38.521-1_TpAnalysisSpur(CA_n48A-n66A)_v2.zip | RAN5#108 |
| CA_n48A-n70A | 0 | 38.521-1_TpAnalysisSpur(CA_n48A-n70A)_v1.zip | RAN5#108 |
| CA_n48A-n71A | 0 | 38.521-1_TpAnalysisSpur(CA_n48A-n71A)_v3.zip | RAN5#108 |
| CA_n66A-n71A | 0 | 38.521-1_TpAnalysisSpur(CA_n66A-n71A)_v2.zip | RAN5#108 |
| CA_n66A-n77A | 0 | 38.521-1_TpAnalysisSpur(CA_n66A-n77A)_v3.zip | RAN5#108 |
| CA_n66A-n78A | 0 | 38.521-1_TpAnalysisSpur(CA_n66A-n78A)_v1.zip | RAN5#108 |
| CA_n70A-n71A | 0 | 38.521-1_TpAnalysisSpur(CA_n70A-n71A)_v1.zip | RAN5#108 |
| CA_n71A-n77A | 0 | 38.521-1_TpAnalysisSpur(CA_n71A-n77A)_v2.zip | RAN5#108 |
| CA_n71A-n78A | 0 | 38.521-1_TpAnalysisSpur(CA_n71A-n78A)_v1.zip | RAN5#102 |

-------- End of changes --------

Remove attached .zip file to TR 38.905: “38.521-1_TpAnalysisSpur(CA_n5A-n77A)_v3.zip”

Add attached .zip file to TR 38.905: “38.521-1_TpAnalysisSpur(CA_n5A-n77A)_v4.zip”

Add attached .zip file to TR 38.905: “38.521-1_TpAnalysisSpur(CA_n5A-n25A).zip”