3GPP TSG-RAN WG5 Meeting #109	R5-256952

DALLAS, USA, 17th – 21st Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4147 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Sidelink relay test case 9.1.7.1 with TT analysis results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The test tolerance for TC 9.1.7.1.1 is not added in and analysis is missing. - RAN4 introduce U2U case in the same test case in Rel-18 NR sidelink relay enhancements WI. If the R17 U2N case uses the clause number. 9.1.7.1, it will be impossible to ensure the correspondence between R18 U2U cases and TS 38.133 when numbering them in the future. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated the TT analysis related parameters based on the analysis - Removed editor’s note - Updated the TC number fot Rel-17 U2N case from 9.1.7.1 to 9.1.7.1.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The sidelink relay test case 9.1.7.1.1 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 1085 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2520155 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

9.1.7	Selection / Reselection of relay UE

9.1.7.0	Minimum conformance requirements

9.1.7.0.1	Minimum conformance requirements for selection / reselection of relay UE

For a remote UE configured by upper layer for relay operation, the remote UE shall search for candidate relay UEs for selection and/or reselection every discovery period which is determined by resource reservation period or SPS transmission periodicity configured by network.

If the remote UE has a selected sidelink relay UE, then the remote UE shall measure the SD-RSRP or SL-RSRP of the selected relay once in every four discovery periods and evaluate if it meets the relay selection criterion as defined in TS 38.331[13] (clause 5.8.15.3).

The remote UE shall measure SD-RSRP or SL-RSRP of the candidate relay UEs every Tmeasure, SL_Relay_Intra for relay UEs that are detected and measured according to the measurement rules.

For intra-frequency relay UEs that are detected, but that has not been selected or reselected to, the remote UE shall be capable of evaluating that the intra-frequency relay UE has met selection or reselection criterion defined in TS 38.331[13] (clause 5.8.15.3) within Tevaluate, SL_Relay_Intra as specified in table 9.1.7.0.1-1.

The minimum requirements are required to meet when the selected and candidate relay UEs are transmitting relay discovery message every discovery period.

Table 9.1.7.0.1-1: Tmeasure, SL_Relay_Intra and Tevaluate, SL_Relay_Intra

| Discovery Period [s] | Tmeasure,SL_Relay_Intra [s] (number of discovery periods) | Tevaluate, SL_Relay_Intra [s] (number of discovery periods) |
| --- | --- | --- |
| 0.04≤Discovery period≤10.24 | Note 1 (4) | Note 1 (16) |
| Note 1: Time depends upon the discovery period which is resource reservation period (in mode 2) or SPS transmission periodicity (in mode 1). Note 2: SL-RSRP or SD-RSRP can be derived from PSCCH-DMRS and/or PSSCH-DMRS. |  |  |

The normative references for this requirement are TS 38.133 [6] clause 12.10.

9.1.7.1	NR SA FR1 Selection / Reselection of relay UE for U2N case

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test case is not complete for U2U

- Test applicability in 38.522 need to be updated with PICS for U2U/U2N

- Test case is incomplete for NR_FDD_FR1_N band”

9.1.7.1.1	Test purpose

The purpose of this test is to verify the requirements related to selection / reselection of relay UE defined in TS 38.133 [6] clauses 12.10. In the test the UE under test is configured with PCell and is configured with resource pools for NR sidelink discovery message as required for remote UE operation.

9.1.7.1.2	Test applicability

This test applies to all types of NR UEs from release 17 onwards and supporting NR sidelink communication and further support the optional feature of sidelink remote UE operation.

9.1.7.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 9.1.7.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.9.1.7.

9.1.7.1.4	Test description

9.1.7.1.4.1	Initial conditions

1.	The parameter settings for the NR sidelink communication are pre-configured according to TS 38.508-1 [14] clause 4.10.1.

2.	Message content exceptions are defined in clause 9.1.7.1.4.3

3.	There is one NR Uu carrier and one NR Cell (Cell 1) specified in this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

4.	Test parameters for Cell 1 are given in Table 9.1.7.1.4.1-0 and Table 9.1.7.1.4.1-1. And NR cell 1 broadcasts SIB12.

5.	There are one remote UE and two active Sidelink relay UEs (Sidelink Relay UE 1, Sidelink Relay UE 2) in this test. The relay UEs are configured in mode 2 to be transmitting relay discovery messages every discovery period, which is determined by resource reservation period indicated by sl-ResourceReservePeriodList. Test parameters for active sidelink UEs are given in Table 9.1.7.1.4.1-2 and Table 9.1.7.1.4.1-3.

Table 9.1.7.1.4.1-0: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table 9.1.7.1.4.1-1: Test parameters for selection / reselection of relay UE test for NR FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS | kHz | 30 |  |
| Active cell |  | Cell 1 | Serving NR cell will broadcasting SIB12 |
| Active Relay UEs |  | Relay UE 1, Relay UE 2 | Transmitting relay discovery message |
| CP length of Cell 1 |  | Normal |  |
| T1 | ms | 100 |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to 1 sec if UE does not transmit SidelinkUEInformationNR during this period. |  |
| T3 | s | 1 |  |
| T4 | s | 2 |  |
| T5 | s | 1 |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |

Table 9.1.7.1.4.1-2: Sidelink discovery configuration for selection / reselection of relay UE test for NR FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| Resource pool configuration for sidelink discovery |  | As specified in TS38.331[13] Table A.3.21.2-2 Note 2 | IE values unless specified otherwise in this test. |
| sl-TimeResource included in sl-DiscTxPoolSelected-r17 and sl-DiscRxPool-r17 |  | 0000111100 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[8]) |
| sl-NumSubchannel included in sl-DiscTxPoolSelected-r17 and sl-DiscRxPool-r17 |  | 2 | Indicates the number of sub-channels for TX resource pool |
| sl-SubchannelSize included in sl-DiscTxPoolSelected-r17 and sl-DiscRxPool-r17 |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
| sl-ResourceReservePeriodList | s | 0.04 | Indicates the resource reservation period for discovery period |
| threshHighRemote within sl-remoteUE-Config |  | -91dBm |  |
| sl-RSRP-Thresh within sl-remoteUE-Config |  | -91dBm |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: This test is according to the principle defined in TS 38.331 [13] section A.3.21. |  |  |  |

Table 9.1.7.1.4.1-3: Sidelink Communication configuration for selection / reselection of relay UE test

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | - | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| Sidelink Communication configuration |  | - | As specified in section TS 38.331 [13] A.3.21.2 | IE values unless specified otherwise in this test. |
| sl-TimeResource |  | - | 0000000011 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[8]) |
| Number of Active Sidelink Relay UEs |  | - | 2 | Active Sidelink UE i = 0, 1 |
| Active Sidelink Relay UEs (UE i = 0, 1) | PSCCH Reference Measurement Channel | - | CC.1A HD | As specified in TS 38.331 [13] Table A.3.21.3-1 |
|  | PSSCH Reference Measurement Channel | - | CD.1A HD | As specified in Table TS 38.331 [13] A.3.21.3-2 |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool | - | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-SubchannelSize-r16 included in SL-ResourcePool |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
|  | sl-StartRB-Subchannel-r16 included in SL-ResourcePool | - | floor(i)x10 | Indicates the lowest RB index of the subchannel with the lowest index for Sidelink Relay UE i = 0 starts RB=0; Sidelink Relay UE i = 1 starts RB=10. |
|  | PSBCH-RSRP | dBm/30kHz | -95 |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

9.1.7.1.4.2	Test procedure

In this test there are one remote UE and two active Sidelink relay UEs (Sidelink Relay UE 1, Sidelink Relay UE 2) in this test. The relay UEs are configured in mode 2 to be transmitting relay discovery messages every discovery period, which is determined by resource reservation period indicated by sl-ResourceReservePeriodList.

The tests consist of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively.

1.	Ensure the UE is in state 3N-A as defined in TS 38.508-1 [14], subclause 4.4A on NR Cell 1 and has transmitted SidelinkUEInformationNR message.

2.	Set the parameters for T1 according to Table 9.1.7.1.5-1 and Table 9.1.7.1.5-2. T1 starts.

3.	When T1 expires, the SS shall switch the power setting from T1 to T2 in Table 9.1.7.1.5-1 and Table 9.1.7.1.5-2. T2 starts.

4.	If the UE sends a SidelinkUEInformationNR message within 1 second from the beginning of T2, continues with step 5, otherwise skips to step 7.

5.	The SS sends an RRCReconfiguration message with SL-ConfigDedicatedNR to provide NR sidelink discovery configuration to the UE.

6.	The UE sends an RRCReconfigurationComplete message.

7.	T2 expires. The SS shall switch the power setting from T2 to T3 in Table 9.1.7.1.5-1 and Table 9.1.7.1.5-2. T3 starts.

8.	If the UE sends a PROSE DIRECT LINK ESTABLISHMENT REQUEST message to Sidelink Relay UE 1 within 680 ms from the beginning of time period T3, then count a success for the event “Select Sidelink Relay UE 1”. Otherwise count a fail for the event “Select Sidelink Relay UE 1”.

9.  If the UE has sent a PROSE DIRECT LINK ESTABLISHMENT REQUEST message within T1, continue with step 10 when T3 expires. If T3 expires and the UE has not yet sent a PROSE DIRECT LINK ESTABLISHMENT REQUEST message, skip to step 17.

10.	The SS shall switch the power setting from T3 to T4 in Table 9.1.7.1.5-1 and Table 9.1.7.1.5-2. T4 starts.

11.	Sidelink Relay UE 1 sends a PROSE DIRECT LINK SECURITY MODE COMMAND message.

12.	The UE sends a PROSE DIRECT LINK SECURITY MODE COMPLETE message.

13.	Sidelink Relay UE 1 sends a PROSE DIRECT LINK ESTABLISHMENT ACCEPT message.T4 ends.

14. The SS shall switch the power setting from T4 to T5 in Table 9.1.7.1.5-1 and Table 9.1.7.1.5-2. T5 starts.

15.	If the UE sends a PROSE DIRECT LINK ESTABLISHMENT REQUEST message Sidelink Relay UE 2 within 840 ms from the beginning of time period T5, then count a success for the event “Re-select Sidelink Relay UE 2”. Otherwise count a fail for the event “Re-select Sidelink Relay UE 2”.

16. If the UE has re-selected Sidelink Relay UE 2 within T5, after the re-selection or when T5 expires and If T5 expires and the UE has not yet re-selected Sidelink Relay UE 2, continue with step 17.

17.	Sidelink Relay UE 2 sends a PROSE DIRECT LINK SECURITY MODE COMMAND message.

18.	The UE sends a PROSE DIRECT LINK SECURITY MODE COMPLETE message.

19.	Sidelink Relay UE 2 sends a PROSE DIRECT LINK ESTABLISHMENT ACCEPT message.T5 ends.

20. The SS shall send an RRCRelease message to UE.

21. The SS shall switch the power setting to T1 in Table 9.1.7.1.5-1 and Table 9.1.7.1.5-2 for next iteration of the test procedure loop.

22. The TE shall switch off and on the UE to ensure the UE is in state 3N-A as defined in TS 38.508-1 [14], subclause 4.4A on NR Cell 1 and has transmitted SidelinkUEInformationNR message.

23. Repeat step 2-22 until a test verdict has been achieved.

Each of the events “Select Sidelink Relay UE 1” and “Re-select Sidelink Relay UE 2” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.

If both events pass, the test passes. If one event fails, the test fails.

9.1.7.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 9.1.7.1.4.3-1: SIB12-IEs-r16 (Preamble and test procedure, Cell 1)

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |
| sl-DiscConfigCommon-r17 SEQUENCE { |  |  |  |
| sl-RelayUE-ConfigCommon-r17 | Not present |  |  |
| sl-RemoteUE-ConfigCommon-r17 | SL-RemoteUE-Config | Table 9.1.7.1.4.3-2 |  |
| } |  |  |  |

Table 9.1.7.1.4.3-2: SL-RemoteUE-Config(Table 9.1.7.1.4.3-1 and Table 9.1.7.1.4.3-8)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-23E |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-RemoteUE-Config-r17 ::= SEQUENCE { |  |  |  |  |
| threshHighRemote-r17 |  | 65 | -91 dBm |  |
| hystMaxRemote-r17 |  | 4 |  |  |
| sl-ReselectionConfig-r17 SEQUENCE { |  |  |  |  |
| sl-RSRP-Thresh-r17 |  | 6 | -91 dBm |  |
| sl-FilterCoefficientRSRP-r17 |  | fc4 |  |  |
| sl-HystMin-r17 |  | 4 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.1.7.1.4.3-3: SL-ResourcePool (UE under test)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-TimeResource-r16 | 0000000011 |  |  |
| } |  |  |  |

Table 9.1.7.1.4.3-4: SL-ResourcePool (active sidelink UE 0,1)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-NumSubchannel-r16 | 1 |  |  |
| sl-StartRB-Subchannel-r16 | floor(i)x10 | i is the index of active sidelink relay UE, i = 0, 1 |  |
| sl-TimeResource-r16 | 0000000011 |  |  |
| } |  |  |  |

Table 9.1.7.1.4.3-5: SL-BWP-ConfigCommon

| Derivation Path: TS 38.508-1 [14], Table 7.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-ConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-BWP-PoolConfigCommonPS-r17 |  | Not present |  |  |
| sl-BWP-DiscPoolConfigCommon-r17 |  | SL-BWP-DiscPoolConfigCommon |  | Table 9.1.7.1.4.3-6 |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.1.7.1.4.3-6: SL-BWP-DiscPoolConfigCommon(Table 9.1.7.1.4.3-5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.6-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-DiscPoolConfigCommon-r17 ::= SEQUENCE { |  |  |  |  |
| sl-DiscRxPool-r17 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16 { |  | 1 entry |  |  |
| SL-ResourcePool-r16[1] |  | SL-ResourcePool-Disc | entry 1 | Table 9.1.7.1.4.3-7 |
| } |  |  |  |  |
| sl-DiscTxPoolSelected-r17 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { |  | 1 entry |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-ResourcePoolID-r16 |  | 1 | Index of the resource pool for normal case |  |
| sl-ResourcePool-r16 |  | SL-ResourcePool-Disc |  | Table 9.1.7.1.4.3-7 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.1.7.1.4.3-7: SL-ResourcePool-Disc(Table 9.1.7.1.4.3-6)

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |
| sl-ResourceReservePeriodList-r16 SEQUENCE (SIZE (1..16)) OF SL-ResourceReservePeriod-r16{ |  |  |  |
| sl-ResourceReservePeriod2-r16 | 40 | 0.04s |  |
| } |  |  |  |
| } |  |  |  |
| sl-NumSubchannel-r16 | 2 |  |  |
| sl-TimeResource-r16 | 0000111100 |  |  |
| } |  |  |  |

Table 9.1.7.1.4.3-8: SL-ConfigDedicatedNR

| Derivation Path: TS 38.508-1 [14] Table 7.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |  |
| sl-DiscConfig-r17 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| sl-RelayUE-Config-r17 |  | Not present |  |  |
| sl-RemoteUE-Config-r17 CHOICE { |  |  |  |  |
| setup |  | SL-RemoteUE-Config |  | Table 9.1.7.1.4.3-2 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

9.1.7.1.5	Test requirement

Table 9.1.7.1.5-1 and Table 9.1.7.1.5-2 define the primary level settings does not including test tolerances for the test.

Table 9.1.7.1.5-1: Sidelink Relay UE specific test parameters for selection / reselection of relay UE test for NR FR1

| Parameter | Unit | Relay UE 1 |  |  |  |  | Relay UE 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  | 1 (HD carrier in Band n47 or n38) |  |  |  |  |  |  |  |  |  |
| BWchannel Note 4 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |  |  |  |  |  |  |  |  |
| SCS | kHz | 30 |  |  |  |  |  |  |  |  |  |
| Sidelink Discovery resource pool configuration |  | As specified in TS 38.331 [13] Table A.9.1.7.1-2 |  |  |  |  |  |  |  |  |  |
| Transmission frequency |  | Every discovery period (40ms) |  |  |  |  |  |  |  |  |  |
| Resource allocation |  | Non-overlapping PRBs |  |  |  |  |  |  |  |  |  |
| Note1 | dBm/30 kHz | -97 |  |  |  |  |  |  |  |  |  |
|  | dB | -inf | -inf | 14.2 | 14.2 | -1.5 | -inf | -inf | -inf | -inf | 14.2 |
| SD-RSRP / SL-RSRP Note2, Note 3 | dBm/30 kHz | -inf | -inf | -82.8 | -82.8 | -98.5 | -inf | -inf | -inf | -inf | -82.8 |
| Antenna Configuration | - | 1x2 |  |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SD-RSRP / SL-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: PSCCH-DMRS Es/Iot is set the same as PSSCH-DMRS Es/Iot. Note 4: This test is according to the principle defined in TS 38.331 [13] section A.3.21.2. |  |  |  |  |  |  |  |  |  |  |  |

Table 9.1.7.1.5-2: Cell specific test parameters for selection / reselection of relay UE test for NR FR1

| Parameter |  | Unit | Cell1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  | 2 |  |  |  |  |
| Serving/Non-serving |  |  | Serving |  |  |  |  |
| Duplex Mode | Config 1 |  | FDD |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| Channel Bandwidth (BWchannel) | Config 1,2 |  | 10: NRB,c = 52 |  |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |
| DRX Cycle |  |  | N/A |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| SMTC Configuration |  |  | SMTC.2 |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |  |  |
| drx-Configuration |  |  | N/A |  |  |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  |  | R.11 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  |  | OP.16 FDD |  |  |  |  |
| Note2 | Config 1,2 | dBm/15 kHz | -97 |  |  |  |  |
|  | Config 3 |  | -97.54 |  |  |  |  |
| Note2 | Config 1,2, | dBm/SCS | -97 |  |  |  |  |
|  | Config 3 |  | -94.54 |  |  |  |  |
|  | Config 1,2, | dB | 14.2 | 1.5 | 1.5 | 1.5 | 1.5 |
|  | Config 3 |  | 11.69 | 1.5 | 1.5 | 1.5 | 1.5 |
| SS-RSRP Note3 | Config 1,2, | dBm/SCS | -82.8 | -95.5 | -95.5 | -95.5 | -95.5 |
|  | Config 3 |  | -83.5 | -92.5 | -92.5 | -92.5 | -92.5 |
| Propagation condition |  |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

The relay UEs are configured in mode 2 to be transmitting relay discovery messages every discovery period, which is determined by resource reservation period indicated by sl-ResourceReservePeriodList. And in test case the discovery period is configured as 40ms.

The Tevaluate, SL_Relay_Intra and Tmeasure, SL_Relay_Intra shall fulfil the requirements specified in clause 9.1.7.0.1.

Tevaluate, SL_Relay_Intra = 16 discovery periods, Tmeasure, SL_Relay_Intra=4 discovery periods.

The SL relay UE selection delay = (Tevaluate, SL_Relay_Intra + Every discovery period) = 680ms.

The SL relay UE reselection delay = (Tmeasure, SL_Relay_Intra + Tevaluate, SL_Relay_Intra + Every discovery period) = 840ms.