3GPP TSG WG RAN5 Meeting #109	R5-256279

# Dallas, USA, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-1 | CR | 0025 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to functional alias IE for MCVideo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, FirstNet, Element, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the generic test procedure 5.3.36, the procedure use a request-type element set to a value of “functional-alias-status-determination”, however the request-type is an enumration that has not defined “functional-alias-status-determination”. The issue was resolved in a CR to the core specification TS 24.281 and the test case should be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 5.3.36.4-3A the IE request-type was replaced by the new IE req-type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure 5.3.36 can not be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.36 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR is dependant on the approval of a CR to TS 24.281. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.36	UE initiated MCX functional alias status determination and subscription

5.3.36.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3.36.2	Void

5.3.36.3	Procedure

Table 5.3.36.3-1: MCX functional alias status determination and subscription

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCX client) request to determine the current status of a functional alias and later notification of status changes of a functional alias. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 2a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 3 | Check: Does the UE (MCX client) send a SIP SUBSCRIBE requesting the status of any existing functional aliases? | --> | SIP SUBSCRIBE | - | P |
| 4 | The SS (MCX server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | The SS (MCX server) sends a SIP NOTIFY with functional alias information | <-- | SIP NOTIFY | - | - |
| 6 | Check: Does the UE (MCX client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 7 | The procedure 'MCX communication release' as described in clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI |  |  |  |  |  |

5.3.36.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3.36.4-1: SIP SUBSCRIBE (step 3, Table 5.3.36.3-1)

| Derivation Path: Table 5.5.2.14-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| value | "4294967295" | to receive the current status and later notification | TS 24.379 [9] clause 9A.2.1.3 TS 24.282 [87] clause 22.2.1.3 |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  | MCPTT |
| MIME-part-body | MCPTT-Info as described in Table 5.3.36.4-2 |  | TS 24.379 [9] clause 9A.2.1.3 |  |
| MIME body part |  | MCData Info |  | MCDATA |
| MIME-part-body | MCData-Info as described in Table 5.3.36.4-3 |  | TS 24.282 [87] clause 22.2.1.3 |  |
| MIME body part |  | MCVideo Info |  | MCVIDEO |
| MIME-part-body | MCVideo-Info as described in Table 5.3.36.4-3A |  | TS 24.281 [86] clause 20.2.1.3 |  |

Table 5.3.36.4-2: MCPTT-Info in SIP SUBSCRIBE (Table 5.3.36.4-1)

| Derivation Path: Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.2.1.3 |  |
| anyExt |  |  |  |  |
| request-type | "functional-alias-status-determination" |  | TS 24.379 [9] clause 9A.2.1.3 |  |

Table 5.3.36.4-3: MCData-Info in SIP SUBSCRIBE (Table 5.3.36.4-1)

| Derivation Path: Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "functional-alias-status-determination" |  | TS 24.282 [87] clause 22.2.1.3 |  |
| mcdata-request-uri | px_MCData_ID_User_A |  | TS 24.282 [87] clause 22.2.1.3 |  |

Table 5.3.36.4-3A: MCVideo-Info in SIP SUBSCRIBE (Table 5.3.36.4-1)

| Derivation Path: Table 5.5.3.2.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-request-uri | px_MCVideo_ID_User_A |  | TS 24.281 [86] clause 20.2.1.3 |  |
| anyExt |  |  |  |  |
| req-type | "functional-alias-status-determination" |  | TS 24.281 [86] clause 20.2.1.3 |  |

Table 5.3.36.4-4: SIP 200 (OK) (step 4, Table 5.3.36.3-1)

Derivation Path: Table 5.5.2.17.1.2-1, condition SUBSCRIBE-RSP

Table 5.3.36.4-5: SIP NOTIFY (step 5, Table 5.3.36.3-1)

| Derivation Path: Table 5.5.2.8-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF for MCPTT as described in Table 5.5.3.5.2-1 (NOTE 1) |  | TS 24.379 [9] clause 9A.2.2.2.5 | MCPTT |
| MIME-part-body | PIDF for MCData as described in Table 5.5.3.5.2-3 (NOTE 1) |  | TS 24.282 [87] clause 22.2.2.2.5 | MCDATA |
| MIME-part-body | PIDF for MCVideo as described in Table 5.5.3.5.2-2 (NOTE 1) |  | TS 24.281 [86] clause 20.2.2.2.5 | MCVIDEO |
| NOTE 1: PIDF document contains tuple with empty <status> element (i.e. there are no <functionalAlias> entries at all) and not containing a <p-id-fa> element |  |  |  |  |