3GPP TSG WG RAN5 Meeting #109	R5-255757

# Dallas, Texas, USA 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-7 | CR | 0010 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.4.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet, Ericsson, Element, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case, 6.4.4 On-network / Emergency Alert / Emergency Alert Area included in the Work Plan is not implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case, 6.4.4 On-network / Emergency Alert / Emergency Alert Area |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Work Plan will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.4 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications. | TS 37.579-1 CR 0021 TS 37.579-4 CR 0007 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table 6.4.2.3.3-5: MCData-Info in SIP MESSAGE (Table 6.4.2.3.3-4)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-calling-group-id> with mcdataURI set to px_MCData_Group_A_ID | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-3 |  |  |
| alert-ind | Encrypted <alert-ind> with mcdataBoolean set to false | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-3 |  |  |

### 6.4.4	On-network / Emergency Alert / Emergency alert area

6.4.4.1	Test Purpose (TP)

(1)

with { UE (MCData Client) registered and authorized for MCData Service }

ensure that {
  when { the MCData Server notifies the UE (MCData Client) of entry into a emergency alert area by sending the UE (MCData Client) a SIP MESSAGE }

then { UE (MCData Client) acknowledges the entry into a emergency alert area notification by sending a SIP 200 (OK) response and notifies the user of the entry into the emergency alert area }

}

(2)

with { UE (MCData Client) having been previously notified of entry into an emergency alert area }

ensure that {
  when { the MCData User requests to send an emergency alert with the location of emergency }

then { UE (MCData Client) sends a SIP MESSAGE initiating an emergency alert and reporting location information }

}

(3)

with { UE (MCData Client) in the “MDEA 3: emergency-alert-initiated” state}

ensure that {
  when { the MCData Server notifies the UE (MCData Client) of exit from a emergency alert area by sending the UE (MCData Client) a SIP MESSAGE }

then { UE (MCData Client) acknowledges the exit from emergency alert area notification by sending a SIP 200 (OK) response and notifies the user of the exit from the emergency alert area }

}

6.6.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 16.2.1.1 and 16.2.1.5. Unless otherwise stated, these are Rel-17 requirements.

[TS 24.282, clause 16.2.1.1]

Upon receiving a request from the MCData user to send an MCData emergency alert, the MCData client shall determine whether or not it is authorised to originate an emergency alert, by following the procedures in clause 6.2.8.1.6.

If the MCData emergency alert origination request is considered an unauthorised request for an MCData emergency alert, the MCData client shall indicate to the MCData user that an MCData emergency alert is not allowed on this group and shall terminate this procedure.

If the request was authorised, but the MCData user has not indicated the identity of the MCData group to receive the emergency alert, the MCData client shall use, in descending order of preference, one of the following: the value of the <uri-entry> element of the <entry> element of the <GroupEmergencyAlert> element of the <Common> element in the MCData user profile, if present; if not, the identity of the MCData group to which the most recent communication or affiliation request was made by the MCData client since last acquiring the MCData service. If an MCData group identity cannot be determined, the MCData client shall indicate the fact to the MCData user and shall terminate this procedure.

The MCData client shall generate a SIP MESSAGE as an out-of-dialog request, in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6], and:

1)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP MESSAGE request;

2)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [5];

4)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element (see clause D.1) with:

a)	the <mcdata-request-uri> element set to the MCData group identity;

b)	the <alert-ind> element set to a value of "true";

c)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client; and

d)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> element set to the URI of the used functional alias; and

e)	if the MCData user has requested an application priority, the <anyExt> element with the <user-requested-priority> element set to the user provided value;

5)	shall include an application/vnd.3gpp.mcdata-location-info+xml MIME body with a <Report> element included in the <location-info> root element (see clause D.4);

6)	shall include in the <Report> element the specific location information configured for the MCData emergency alert location trigger;

7)	shall set the MCData emergency state if not already set;

8)	shall set the MCData emergency alert state to "MDEA 2: emergency-alert-confirm-pending";

9)	shall set the Request-URI to the public service identity identifying the participating MCData function serving the group identity; and

10)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP 24.229 [5];

On receiving a SIP 2xx response to the SIP MESSAGE request, the MCData client shall set the MCData emergency alert state to "MDEA 3: emergency-alert-initiated" and shall give the MCData user an indication of success.

On receiving a SIP 4xx response a SIP 5xx response or a SIP 6xx response to the SIP MESSAGE request, the MCData client shall set the MCData emergency alert state to "MDEA 1: no-alert" and shall indicate the failure to the MCData user.

NOTE:	If no response is received after an implementation dependent amount of time or if there is an indication of communication failure, the MCData client can inform the user, and can clear the MCData emergency alert state or can retry sending the emergency alert to the MCData participating server. The MCData emergency state is left unchanged, as the MCData user presumably is in the best position to determine whether or not there still is an emergency situation and can use manual clearing, as necessary.

[TS 24.282, clause 16.2.1.5]

Upon receipt of a "SIP MESSAGE request for notification of entry into or exit from an emergency alert area", the MCData client:

1)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcdata-info+xml MIME body with the <emergency-alert-area-ind> element of the value:

a)	set to "true":

i)	may display to the MCData user an indication that MCData client has entered a pre-defined emergency alert area; and

ii)	if the MCData user is not in emergency state, shall initiate the emergency alert origination procedure as specified in clause 12.1.1.1; or

b)	set to "false":

i)	may display to the MCData user an indication that MCData client has exited a pre-defined emergency alert area.

NOTE:	In this case, the MCData emergency state remains set, as the MCData user is in the best position to determine whether or not they are in a life-threatening condition. The MCData user can clear the MCData emergency state manually, if needed.

2)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [5]; and

3)	shall send the SIP 200 (OK) response towards the MCData server according to rules and procedures of 3GPP TS 24.229 [5].

6.4.4.3	Test description

6.4.4.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.4.4.3.2	Test procedure sequence

Table 6.4.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to receive notification of entry into an emergency alert area? | - | - | 1 | P |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication that an emergency alert area has been entered. | - | - | - | - |
| 2a1 | IF pc_MCX_DisplayInfoEmergencyAlertArea THEN Check: Does the UE (MCData client) notify the user that an emergency alert area has been entered? (NOTE 1) | - | - | 1 | P |
| 3 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE Request - Accept CO' as described in TS 37.579-1 [2] Table 5.3.30.3-1 to request an emergency alert providing location information? |  |  | 2 | P |
| 4 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to receive notification of exit from an emergency alert area? | - | - | 3 | P |
| - | EXCEPTION: Ste5a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication that an emergency alert area has been exited. |  |  |  |  |
| 5a | IF pc_MCX_DisplayInfoEmergencyAlertArea THEN Check: Does the UE (MCData client) notify the user that a group geographic area has been exited? (NOTE 1) |  |  | 3 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.4.4.3.3	Specific message contents

Table 6.4.4.3.3-1: SIP MESSAGE from the SS (step 1, Table 6.4.4.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-INFO, ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.4.4.3.3-2 |  |  |  |

Table 6.4.4.3.3-2: MCData-Info in SIP MESSAGE (Table 6.4.4.3.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| associated-group-id | px_MCData_Group_A_ID |  |  |  |
| anyExt |  |  |  |  |
| emergency-alert-area-ind | true |  |  |  |

Table 6.4.4.3.3-3: SIP MESSAGE from the UE(step 3, Table 6.4.4.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.1-1, condition LOCATION-INFO |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.4.4.3.3-4 |  |  |  |
| MIME body part |  | Location-info |  |  |
| MIME-part-body | Location-info as described in Table 6.4.4.3.3-5 |  |  |  |

Table 6.4.4.3.3-4: MCData-Info in SIP MESSAGE (Table 6.4.4.3.3-3)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-request-uri | Encrypted <mcdata-request-uri> with mcdataURI set to px_MCData_Group_A_ID | Encryption according to NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-3 |  |  |
| alert-ind | Encrypted <alert-ind> with mcdataBoolean set to true | Encryption according to NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-3 |  |  |
| mcdata-client-id | encrypted <mcdata-client-id> with mcdataString set to the UE’s UUID URN | Encryption according to NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-3 |  |  |

Table 6.4.4.3.3-5: Location-Info in SIP MESSAGE (Table 6.4.4.3.3-3)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.4.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| CurrentLocation |  |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted <longitude> with any content | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.4.1-3 |  |  |
| latitude | Encrypted <latitude> with any content | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.4.1-3 |  |  |

Table 6.4.4.3.3-6: SIP MESSAGE from the SS (step 3, Table 6.4.4.3.2-1;
step 4, TS 37.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.4.4.3.3-7 |  |  |  |

Table 6.4.4.3.3-7: MCData-Info in SIP MESSAGE (Table 6.4.4.3.3-6)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |
| alert-ind | Encrypted <alert-ind> with mcdataBoolean set to true | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-3 |  |  |
| mcdata-client-id | Encrypted <mcdata-client-id> with mcdataString set to the client-id as received from the UE | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-3 |  |  |
| alert-ind-rcvd | true |  |  |  |

Table 6.4.4.3.3-8: SIP MESSAGE from the SS (step 4, Table 6.4.4.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-INFO, ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.4.4.3.3-9 |  |  |  |

Table 6.4.4.3.3-9: MCData-Info in SIP MESSAGE (Table 6.4.4.3.3-8)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| associated-group-id | px_MCData_Group_A_ID |  |  |  |
| anyExt |  |  |  |  |
| emergency-alert-area-ind | false |  |  |  |