3GPP TSG-RAN5 Meeting #109	R5-255852

# Dallas, Texas, USA, 17th - 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-7 | CR | 0016 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MCData test case 6.2.17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test case 6.2.17 needs specific user profile which shall be mentioned in the Pre-test conditions |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarification added to Pre-test conditions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.17	On-network / File Distribution (FD) / FD Using Media Plane / Group Standalone FD / Preconfigured Group Use Only

6.2.17.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a Group Standalone FD message using the media plane to a group configured for preconfigured group use only }

then { UE (MCDATA Client) indicate to the MCDATA user that group standalone FD is not allowed on the indicated group and does not send a SIP INVITE request }

}

6.2.17.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clause 10.2.5.2.3.  Unless otherwise stated, these are Rel-17 requirements.

[TS 24.282, clause 10.2.5.2.3]

The MCData client shall generate a SIP INVITE request in accordance with 3GPP TS 24.229 [5] with the clarifications given below.

The MCData client:

1)	shall include the g.3gpp.mcdata.fd media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcdata.fd media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP INVITE request;

5)	should include the "timer" option tag in the Supported header field;

6)	should include the Session-Expires header field according to IETF RFC 4028 [38]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

7)	shall generate and contain an application/vnd.3gpp.mcdata-signalling MIME body with the FD SIGNALLING PAYLOAD as described in clause 6.2.2.3;

8)	if a one-to-one file distribution is requested:

a0)	if the MCData user has requested the origination of an MCData emergency one-to-one communication or is originating an MCData one-to-one communication and the MCData emergency state is already set, then:

i)	if this is an authorised request for an MCData emergency one-to-one communication as determined by the procedures of clause 6.2.8.3.1.1, shall comply with the procedures in clause 6.2.8.3.2; or

ii)	if this is an unauthorised request for an MCData emergency one-to-one communication as determined in step i) above, should indicate to the MCData user that initiation of an MCData emergency one-to-one communication is not authorized and shall release the generated SIP INVITE request and end the procedure;

a)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user or the functional alias to be called, according to rules and procedures of IETF RFC 5366 [18];

NOTE 0:	The MCData client indicates whether an MCData ID or a functional alias is to be called as specified in step 8) b) below.

b)	shall contain an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "one-to-one-fd";

ii)	with the <call-to-functional-alias-ind> set to "true" if the functional alias is used as a target of the call request;

iii)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP INVITE request, the <functional-alias-URI> element set to the URI of the used functional alias; and

iv)	if the MCData user has requested an application priority, the <anyExt> element with the <user-requested-priority> element set to the user provided value;

c)	if an end-to-end security context needs to be established and the security context does not exist or if the existing security context has expired, then:

i)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [26];

ii)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [26];

iii)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect one-to-one communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.180 [26];

iv)	shall encrypt the PCK to a UID associated to the MCData client using the MCData ID of the invited user and a time related parameter as described in 3GPP TS 33.180 [26];

v)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [26]; and

vi)	shall add the MCData ID of the originating MCData user to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26]; and

vii)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCData user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.180 [26]; and

d)	if the MCData emergency private communication state is set to either "MDEPC 2: emergency-pc-requested" or "MDEPC 3: emergency-pc-granted" or if the MCData emergency private priority state of this one-to-one communication is set to a value other than "MDEPP 2: in-progress" or "MDEPP 3: confirm-pending", shall execute the procedures in clause 6.2.8.3.3 to include the Resource-Priority header field;

9)	if a group file distribution is requested:

a)	if the "/<x>/<x>/Common/MCData/AllowedFD" leaf node present in the group document of the requested MCData group, configured on the group management client as specified in 3GPP TS 24.483 [42] is set to "false", shall reject the request for FD and not continue with the rest of the steps in this clause;

a1)	if the group document contains a <list-service> element that contains a <preconfigured-group-use-only> element. If a <preconfigured-group-use-only> element exists and is set to the value "true", then the MCData client:

i)	should indicate to the MCData user that group file distribution is not allowed on the indicated group; and

ii)	shall skip the remainder of this procedure; and

b)	shall contain in an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "group-fd";

ii)	the <mcdata-request-uri> element set to the MCData group identity;

iii)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client;

NOTE 1:	The MCData client does not include the MCData ID of the originating MCData user in the body, as this will be inserted into the body of the SIP INVITE request that is sent from the originating participating MCData function.

iv)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP INVITE request, the <functional-alias-URI> element set to the URI of the used functional alias; and

v)	if the MCData user has requested an application priority, the <anyExt> element with the <user-requested-priority> element set to the user provided value;

c)	if the MCData user has requested the origination of an MCData emergency group communication or is originating an MCData pre-arranged group communication and the MCData emergency state is already set, the MCData client shall execute the procedures in clause 6.2.8.1.1;

d)	if the MCData user has requested the origination of an MCData imminent peril group communication, the MCData client shall execute the procedures in clause 6.2.8.1.9;

e)	if the MCData client emergency group state for this group is set to "MDEG 2: in-progress" or "MDEG 4: confirm-pending", the MCData client shall execute the procedures in clause 6.2.8.1.2 to include the Resource-Priority header field; and

f)	if the MCData client imminent peril group state for this group is set to "MDIG 2: in-progress" or "MDIG 4: confirm-pending", shall execute the procedures in clause 6.2.8.1.12 to include the Resource-Priority header field;

10)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCData function serving the MCData user;

NOTE 2:	The MCData client is configured with public service identity identifying the participating MCData function serving the MCData user.

11)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [5];

12)	shall include an SDP offer according to 3GPP TS 24.229 [5] with the clarifications given in clause 10.2.5.2.1; and

13)	shall send the SIP INVITE request towards the MCData server according to 3GPP TS 24.229 [5].

Upon receiving a SIP 300 (Multiple Choices) response to the SIP INVITE request the MCData client shall use the MCData ID of MCData user contained in the <mcdata-request-uri> element of the received application/vnd.3gpp.mcdata-info MIME body as the MCData ID of the invited MCData user and shall generate an initial SIP MCData request by following the UE originating session procedures specified in 3GPP TS 24.229 [5], with the clarifications given in this clause and with the following additional clarifications:

1)	shall insert in the newly generated SIP INVITE request a MIME resource-lists body with the MCData ID of the invited MCData user in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info MIME body  in the received SIP 300 (Multiple Choices) response;

2)	shall not include a <call-to-functional-alias-ind> element into the <mcdata-Params> element of the <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body; and

3)	shall include a <called-functional-alias-URI> element into the <mcdata-Params> element of the <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body with the target functional alias used in the initial SIP INVITE request for establishing a session for one-to-one file distribution.

On receipt of a SIP 2xx response to the SIP INVITE request, the MCData client:

0)	if the response is to a SIP INVITE request for an MCData emergency group an MCData imminent peril group communication, shall perform the actions specified in clause 6.2.8.1.4;

1)	if the response is to a SIP INVITE request for an MCData emergency one-to-one communication, shall perform the actions specified in clause 6.2.8.3.4;

2)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5];

3)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38]; and

4)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 7.1.2.

On receipt of a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request, the MCData client:

0)	if the response is to a SIP INVITE request for an MCData emergency group communication an MCData imminent peril group communication, shall perform the actions specified in clause 6.2.8.1.5;

1)	if the response is to a SIP INVITE request for an MCData emergency one-to-one communication, shall perform the actions specified in clause 6.2.8.3.5;

2)	shall indicate to the MCData user that the file could not be sent; and

3)	shall send a SIP ACK request as specified in 3GPP TS 24.229 [5].

On receipt of a SIP INFO request where the Request-URI contains an MCData session ID identifying an ongoing group session, the MCData client shall follow the actions specified in clause 6.2.8.1.13.

On receipt of a SIP INFO request where the Request-URI contains an MCData session ID identifying an ongoing one to-one session, the MCData client shall follow the actions specified in clause 6.2.8.3.7.

On receipt of an indication from the media plane indicating that the file was not sent successfully, the MCData client shall:

1)	shall generate a SIP BYE request according to 3GPP TS 24.229 [5] with:

a)	Reason code set to "SIP";

b)	cause set to "480"; and

c)	text set to "transmission failed";

2)	shall set the Request-URI to the MCData session identity to release; and

3)	shall send a SIP BYE request towards MCData server according to 3GPP TS 24.229 [5].

6.2.17.3	Test description

6.2.17.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

-	Test File 1 for CO FD as specified in annex A.2.1 as specified in Annex A.2.2 is available at the UE for upload.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2 with the following clarifications:

-	In the group configuration document the <preconfigured-group-use-only> element is set to "true".

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.2.17.3.2	Test procedure sequence

Table 6.2.17.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) send test file 1 (Annex A.2.1) for CO group FD over media plane. (NOTE 1, NOTE 2) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send an SIP INVITE request to establish a MCData call within the next 5s? | --> | SIP INVITE | 1 | F |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication to the MCData user that group standalone FD is not allowed on the indicated group1. | - | - | - | - |
| 3a1 | IF pc_MCX_DisplayInfoFDNotAllowedOnGroup THEN Check: Does the UE (MCData client) indicate to the MCData user that group standalone FD is not allowed on the indicated group? (NOTE 1) | - | - | 1 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 1 for CO FD as specified in annex A.2.1. |  |  |  |  |  |

6.2.17.3.3	Specific message contents

Table 6.2.17.3.3-1: Group Configuration document (Preamble)

| Derivation Path: TS 37.579-1 [2], Table 5.5.7.4-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service[1] |  |  |  |  |
| mcpttgi:preconfigured-group-use-only | true |  | TS 24.481 [31] clause 7.2.4.2 |  |