3GPP TSG-RAN5 Meeting #109	R5-256898

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4176 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Annex F for ATG test cases including TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Annex F need to be updated based on TT analysis for the following ATG RRM test cases: - 1.19.2.3.2.1 NR SA FR1 4-step RA type contention based random access test for ATG 2.19.2.3.2.2 NR SA FR1 4-step RA type non-contention based random access test for ATG |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex F has been updated based on TT analysis results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.903 CR 1096 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-255646r1. |  |  |  |  |  |  |  |  |

==============First change==============

F.1.1.2	Measurement of RRM requirements

This clause defines the maximum test system uncertainty for the RRM requirements. The maximum uncertainty values allowed for the typical RRM measurement uncertainty contributors is defined in Table F.1.1.2-1 and Table F.1.1.2-2. Unless explicitly stated for a particular test case, these maximum uncertainty values should be used as starting point to perform the test tolerance analysis in TR 38.903 [22] for each of the test cases. Specific test cases might require a tighter measurement uncertainty value for some of the contributors. Exceptions to the general values in Table F.1.1.2-1 and Table F.1.1.2-2 shall be handled case by case.

Table F.1.1.2-1: Maximum allowed measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Cell BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | ±1.5 |  |
| AWGN absolute power, Noc for Measurement PRB | dB | ±1.5 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 | Same as in LTE |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based, or configured RBs in the SRS resource for CLI-based |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx | Depends on fading profile, can be referenced from TS 38.101-4 [20] |
| AWGN and signal flatness* | dB | ±2.0 | Same as in LTE, can be referenced from TS 38.101-4 [20] |
| Uplink absolute power measurement | dB | ±1.5 | Same as in TS 38.521-1 [17] |
| Uplink relative power measurement | dB | ±0.7 | Same as in TS 38.521-1 [17] |
| Uplink signal transmit timing relative to downlink | Tc | ±112 | Calculated as 25% of tightest UE core requirement for FR1 (25% of 7*64*Tc) |
| Uplink signal transmit timing relative to downlink for satellite access | Tc | ±352 | Calculated as 25% of tightest UE core requirement for FR1 for satellite access (25% of 22*64*Tc) |
| Uplink signal transmit timing relative to downlink for air-to-ground | Tc | ±256 | Calculated as 25% of tightest UE core requirement for FR1 for air-to-ground (25% of 16*64*Tc) |
| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 | Calculated as 25% of tightest UE core requirement for FR1 (25% of 5.5*64*Tc) |
| Timing Advance Adjustment accuracy | Tc | ±88 | Calculated as 25% of tightest UE core requirement for FR1 (25% of 5.5*64*Tc) |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combined these values in the TT analysis in TR 38.903 Note 2: These values apply for cell BW ≤ 40 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. Note 3: Void |  |  |  |

<<skipped part>>

Table F.1.1.2-17: Maximum test system uncertainty for RRM requirements for SA FR1 Air-to-ground test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 19.2.1.1 Intra-frequency handover from FR1 to FR1; known target cell | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 19.2.1.2 Inter-frequency handover from FR1 to FR1; unknown target cell | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 19.2.2.1 Intra-frequency distance-based conditional Handover from FR1 to FR1 | Same as 6.3.3.1 | Same as 6.3.3.1 |
| 19.2.2.2 Inter-frequency distance-based conditional Handover from FR1 to FR1 | Same as 6.3.3.2 | Same as 6.3.3.2 |
| 19.2.3.1.1 Intra-frequency RRC Re-establishment in FR1 for ATG | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 |
| 19.2.3.1.2 Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 |
| 19.2.3.2.1 NR SA FR1 4-step RA type contention based random access test for ATG | Noc ±1.5 dB Ês / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±256Tc Uplink signal transmit timing relative to downlink for air-to-ground | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 19.2.3.2.2 NR SA FR1 4-step RA type non-contention based random access test for ATG | Same as 19.2.3.2.1 | Same as 19.2.3.2.1 |
| 19.2.3.3.1 NR SA FR1-FR1 RRC connection release with Redirection for ATG | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 |
| 19.6.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.1 NR SA FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.1 NR SA FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for ATG | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 19.6.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy for ATG | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 19.6.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for ATG | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 19.6.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for ATG | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 19.6.5.1.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 19.6.5.1.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 19.6.5.2 NR SA FR1 L1-SINR absolute measurement accuracy with SSB based CMR and dedicated IMR for ATG | Same as 6.7.9.2 | Same as 6.7.9.2 |
| 19.6.5.3.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 19.6.5.3.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |

==============Next change==============

F.1.3.2	Measurement of RRM requirements

Because the relationships between the test system uncertainties and the test tolerances are often complex, it is not always possible to give a simple derivation of the test requirement in this document. The analysis is recorded in 3GPP TR 38 903 [22].

The derivation of the test requirements for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1.

The derivation of the test requirements for the NE-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1a.

The derivation of the test requirements for the NR/5GC FR1 test cases in chapter 6 is defined in Table F.1.3.2-2.

The derivation of the test requirements for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.3.2-3.

The derivation of the test requirements for the NR/5GC FR2 test cases in chapter 7 is defined in Table F.1.3.2-4.

The derivation of the test requirements for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases in chapter 8 is defined in Table F.1.3.2-5.

The derivation of the test requirements for the FR1 NR sidelink test cases in chapter 9 is defined in Table F.1.3.2-6.

The derivation of the test requirements for the EN-DC FR1 test cases for shared spectrum in chapter 10 is defined in Table F.1.3.2-10.

The derivation of the test requirements for the NR/5GC FR1 test cases for shared spectrum in chapter 11 is defined in Table F.1.3.2-11.

The derivation of the test requirements for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases for shared spectrum in chapter 12 is defined in Table F.1.3.2-12.

The derivation of the test requirements for the NR/5GC FR1 test cases with SCell in shared spectrum in chapter 13 is defined in Table F.1.3.2-13.

The derivation of the test requirements for SA FR1 Satellite access test cases in chapter 14 is defined in Table F.1.3.2-14.

The derivation of the test requirements for the NR/5GC FR1 test cases for RedCap in chapter 16 is defined in Table F.1.3.2-7.

The derivation of the test requirements for the NR/5GC FR2 test cases for RedCap in chapter 17 is defined in Table F.1.3.2-8.

The derivation of the test requirements for the E-UTRA – NR inter-RAT with E-UTRA serving cell test cases for RedCap in chapter 18 is defined in Table F.1.3.2-9.

The derivation of the test requirements for the NR/5GC FR1 test cases for ATG in chapter 19 is defined in Table F.1.3.2-15.

<<skipped part>>

Table F.1.3.2-15: Derivation of test requirements for RRM requirements for SA FR1 Air-to-ground test cases

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 19.2.1.1 Intra-frequency handover from FR1 to FR1; known target cell | Same as 6.3.1.1 | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 19.2.1.2 Inter-frequency handover from FR1 to FR1; unknown target cell | Same as 6.3.1.3 | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 19.2.2.1 Intra-frequency distance-based conditional Handover from FR1 to FR1 | Same as 6.3.3.1 | Same as 6.3.3.1 | Same as 6.3.3.1 |
| 19.2.2.2 Inter-frequency distance-based conditional Handover from FR1 to FR1 | Same as 6.3.3.2 | Same as 6.3.3.2 | Same as 6.3.3.2 |
| 19.2.3.1.1 Intra-frequency RRC Re-establishment in FR1 for ATG | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 |
| 19.2.3.1.2 Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 |
| 19.2.3.2.1 NR SA FR1 4-step RA type contention based random access test for ATG | Absolute uplink power: Normal conditions ±17dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±20*64*Tc 30kHz SCS Te ±16*64*Tc | 2.1dB 0.7dB 256Tc 256Tc | Absolute uplink power: Normal conditions ±19.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ± 1536*Tc 30kHz SCS Te ± 1280*Tc |
| 19.2.3.2.2 NR SA FR1 4-step RA type non-contention based random access test for ATG | Same as 19.2.3.2.1 | Same as 19.2.3.2.1 | Same as 19.2.3.2.1 |
| 19.2.3.3.1 NR SA FR1-FR1 RRC connection release with Redirection for ATG | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 |
| 19.6.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.1 NR SA FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.1 NR SA FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for ATG | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 19.6.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy for ATG | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 19.6.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for ATG | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 19.6.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for ATG | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 19.6.5.1.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 |
| 19.6.5.1.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 |
| 19.6.5.2 NR SA FR1 L1-SINR absolute measurement accuracy with SSB based CMR and dedicated IMR for ATG | Same as 4.7.7.2 | Same as 4.7.7.2 | Same as 4.7.7.2 |
| 19.6.5.3.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | Same as 4.7.7.3.1 | Same as 4.7.7.3.1 | Same as 4.7.7.3.1 |
| 19.6.5.3.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 |

==============End of change==============