3GPP TSG RAN WG5#109	R5-256921

# Dallas, USA, 17 – 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0685 | rev | 1 | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability for eRedCap for PDCCH performance test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | eRedCap has been added in applicability for PDCCH performance test cases 5.3.1.1.1, 5.3.1.2.1, 5.3.2.1.4, 5.3.2.2.4 in 38.521-4. 38.522 needs to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added eRedCap applicability for test cases 5.3.1.1.1, 5.3.1.2.1, 5.3.2.1.4, 5.3.2.2.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The eRedCap requirement for PDCCH performance test cases will be untested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of R5-256353 and the result of 1 revision |  |  |  |  |  |  |  |  |

#### << Start of changes >>

### 4.1.4	Performance conformance test cases

Table 4.1.4-1: Applicability of performance test cases, ref. TS 38.521-4 [4]

| Clause | TC Title | Release | Applicability |  | Tested Bands Selection | Additional Information | Branch | Subtest Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |  |
| 5 | Demodulation performance requirements (Conducted requirements) |  |  |  |  |  |  |  |
| 5.2 | PDSCH demodulation requirements |  |  |  |  |  |  |  |
| << Rows omitted here >> |  |  |  |  |  |  |  |  |
| 5.3.1.1.1 | 1Rx FDD FR1 PDCCH performance for RedCap | Rel-17 | C177a | RedCap UEs supporting 5GS FDD FR1 and 1Rx UE capability | D008 |  |  |  |
|  |  | Rel-18 | C372a | eRedCap UEs supporting 5GS FDD FR1 and 1Rx UE capability | D008 |  |  |  |
| 5.3.1.2.1 | 1Rx TDD FR1 PDCCH performance for RedCap | Rel-17 | C177c | RedCap UEs supporting 5GS TDD FR1 and 1Rx UE capability | D009 |  |  |  |
|  |  | Rel-18 | C372e | eRedCap UEs supporting 5GS TDD FR1 and 1Rx UE capability | D009 |  |  |  |
| 5.3.2.1.1 | 2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.3.2.1.2 | 2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.3.2.1.3 | 2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving | Rel-16 | C088 | UEs supporting 5GS FDD FR1 and Long DRX Cycle and DRX adaptation and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.3.2.1.4 | 2Rx FDD FR1 PDCCH performance for RedCap | Rel-17 | C177b | RedCap UEs supporting 5GS FDD FR1and 2Rx UE capability | D008 |  |  |  |
|  |  | Rel-18 | C372c | eRedCap UEs supporting 5GS FDD FR1and 2Rx UE capability | D008 |  |  |  |
| 5.3.2.1.5 | 2RX FDD Minimum requirements for PDCCH with intra-slot repetition | Rel-17 | C015e | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.3.2.1.7 | 2RX FDD FR1 PDCCH antenna performance for 3 MHz channel bandwidth | Rel-18 | C343a | UEs supporting 5GS FDD FR1 and 2Rx antenna ports and bandwidth of less than 5 MHz, but not supporting FDD bands with 4Rx UE capability | D008a D029 |  |  |  |
| 5.3.2.2.1 | 2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability | D010 D029 |  |  |  |
| 5.3.2.2.2 | 2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability | D010 D029 |  |  |  |
| 5.3.2.2.3 | 2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving | Rel-16 | C089 | UEs supporting 5GS TDD FR1 and Long DRX Cycle and DRX adaptation and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability | D010 D029 |  |  |  |
| 5.3.2.2.4 | 2Rx TDD FR1 PDCCH performance for RedCap | Rel-17 | C177d | RedCap UEs supporting 5GS TDD FR1 and 2Rx UE capability | D009 |  |  |  |
|  |  | Rel-18 | C372g | eRedCap UEs supporting 5GS TDD FR1 and 2Rx UE capability | D009 |  |  |  |
| 5.3.2.2.5 | 2RX TDD Minimum requirements for PDCCH with intra-slot repetition | Rel-17 | C016e | UEs supporting 5GS TDD FR1 and 2Rx antenna ports, but not supporting TDD bands with 4Rx UE capability | D009 D010 D029 |  |  |  |
| #### << Rows omitted here >> |  |  |  |  |  |  |  |  |

#### << End of changes >>