3GPP TSG-RAN5 Meeting #109	R5-256455

# Dallas, Texas, USA, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3613 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of EVM for Tx Diversity for UE supporting 4Tx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For EVM measurements the power level has to be above Pmin. However, in the test procedure of 4Tx test cases this is not considered in all cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The antenna connectors at which the EVM is to be measured have been clarified in the test procedure and Annex E. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4G.2.1_1.4.2, E.4.8, E.5.9.4, E.6.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 6.4G.2.1_1	Error Vector Magnitude for Tx Diversity for UE supporting 4Tx

< Unchanged sections omitted >

6.4G.2.1_1.4.2	Test procedure

Same as the test procedure given in clause 6.4G.2.1.4.2 with following exceptions:

-	Step 1.4, 1.7, 2.5, 2.8 and 3.8 are replaced by the following steps respectively:

Test procedure for PUSCH:

1.4.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E). For TDD, only slots consisting of only UL symbols are under test.

-	If the UE’s transmit power measured in step 1.3 is higher than the minimum output power requirement in 6.3.1.5 at more than one antenna connectors, the composite EVM is measured according to E.4.8 using these antenna connectors.

-	If UE’s transmit power measured in step 1.3 is higher than the minimum output power requirement in 6.3.1.5 at only one antenna connector, the EVM is measured only on this antenna connector according to E.4.2 and E.4.6.

1.7.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E). For TDD, only slots consisting of only UL symbols are under test.

-	If the UE’s transmit power measured in step 1.6 is higher than the minimum output power requirement in 6.3.1.5 at more than one antenna connectors, the composite EVM is measured according to E.4.8 using these antenna connectors.

-	If UE’s transmit power measured in step 1.6 is higher than the minimum output power requirement in 6.3.1.5 at only one antenna connector, the EVM is measured only on this antenna connector according to E.4.2 and E.4.6.

Test procedure for PUCCH:

2.5.	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E).

-	If the UE’s transmit power measured in step 2.4 is higher than the minimum output power requirement in 6.3.1.5 at more than one antenna connectors, the composite EVM is measured according to E.5.9.4 using these antenna connectors.

-	If UE’s transmit power measured in step 2.4 is higher than the minimum output power requirement in 6.3.1.5 at only one antenna connector, the EVM is measured only on this antenna connector according to E.5.9.2.

2.8.	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E).

-	If the UE’s transmit power measured in step 2.7 is higher than the minimum output power requirement in 6.3.1.5 at more than one antenna connectors, the composite EVM is measured according to E.5.9.4 using these antenna connectors.

-	If UE’s transmit power measured in step 2.7 is higher than the minimum output power requirement in 6.3.1.5 at only one antenna connector, the EVM is measured only on this antenna connector according to E.5.9.4.

Test procedure for PRACH:

3.8.	Repeat step 3.5 ~ 3.7 until the SS collect enough PRACH preambles. Measure the EVM in PRACH channel using Global In-Channel Tx-Test (Annex E).

-	If the UE’s transmit power measured in step 3.7 is higher than the minimum output power requirement in 6.3.1.5 at more than one antenna connectors, the composite EVM is measured according to E.6.9.3 using these antenna connectors.

-	If UE’s transmit power measured in step 3.7 is higher than the minimum output power requirement in 6.3.1.5 at only one antenna connector, the EVM is measured only on this antenna connector according to E.6.9.2.

< Unchanged sections omitted >

## E.4.8	EVM measurement for multiple Tx

For UE with multiple transmission antennas, if UE indicates Tx diversity capability, EVM is determined as follows: If the UE’s transmit power is higher than the minimum output power requirement in 6.3.1.5 at k antenna connectors, EVM is measured at these antenna connectors to get EVMi. The total EVM is calculated by values of EVMi with weighting factor of linear power Pi measured at the corresponsing antenna connector.

EVM=i=1kPi*EVMii=1kPi

< Unchanged sections omitted >

### E.5.9.4	EVM measurement for multiple Tx

For UE with multiple transmission antennas, if UE indicates Tx diversity capability, EVM is determined as follows:  If the UE’s transmit power is higher than the minimum output power requirement in 6.3.1.5 at k antenna connectors, EVM is measured at these antenna connectors to get EVMi. The total EVM is calculated by values of EVMi with weighting factor of linear power Pi measured at the corresponding antenna connector.

EVM=i=1kPi*EVMii=1kPi

< Unchanged sections omitted >

### E.6.9.3	EVM measurement for multiple Tx

For UE with multiple transmission antennas, if UE indicates Tx diversity capability, EVM is determined as follows: If the UE’s transmit power is higher than the minimum output power requirement in 6.3.1.5 at k antenna connectors, EVM is measured at these antenna connectors to get EVMi. The total EVM is calculated by values of EVMi with weighting factor of linear power Pi measured at the corresponding antenna connector.

EVM=i=1kPi*EVMii=1kPi

< End of changes >