3GPP TSG-RAN5 Meeting #109	R5-256425

# Dallas, USA, November 17 - 21 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0672 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR RSTD measurement accuracy in FR1 for Redcap UE with Rx FH test case 14.3.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR RSTD measurement reporting delay test case 14.3.13 is not complete |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been corrected with the following aspects: - Test procedure defined - Message content added - Applicability defined - Editors notes corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/// Start of changes

### 14.3.13	NR RSTD measurement accuracy test case in FR1 for Redcap UE with Rx FH

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is missing.

#### 14.3.13.1	Test purpose

The purpose of the test is to verify that the RSTD measurement for RedCap UE without FH in RRC CONNECTED state meets the accuracy requirements specified in TS 38.133 [50] clause 10.1A.16.2 in an environment with AWGN propagation conditions.

#### 14.3.13.2	Test applicability

This test applies to RedCap UE release 18 onwards that support DL-TDOA positioning.

#### 14.3.13.3	Minimum conformance requirements

The accuracy requirements for RSTD measurement shall be within ±(X+Y+Z+Δ) Tc. The values of Y, Z and Δ and Rx TEG based requirement are as defined in TS 38.133 [50] clause 10.1.23.2. For Rx FH, PRS BW in TS 38.133 [50] in tables 10.1.23.2-5, 10.1.23.2-5a, 10.1.23.2-6, and 10.1.23.2-6a refer to per hop BW. The requirements for fading channel in this clause are derived based on TDL-A (30 ns delay spread, 5Hz) and TDL-C (60 ns delay spread, 300 Hz) channel models for FR1 and FR2, respectively.

#### 14.3.13.4	Test description

##### 14.3.13.4.1	Initial conditions

The supported test configurations in listed in Table 14.3.13.4-1.

Table 14.3.14.4-1: Test Configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, cell bandwidth 50 Hz, FDD duplex mode |
| 2 | 15 kHz SSB SCS, cell bandwidth 50 Hz, TDD duplex mode |
| 3 | 30 kHz SSB SCS, cell bandwidth 100 Hz, TDD duplex mode |
| 4 | 15 kHz SSB SCS, cell bandwidth 50 Hz, HD-FDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: UE with 1 Rx or 2 Rx is required to meet the same requirements specified in this clause. NOTE 3: This test case is applicable to the UE supporting per hop bandwidth ≥ 10 MHz for SCS 15 kHz. NOTE 4: This test case is applicable to the UE supporting per hop bandwidth = 20 MHz for SCS 30 kHz. |  |

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid-Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 14.3.13.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.14.

2.	The general test parameter settings are set up according to Table 14.3.13.5-1.

3.	Propagation conditions are set according to clause 4.15.2.

4.	Message contents are defined in clause 14.3.13.4.3.

5.	In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured.

6.	The true RSTD (which is the receive time difference for frame 0 between cell 2 and cell 1 as seen at the UE antenna connector) is set to 5898 Tc (about 3 s)

##### 14.3.13.4.2	Test procedure

Same test procedure as in 14.3.11.

##### 14.3.13.4.3	Message contents

Same test message content as in 14.3.9.4.3 with the following exceptions:

Table 14.3.13.4.3-1 replaces Table 14.2.9.4.3-3, Table 14.3.13.4.3-2 replaces Table 14.3.9.4.3-5, Table 14.3.14.4.3-3 replaces Table 14.3.9.4.3-6, Table 14.3.13.4.3-4 replaces Table 14.3.9.4.3-7.

Table 14.3.13.4.3-1: LPP RequestLocationInformation

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |
| locationInformationType | locationMeasurementsPreferred |  |  |
| triggeredReporting | Not present |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | See 14.2.13.5 | Result of the response time calculation rounded up to the next second if response time <= 128s. Result of the response time calculation rounded up to the next multiple of ten seconds if response time > 128s |  |
| responseTimeEarlyFix-r12 | Not present |  | Rel-12 onwards |
| unit-r15 | Not present |  |  |
|  | ten-seconds |  | Calculated response time >128s |
| } |  |  |  |
| velocityRequest | FALSE |  |  |
| } |  |  |  |
| environment | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |
| a-gnss-RequestLocationInformation | Not present |  |  |
| otdoa-RequestLocationInformation | Not present |  |  |
| ecid-RequestLocationInformation | Not present |  |  |
| epdu-RequestLocationInformation | Not Present |  |  |
| sensor-RequestLocationInformation-r13 | Not present |  |  |
| tbs-RequestLocationInformation-r13 | Not present |  |  |
| wlan-RequestLocationInformation-r13 | Not present |  |  |
| bt-RequestLocationInformation-r13 | Not present |  |  |
| nr-ECID-RequestLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-TDOA-RequestLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-RstdMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | bit 0 = 0 (prsrsrpReq) |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-TDOA-ReportConfig-r16 | Not present |  |  |
| additionalPaths-r16 | Not present |  |  |
| nr-UE-RxTEG-Request-r17 | Not present |  |  |
| nr-los-nlos-IndicatorRequest-r17 | Not present |  |  |
| additionalPathsExt-r17 | Not present |  |  |
| additionalPathsDL-PRS-RSRP-Request-r17 | Not present |  |  |
| multiMeasInSameReport-r17 | Not present |  |  |
| nr-DL-PRS-JointMeasurementRequest-r18 | Not present |  |  |
| nr-DL-PRS-RxHoppingRequest-r18 SEQUENCE { |  |  |  |
| nr-DL-PRS-RxHoppingTotalBandwidth-r18 CHOICE { |  |  |  |
| fr1 | mhz50 |  | Configuration 1, Configuration 2 and Configuration 4 |
|  | mhz100 |  | Configuration 3 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.3.13.4.3-2: NR-DL-PRS-AssistanceData

| Derivation Path: TS 37.355 [49] clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz15 |  | Config 1 Config 2 and Config 4 |
|  | kHz30 |  | Config 3 |
| dl-PRS-ResourceBandwidth-r16 | 62 | 268 PRBs | Configuration 1, Configuration 2 and Configuration 4 |
|  | 63 | 272 PRBs | Configuration 3 |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA + offsetToCarrier, as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n4 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 3 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.15.4.3-6 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 | 0 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 38 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.15.4.3-6 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[3] SEQUENCE { |  | entry 3 | Cell 3 |
| dl-PRS-ID-r16 | 2 |  |  |
| nr-PhysCellID-r16 | Cell 3 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | Cell 3 |  |  |
| mnc-r15 | Cell 3 |  |  |
| nr-cellidentity-r15 | Cell 3 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | Cell 3 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 38 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.15.4.3-6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-SSB-Config-r16 | Not present |  |  |
| nr-DL-PRS-AggregationInfo-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.3.13.4.3-3: NR-DL-PRS-Info

| Derivation Path: TS 37.355 [49] clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs15-r16 CHOICE { |  |  | Config 1, Config 2 and Config 4 |
| n160-r16 | 10 |  |  |
| } |  |  |  |
| scs30-r16 CHOICE { |  |  | Config 3 |
| n160-r16 | 10 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | n6 |  |  |
| dl-PRS-ResourceTimeGap-r16 | s1 |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 SEQUENCE { |  |  |  |
| dl-prs-MutingBitRepetitionFactor-r16 | Not present |  |  |
| nr-option1-muting-r16 CHOICE { |  |  |  |
| po2-r16 | 10 |  | Cell 1 and Cell 3 |
|  | 01 |  | Cell 2 |
| } |  |  |  |
| } |  |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n2-r16 | 0 |  | Cell 1 and Cell 2 |
|  | 1 |  | Cell 3 |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  |  |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.3.13.4.3-4: LPP ProvideLocation Information

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | (0..255) |  |  |
| acknowledgement |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | May be present. Any value acceptable. |  |  |
| a-gnss-ProvideLocationInformation | Not present |  |  |
| otdoa-ProvideLocationInformation | Not present |  |  |
| ecid-ProvideLocationInformation | Not present |  |  |
| epdu-ProvideLocationInformation | Not present |  |  |
| sensor-ProvideLocationInformation-r13 | Not present |  |  |
| tbs-ProvideLocationInformation-r13 | Not present |  |  |
| wlan-ProvideLocationInformation-r13 | Not present |  |  |
| bt-ProvideLocationInformation-r13 | Not present |  |  |
| nr-ECID-ProvideLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 | Not present |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 SEQUENCE { |  |  |  |
| dl-PRS-ReferenceInfo-r16 |  |  |  |
| nr-DL-TDOA-MeasList-r16 SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16 { | 3 entries |  |  |
| NR-DL-TDOA-MeasElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 |  |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-RSTD-r16 | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |
| nr-TimingQuality-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |
| } |  |  |  |
| NR-DL-TDOA-MeasElement-r16[2] SEQUENCE { |  | entry 2 |  |
| dl-PRS-ID-r16 |  |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-RSTD-r16 | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |
| nr-TimingQuality-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |
| } |  |  |  |
| NR-DL-TDOA-MeasElement-r16[3] SEQUENCE { |  | entry 3 |  |
| dl-PRS-ID-r16 |  |  |  |
| nr-PhysCellID-r16 | Cell 3 |  |  |
| nr-CellGlobalID-r16 |  |  |  |
| nr-ARFCN-r16 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |
| nr-TimeStamp-r16 |  |  |  |
| nr-RSTD-r16 | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |
| nr-TimingQuality-r16 |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |
| } |  |  |  |
| nr-dl-tdoa-LocationInformation-r16 |  |  |  |
| nr-DL-TDOA-Error-r16 |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 14.3.13.5	Test requirement

Table 14.3.13.5-1 define the primary level settings including test tolerances for the test.

Table 14.3.13.5-1: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| PRS ARFCN | 1,2,3,4 |  | freq1 | Freq1 |
| BWchannel | 1 | MHz | 50: NRB,c = 270 |  |
|  | 2 |  | 50: NRB,c = 270 |  |
|  | 3 |  | 100: NRB,c = 273 |  |
|  | 4 |  | 50: NRB,c = 270 |  |
| Duplex mode | 1 |  | FDD |  |
|  | 2 |  | TDD |  |
|  | 3 |  | TDD |  |
|  | 4 |  | HD-FDD |  |
| TDD configuration | 1 |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
|  | 4 |  | N/A |  |
| Measurement gap | 1, 2, 3, 4 |  | GP#24 or GP#0 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 TDD |  |
|  | 4 |  | SR.1.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 TDD | - |
|  | 4 |  | CR.1.1 FDD |  |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - |
|  | 4 |  | CCR.1.1 FDD |  |
| SSB configuration | 1 |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.1 RedCap FR1 |  |
|  | 4 |  | SSB.1 FR1 |  |
| OCNG Patterns | 1,2,3,4 |  | OP.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD | - |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
|  | 4 |  | TRS.1.1 FDD |  |
| Initial BWP Configuration | 1,2,3,4 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1,2,3,4 |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 1 | 1,2,3,4 | s | - | 3 |
| SMTC configuration | 1,4 |  | SMTC.2 |  |
|  | 2,3 |  | SMTC.1 |  |
| PRS configuration | 1 |  | PRS.1.5 FR1 |  |
|  | 2 |  | PRS.1.5 FR1 |  |
|  | 3 |  | PRS.2.6 FR1 |  |
|  | 4 |  | PRS.1.5 FR1 |  |
| PRS Resource slot offset | 1, 2, 3, 4 | slot | 0 | 4 |
| Expected RSTD | 1, 2, 3, 4 | s | N/A | 3 |
| Expected RSTD uncertainty | 1, 2, 3, 4 | s | N/A | 5 |
| EPRE ratio of PSS to SSS | 1,2,3,4 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS | 1,2,3,4 | dB | 0 | 0 |
| Note2 | 1,2,4 | dBm/ SCS | -98 |  |
|  | 3 |  | -95 |  |
| PRS | 1,2,3,4 | dB | -6 | -13 |
| PRPNote3 | 1,2,4 | dBm/SCS | -103.7 | -109.9 |
|  | 3 |  | -100.7 | -106.9 |
| IoNote3 | 1,2,4 | dBm/ 48.6MHz | -61.64 | -61.64 |
|  | 3 | dBm/ 98.28MHz | -58.51 | -58.51 |
| PRS | 1,2,3,4 | dB | -5.7 | -11.9 |
| Propagation condition | 1,2,3,4 | - | AWGN |  |
| Antenna configuration | 1,2,3,4 |  | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. The Io is calculated based only on the symbols in which PRS is transmitted. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 14.4.13.3.

TBD

/// End of changes