3GPP TSG-RAN5 Meeting #109	R5-255820

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1638 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EPC IMS DC TC 8.18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to RFC 6665 cl. 3.1.1, the “expires” shall not be set in the “Contact” header. An "expires" parameter on the "Contact" header field has no semantics for the SUBSCRIBE method and is explicitly not equivalent to an "Expires" header field in a SUBSCRIBE request or response. Therefore, for this Re-Registration test case, the “expires” needs to be moved the 200 OK for REGISTER. |  |  |  |  |  |  |  |  |
| Summary of change: |  | “expires” was moved to 200 OK for REGISTER. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

8.18	Re-Registration / IMS data channel

8.18.1	Definition

Test to verify that the UE can re-register a previously registered public user identity at any time when UE supporting IMS data channel and being registered to IMS. This process is described in 3GPP TS 24.186 [159], clause 9.2.2.1.

8.18.2	Conformance requirement

[TS 24.186, clause 9.2.2.1]:

If the UE is allowed to use IMS data channel, on reception of Re-REGISTER request, for user-initiated reregistration, the UE supporting IMS data channel shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type. For the IMS data channel capability indication, the UE shall use +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4].

NOTE:	The policy related to the IMS data channel allowed at the UE, can be provided by the network to the UE using e.g., OMA-DM with the management objects specified in 3GPP TS 24.275 [11] or UICC configuration, as specified in clause 9.2.1.1.

…

On receiving the 200 (OK) response to the REGISTER request, if the 200 (OK) response does not include a Feature-Caps header field containing feature-capability indicator "g.3gpp.datachannel", the UE shall keep established data channel media of the UE's existing IMS session.

Reference(s)

3GPP TS 24.186 [159], clauses 9.2.2.1.

8.18.3	Test purpose

1)	To verify that the UE populates the header field in the re-REGISTER request with a value "webrtc-datachannel" in the Feature-Caps header field of the Contact header field; and

2)	Upon receiving 200 OK without a Feature-Caps header field with the "+g.3gpp.datachannel" header field parameterfor re-REGISTER, the UE shall store the new expiration time of the registration for this public user identity and not determine that the Network supports the IMS data channel capability.

8.18.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE is not registered to IMS services but has an active PDP context and has discovered the SS as P-CSCF by executing the generic test procedure in Annex C.2 up to step 3.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure

1-11) The same procedure as in Annex C.2 is used with the exception that the SS sets the expiration time to 120 seconds in Step 7.

12)	Before half of the time has expired from the initial registration SS receives re-register message request with a value "webrtc-datachannel" in the Feature-Caps header field of the Contact header field.

13)	SS responds to the REGISTER request with valid 200 OK response with the new expiration time (180 seconds) of the registration for this public user identity, but without Feature-Caps header field with the "+g.3gpp.datachannel" header field parameter.

14-26)	The same procedure as in Annex C.21 is used with the exception that the UE sends an INVITE without IMS data channel in the SDP offer.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-11 |  |  | Steps 1-11 of Annex C.2 | The same messages as in Annex C.2 are used with the exception that: in Step 4 of C.2, the UE sends a correctly composed initial REGISTER request including the sip.app-subtype media feature tag with a value "webrtc-datachannel"; in Step 7 of C.2, The SS responds with 200 OK response including 120 seconds expiration time and a Feature-Caps header field with header field parameter "+g.3gpp.datachannel" . |
| 12 |  |  | REGISTER | The SS receives re-REGISTER from the UE 60 seconds before the expiration time set in the initial registration request including the sip.app-subtype media feature tag with a value "webrtc-datachannel". |
| 13 |  |  | 200 OK | The SS responds with 200 OK indicating 180 seconds expiration time, but without Feature-Caps header field with the "+g.3gpp.datachannel" header field parameter. |
| 14 | - |  | Make the UE attempt an IMS voice call. |  |
| 15 |  |  | INVITE | UE sends an INVITE without IMS data channel in the SDP offer. |
| 16-26 | - |  | Steps 2-12 of Annex C.21 |  |

Specific Message Contents

Messages in Steps 1-11

Messages in Steps 1-11 are the same as those specified in Annex C.2 with the following exceptions:

in Step 4 of C.2, use the default message "REGISTER" in Annex A.1.1 with conditions A1, A36

in Step 6 of C.2, use the default message "REGISTER" in Annex A.1.1 with conditions A2, A17, A36

in Step 7 of C.2, use the default message "200 OK for REGISTER" in Annex A.1.3 with conditions A6 and following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| expires | 120 |

in Step 9 of C.2, use the default message "200 OK for SUBSCRIBE" in Annex A.1.5.

REGISTER (Step 12)

Use the default message "REGISTER" in annex A.1.1 with conditions A2, A17, A36 and with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Security-Client |  |
| spi-c | new SPI number of the inbound SA at the protected client port, shall be different from previously used number |
| spi-s | new SPI number of the inbound SA at the protected server port, shall be different from previously used number |
| port-c | new protected client port, shall be different from previously used number |
| port-s | same value as in the previous REGISTER |

200 OK for REGISTER (Step 13)

Use the default message "200 OK for REGISTER" in annex A.1.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| expires | 180 |

INVITE (Step 15)

Use the default message "INVITE for MO Call Setup" in step 2 of Annex C.21.

8.18.5	Test requirements

1.	The UE shall in step 12 send the REGISTER request with a value "webrtc-datachannel" in the Feature-Caps header field of the Contact header field within 60 seconds from the time instant that it receives 200 OK in step 7 from the SS.

2.	The UE shall in step 15 send the INVITE request without IMS data channel in the SDP offer.

==============End of change==============