3GPP TSG-WG5 Meeting #109	R5-256382

# Dallas, USA, Nov 17 – 21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3607 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating 6.2D.3 message contents for NS_17 and NS_18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_n28_PC2_40MHz_CBW-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PC2 requirement for NS_18 on band n28 was introduced in 6.2D.3, but the message content is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the message content of NS_18 Updating test requirement table to add band information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is incomplete.S |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.3.4.3.25 (new), 6.2D.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.2D.3.4.3.24	Message contents exceptions for network signalling value "NS_47"

1.	Information element additionalSpectrumEmission is set to NS_47. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.24-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_47) |  |  |

6.2D.3.4.3.25	Message contents exceptions for network signalling value "NS_18"

1.	Information element additionalSpectrumEmission is set to NS_18. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.25-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_18"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_18) |  | n28 |

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-19. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

============= Next change==============

Table 6.2D.3.5-31: UE Power Class 2 test requirements (NS_18) for n28

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 3, 5 | 26 | 4.0 | 5.5 | 0 | 20.5 | 5 | 3 | 28+TT | 15.5-TT |
|  | 10, 15, 20 | 26 | 4.0 | 9.5 | 0 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 3, 4 | 3, 5 | 26 | 4.0 | 5.5 | 0 | 20.5 | 5 | 3 | 28+TT | 15.5-TT |
|  | 10, 15, 20 | 26 | 4.0 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 5, 6 | 3, 5 | 26 | 4.5 | 6 | 0 | 20 | 5 | 3 | 28+TT | 15-TT |
|  | 10, 15, 20 | 26 | 4.5 | 11.5 | 0 | 14.5 | 6 | 3 | 28+TT | 8.5-TT |
| 7, 8 | 3, 5 | 26 | 8.0 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
|  | 10, 15, 20 | 26 | 8.0 | 11.5 | 0 | 14.5 | 6 | 3 | 28+TT | 8.5-TT |
| 9, 10, 11 | 25 | 26 | 8.0 | 5.5 | 0 | 18 | 5 | 3 | 28+TT | 13-TT |
|  | 30, 40 | 26 | 8.5 | 5.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 12 | 25 | 26 | 8 | 12.5 | 0 | 13.5 | 6 | 3 | 28+TT | 7.5-TT |
|  | 30, 40 | 26 | 8.5 | 12.5 | 0 | 13.5 | 6 | 3 | 28+TT | 7.5-TT |
| 13 | 25 | 26 | 4.5 | 7 | 0 | 19 | 5 | 3 | 28+TT | 14-TT |
|  | 30, 40 | 26 | 5 | 7 | 0 | 19 | 5 | 3 | 28+TT | 14-TT |
| 14 | 25 | 26 | 8 | 8 | 0 | 18 | 5 | 3 | 28+TT | 13-TT |
|  | 30, 40 | 26 | 8.5 | 8 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 15, 16 | 25 | 26 | 3.5 | 4.5 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
|  | 30, 40 | 26 | 4 | 4.5 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 17, 18 | 25 | 26 | 4.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16-TT |
|  | 30, 40 | 26 | 5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16-TT |
| 19, 20 | 25 | 26 | 8 | 6.5 | 0 | 18 | 5 | 3 | 28+TT | 13-TT |
|  | 30, 40 | 26 | 8.5 | 6.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |

==============End of change==============