3GPP TSG-RAN5 Meeting #109	R5-255853

# Dallas, Texas, USA, 17th - 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1639 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections in SDP message contents |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.23a, 16.4, C.39, C.40 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

## 12.23a	MO MTSI speech call / EVS / Default configuration

### 12.23a.1	Definition

Test to verify that the UE correctly performs IMS mobile originated voice call setup with EVS when using IMS Multimedia Telephony. This process is described in 3GPP TS 24.229 [10], clauses 5.1.3 and 6.1, TS 24.173 [65] and TS 26.114 [66].

### 12.23a.2	Conformance requirement

Same as TC 12.23.

### 12.23a.3	Test purpose

1)	To verify that when initiating MO MTSI speech call and SS needs to reserve resources, the UE performs correct exchange of SIP protocol signalling messages for setting up the session.

2)	To verify that within SIP signalling the UE performs the correct exchange of SIP header and parameter contents.

3)	To verify that within SIP signalling the UE performs the correct exchange of SDP contents.

4)	To verify that the UE is able to release the call.

### 12.23a.4	Method of test

Initial conditions

UE contains ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security).

Test procedure applicable for a UE with EN-DC support (TS 38.508-2 [158] A.4.1-3/2)

1-14)	The UE executes the procedure described in TS 36.508 [94] table 4.5A.19.3-1 steps 1 to 14.

NOTE:	No NR cell is required to be setup in the preamble

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Step 1 defined in annex C.44 | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.19.3-1 for a UE with EN-DC support. |
| 2 |  |  | INVITE | UE sends INVITE with the first SDP offer. |
| 3 |  |  | Step 3 defined in annex C.44 | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.19.3-1 for a UE with EN-DC support. |
| 4 |  |  | 183 Session Progress | SS sends an SDP answer. |
| 5 |  |  | PRACK | UE acknowledges and optionally offers the second SDP if a dedicated EPS bearer is already established by the network. |
| 6 |  |  | 200 OK | SS sends a 200 OK and answers the second SDP if present. |
| 7 |  |  | UPDATE | Optional step: UE sends an UPDATE with the second SDP if the second SDP was not sent at step 5. |
| 8 |  |  | 200 OK | Optional step: SS sends a 200 OK for UPDATE. |
| 9-13 |  |  | Steps 9-13 defined in annex C.44 |  |
| 14 |  |  | The UE is triggered by MMI to release the call |  |
| 15 |  |  | BYE | The UE releases the call with BYE |
| 16 |  |  | 200 OK | The SS sends 200 OK for BYE |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE for MO Call” in annex A.2.1 with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) [Note 9] - b=RR: (bandwidth-value) [Note 9] Attributes for media: - a=rtpmap: (payload type) EVS/16000 [Note 5, 10] - a=fmtp: (format) br=5.9-24.4; bw=nb-swb; max-red= (att-field) [Note 6, 7] - a=rtpmap: (payload type) AMR-WB/16000 [Note 5, 10] - a=fmtp: (format) mode-change-capability=2; max-red= (att-field) [Note 6, 8] - a=rtpmap: (payload type) telephone-event/16000 - a=fmtp: (format) - a=rtpmap: (payload type) AMR/8000 [Note 5, 10] - a=fmtp: (format) mode-change-capability=2; max-red= (att-field) [Note 6, 8] - a=rtpmap: (payload type) telephone-event/8000 - a=fmtp: (format) - a=ecn-capable-rtp: leap ect=0 [Note 2] - a=rtcp-fb:* nack ecn [Note 2] - a=rtcp-xr:ecn-sum [Note 2] - a=rtcp-rsize [Note 2] - a=ptime:20 - a=maxptime:240 Attributes for media security mechanism: - a=3ge2ae: requested [Note 3] - a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:WVNfX19zZW1jdGwgKCkgewkyMjA7fQp9CnVubGVz¦2^20¦ 1:4FEC_ORDER=FEC_SRTP" [Note 3] Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: Attributes for ECN Capability may be present if the UE supports Explicit Congestion Notification. Note 3: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 4: Void Note 5: The channel number shall be “/1” or omitted. Note 6: Values from 0 to 220 are allowed in the att-field. Note 7: The parameters dtx, dtx-recv and evs-mode-switch shall not be present. Note 8: The parameters mode-set, mode-change-period, mode-change-neighbor, crc, robust-sorting and interleaving shall not be present. Note 9: The RR value must be greater than 0. The RS value can be any value. Note 10: The ordering of payload types shall be as listed, i.e., EVS before AMR-WB before AMR. |

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:65 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1, 4] - b=AS:65 - b=RS: (bandwidth-value) [Note 5] - b=RR: (bandwidth-value) [Note 5] Attributes for media: - a=rtpmap: (payload type) EVS/16000/1 [Note 1] - a=fmtp: (format) br=5.9-24.4; bw=nb-swb; max-red=220 - a=ecn-capable-rtp: leap ect=0 [Note 2] - a=rtcp-fb:* nack ecn [Note 2] - a=rtcp-xr:ecn-sum [Note 2] - a=ptime:20 - a=maxptime:240 Attributes for media security mechanism: - a=3ge2ae: requested [Note 3] - a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:PS1uQCVeeCFCanVmcjkpPywjNWhcYD0mXXtxaVBR¦2^20¦1:4 [Note 3] Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv - a=conf:qos remote sendrecv Note 1: The values for fmt, payload type and format are copied from step 2. Note 2: Attributes for ECN Capability are present if the UE supports Explicit Congestion Notification. Note 3: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 4: Transport port is the port number of the SS (see RFC 3264 clause 6). Note 5: The bandwidth-value is copied from step 2. |

PRACK (Step 5)

Use the default message “PRACK” in annex A.2.4 with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Require option-tag | precondition (shall be present if SDP message-body present) |
| Message-body | Contents if present: The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 3] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) EVS/16000 [Note 3] [Note 5] - a=fmtp: (format) [Note 3][Note 4] Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one Note 3: The value for fmt, payload type and format is not checked Note 4: Parameters for the codec are not checked Note 5: The channel number shall be “/1” or omitted. |

200 OK for PRACK (Step 6)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition (shall be present if SDP message-body present) |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length | Contents if header Content-Type is present: |
| Value | length of message-body |
| Message-body | Contents present if PRACK (step 5) contained a SDP. SDP body of the 200 OK response copied from the received PRACK in step 4 and modified as follows: - IP address on "c=" lines and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; - "o=" line identical to previous SDP sent by SS except that sess-version is incremented. Attributes for preconditions: - a=curr:qos remote sendrecv |

UPDATE (Step 7)

Use the default message “UPDATE” in annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require | Same contents as specified in step 5. |
| Message-body | Same contents as specified in step 5. |

200 OK for UPDATE (Step 8)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| Value | length of message-body |
| Message-body | SDP body of the 200 response copied from the received UPDATE in step 7 and modified as follows: - IP address on "c=" lines and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; - “o=” line identical to previous SDP sent by SS except that sess-version is incremented. Attributes for preconditions: - a=curr:qos remote sendrecv |

### 12.23a.5	Test requirements

The UE shall send requests and responses as described in clause 12.23a.4.

<End of modification>

<Start of modification>

## 16.4	Speech AMR-WB, indicate selective codec modes

### 16.4.1	Definition

Test to verify that the UE correctly performs IMS Multimedia Telephony speech call setup when selective AMR-WB codec modes are offered. This process is described in 3GPP TS 24.173 [65], TS 24.229 [10] and TS 26.114 [66].

### 16.4.2	Conformance requirement

Same as 34.229-1 clause 16.3.2.

### 16.4.3	Test purpose

1) To verify that, when initiating MT MTSI speech AMR-WB call with selective codec modes and with the remote UE already having resources available, the UE performs correct exchange of SIP protocol signalling messages for setting up the session.

2) To verify that within SIP signalling the UE performs the correct exchange of SIP header and parameter contents.

3) To verify that within SIP signalling the UE performs the correct exchange of SDP contents.

4) To verify that the UE is able to release the call.

### 16.4.4	Method of test

Same as 34.229-1 clause 16.3.4 except

Specific Message Contents

INVITE (Step 1)

Use the default message "INVITE for MT Call" in annex A.2.9, with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - b=AS:38 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP 97 98 99 100 - c= IN (addrtype) (connection-address for SS) - b=AS:38 - b=RS:0 - b=RR:2000 Attributes for media: - a=rtpmap:97 AMR-WB/16000/1 - a=fmtp:97 mode-set=0,1,2; mode-change-capability=2; max-red=220 - a=rtpmap: 98 telephone-event/16000 - a=fmtp: 98 0-15 - a=rtpmap:99 AMR/8000/1 - a=fmtp:99 mode-set=0,2,4,7; mode-change-capability=2; max-red=220 - a=rtpmap: 100 telephone-event/8000 - a=fmtp: 100 0-15 - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv |

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Status-Line |  |
| Reason-Phrase | Not checked |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 2] - a=fmtp: (format) mode-set=0,1,2; Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. |

180 Ringing (Step 9)

Use the default message “180 Ringing for INVITE” in annex A.2.6 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length | header shall be present if UE uses TCP to send this message and if there is a message body |
| value | length of message-body |
| Message-body | optional if 183 Session Progress is not used not present if 183 Session Progress is used (step 4) Contents if present: The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c= IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap:(payload type) AMR-WB/16000 [Note 2] - a=fmtp:(format) mode-set=0,1,2; Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The AMR channel number shall be “/1” or omitted. |

200 OK for INVITE (Step 12)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length | header shall be present if UE uses TCP to send this message and if there is a message body |
| value | length of message-body |
| Message-body | not present if 183 Session Progress is used (step 4) or 180 Ringing (step 9) contained SDP. present if 183 Session Progress is not used (step 4) and 180 Ringing (step 9) did not contain SDP. Contents if present: The same requirements for SDP types and values as specified in step 9. |

### 16.4.5	Test requirements

The UE shall send requests and responses as described in clause 16.4.4.

<End of modification>

<Start of modification>

# C.39	Generic test procedure for setting up MTSI MO speech call for rSRVCC - EPS

Test procedure:

- MO speech is initiated on the UE as a result of receiving information from the lower layers that the CS to PS SRVCC access transfer is initiated. The call is initiated towards the URI of the STI-rSR as received during registration.

2)	UE sends an INVITE request to the SS.

3)	SS responds to the INVITE request with a 100 Trying response.

4)	SS responds to the INVITE request with a 200 OK.

5)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for INVITE.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | UE attempt an IMS speech call a result of an initiation of the rSRVCC procedure |  |
| 2 |  |  | INVITE | UE sends INVITE with the first SDP offer. |
| 3 |  |  | 100 Trying | SS sends a 100 Trying provisional response. |
| 4 |  |  | 200 OK | SS responds INVITE with 200 OK. |
| 5 |  |  | ACK | UE acknowledges. |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE for MO Call” in annex A.2.1 with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Request-Line |  |
| Request-URI | sip:sti-sr@atcf.visited2.net NOTE: This value was received by the UE in the preceeding registration procedure |
| To |  |
| addr-spec | sip:sti-sr@atcf.visited2.net NOTE: This value was received by the UE in the preceeding registration procedure |
| Message-body | The following SDP types and values. Session description: - v=0 - o=-(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) [Note 7] - b=RR: (bandwidth-value) [Note 7] Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 8] - a=fmtp: (format) mode-change-capability=2; max-red= (att-field) [Note 9] - a=rtpmap: (payload type) telephone-event/16000 - a=fmtp: (format) - a=ecn-capable-rtp: leap ect=0 [Note 3] - a=rtcp-fb:* nack ecn [Note 3] - a=rtcp-xr:ecn-sum [Note 3] - a=rtcp-rsize [Note 3] - a=ptime:20 - a=maxptime:240 Attributes for media security mechanism: - a=3ge2ae: requested [Note 4] - a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:WVNfX19zZW1jdGwgKCkgewkyMjA7fQp9CnVubGVz¦2^20¦ 1:4FEC_ORDER=FEC_SRTP" [Note 4] Note 1: At least one "c=" field shall be present. Note 2: Void. Note 3: Attributes for ECN Capability may be present if the UE supports Explicit Congestion Notification. Note 4: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 5: Void Note 6: Void Note 7: The RR value must be greater than 0. The RS value can be any value. Note 8: The AMR channel number shall be “/1” or omitted. Note 9: values from 0 to 220 are allowed |

200 OK for INVITE (Step 4)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in annex A.3.1 with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Content-Type |  |
| media-type | application/sdp |
| Feature-Caps |  |
| feature-param | +g.3gpp.ti= (value) Note: The value of this parameter shall be the same as the Transaction Identifier sent by the SS in the preceding CS call setup. |
| Content-Length |  |
| Value | length of message-body |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1, 4] - b=AS:37 - b=RS: (bandwidth-value) [Note 5] - b=RR: (bandwidth-value) [Note 5] Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000/1 [Note 1] - a=fmtp: (format) mode-change-capability=2; max-red=220 [Note 1] - a=ecn-capable-rtp: leap ect=0 [Note 2] - a=rtcp-fb:* nack ecn [Note 2] - a=rtcp-xr:ecn-sum [Note 2] - a=ptime:20 - a=maxptime:240 Attributes for media security mechanism: - a=3ge2ae: requested [Note 1] - a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:PS1uQCVeeCFCanVmcjkpPywjNWhcYD0mXXtxaVBR¦2^20¦1:4 [Note 3] Note 1: The value for fmt, payload type (AMR) and format is copied from step 2. Note 2: Attributes for ECN Capability are present if the UE supports Explicit Congestion Notification. Note 3: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 4: transport port is the port number of the SS (see RFC 3264 clause 6). Note 5: The bandwidth-value is copied from step 2. |

<End of modification>

<Start of modification>

# C.40	Generic test procedure for MTSI MO speech call for rSRVCC in alerting state - EPS

Test procedure:

1)	MO speech is initiated on the UE as a result of receiving information from the lower layers that the CS to PS SRVCC access transfer is initiated. The call is initiated towards the URI of the STI-rSR as received during registration.

2)	UE sends an INVITE request to the SS.

3)	SS responds to the INVITE request with a 100 Trying response.

4)	SS responds to the INVITE request with a 183 Session Progress response.

5)	SS waits for the UE to send a PRACK request.

6)	SS responds to the PRACK request with a 200 OK.

7)	UE waits for the SS to send an INFO request.

8)	UE responds to the INFO request with a 200 OK.

9)	SS waits for the UE to send an INFO request.

10)	SS responds to the INVITE request with a 200 OK.

11)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for INVITE.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | UE attempt an IMS speech call a result of an initiation of the rSRVCC procedure |  |
| 2 |  |  | INVITE | UE sends INVITE with the first SDP offer. |
| 3 |  |  | 100 Trying | SS sends a 100 Trying provisional response. |
| 4 |  |  | 183 Session Progress | SS sends an 183 Session Progress |
| 5 |  |  | PRACK | UE acknowledges the 183 Session PRogress |
| 6 |  |  | 200 OK | SS sends a 200 OK |
| 7 |  |  | INFO | SS sends INFO request for transfer of incoming early session |
| 8 |  |  | 200 OK | UE responds INFO with 200 OK. |
| 8A |  |  |  | Make UE accept the speech call. |
| 9 |  |  | INFO | UE sends INFO request to confirm the call |
| 9A |  |  | 200 OK | SS responds INFO with 200 OK. |
| 10 |  |  | 200 OK | SS responds INVITE with 200 OK. |
| 11 |  |  | ACK | UE acknowledges. |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE for MO Call” in annex A.2.1 with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Request-Line |  |
| Request-URI | sip:sti-sr@atcf.visited2.net NOTE: This value was received by the UE in the preceding registration procedure |
| To |  |
| addr-spec | sip:sti-sr@atcf.visited2.net NOTE: This value was received by the UE in the preceding registration procedure |
| Message-body | The following SDP types and values. Session description: - v=0 - o=-(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) [Note 7] - b=RR: (bandwidth-value) [Note 7] Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 8] - a=fmtp: (format) mode-change-capability=2; max-red= (att-field) [Note 9] - a=rtpmap: (payload type) telephone-event/16000 - a=fmtp: (format) - a=ecn-capable-rtp: leap ect=0 [Note 3] - a=rtcp-fb:* nack ecn [Note 3] - a=rtcp-xr:ecn-sum [Note 3] - a=rtcp-rsize [Note 3] - a=ptime:20 - a=maxptime:240 Attributes for media security mechanism: - a=3ge2ae: requested [Note 4] - a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:WVNfX19zZW1jdGwgKCkgewkyMjA7fQp9CnVubGVz¦2^20¦ 1:4FEC_ORDER=FEC_SRTP" [Note 4] Note 1: At least one "c=" field shall be present. Note 2: Void. Note 3: Attributes for ECN Capability may be present if the UE supports Explicit Congestion Notification. Note 4: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 5: Void Note 6: Void Note 7: The RR value must be greater than 0. The RS value can be any value. Note 8: The AMR channel number shall be “/1” or omitted. Note 9: values from 0 to 220 are allowed |

183 Session Progress (Step 4)

Use the default message “183 Session Progress for INVITE” in annex A.2.3 with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Recv-Info |  |
| Info-package-type | g.3gpp.state-and-event (cf. RFC 6086 [139] for Recv-Info header) |
| Feature-Caps |  |
| feature-param | +g.3gpp.ti= (value) Note: The value of this parameter shall be the same as the Transaction Identifier sent by the SS in the preceding CS call setup. |
| Message-body | The following SDP types and values. Session description: - v=0 - o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:37 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1, 4] - b=AS:37 - b=RS: (bandwidth-value) [Note 5] - b=RR: (bandwidth-value) [Note 5] Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000/1 [Note 1] - a=fmtp: (format) mode-change-capability=2; max-red=220 [Note 1] - a=ecn-capable-rtp: leap ect=0 [Note 2] - a=rtcp-fb:* nack ecn [Note 2] - a=rtcp-xr:ecn-sum [Note 2] - a=ptime:20 - a=maxptime:240 Attributes for media security mechanism: - a=3ge2ae: requested [Note 1] - a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:PS1uQCVeeCFCanVmcjkpPywjNWhcYD0mXXtxaVBR¦2^20¦1:4 [Note 3] Note 1: The value for fmt, payload type (AMR) and format is copied from step 2. Note 2: Attributes for ECN Capability are present if the UE supports Explicit Congestion Notification. Note 3: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 4: transport port is the port number of the SS (see RFC 3264 clause 6). Note 5: The bandwidth-value is copied from step 2. |

PRACK (Step 5)

Use the default message “PRACK” in annex A.2.4

INFO (Step 7)

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Method | INFO |
| Request-URI | UE’s contact address in SIP URI form, as provided in the Contact header within the INVITE creating the dialog |
| SIP-Version | SIP/2.0 |
| Via | order of the parameters in this header must be like in this table |
| via-parm1: |  |
| Sent-protocol | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |
| sent-by | IP address and protected server port of SS |
| via-branch | value starting with ‘z9hG4bK’ (NOTE 1) |
| via-parm2: |  |
| sent-protocol | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |
| sent-by | scscf.3gpp.org |
| via-branch | value starting with ‘z9hG4bK’ (NOTE 1) |
| From |  |
| addr-spec | sip:sti-sr@atcf.visited2.net |
| tag | tag value corresponding to the SIP URI in the From header |
| To |  |
| addr-spec | any IMPU within the set of IMPUs on ISIM |
| tag | tag value corresponding to the SIP URI in the To header |
| Call-ID |  |
| callid | same as value received in INVITE message |
| CSeq |  |
| value | value of CSeq sent by the UE within its previous request in the same dialog but increased by one |
| method | INFO |
| Contact |  |
| addr-spec | sip:sti-sr@atcf.visited2.net |
| Content-Type |  |
| media-type | application/vnd.3gpp.state-and-event-info+xml |
| Max-Forwards |  |
| value | non-zero value |
| Recv-Info |  |
| Info-package-type | g.3gpp.state-and-event (cf. RFC 6086 [139] for Recv-Info header) |
| Content-Length |  |
| value | length of message-body |
| Message-body | <?xml version="1.0" encoding="UTF-8"?> <state-and-event-info> <state-info>early</state-info> <direction>receiver</direction> </state-and-event-info> |

NOTE 1:	Branch parameter values sent by SS are different within a test case execution.

INFO (Step 9)

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Method | INFO |
| Request-URI | sip:sti-sr@atcf.visited2.net NOTE: This value was received by the UE in the preceeding registration procedure. |
| SIP-Version | SIP/2.0 |
| Via | order of the parameters in this header must be like in this table |
| via-parm1: |  |
| Sent-protocol | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |
| sent-by | IP address and protected server port of SS |
| via-branch | value starting with ‘z9hG4bK’ |
| via-parm2: |  |
| sent-protocol | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |
| sent-by | scscf.3gpp.org |
| via-branch | value starting with ‘z9hG4bK’ |
| From |  |
| addr-spec | SIP URI of the UE |
| tag | tag value corresponding to the SIP URI in the From header |
| To |  |
| addr-spec | sip:sti-sr@atcf.visited2.net NOTE: This value was received by the UE in the preceeding registration procedure. |
| tag | tag value corresponding to the SIP URI in the To header |
| Call-ID |  |
| callid | same as value received in INVITE message |
| CSeq |  |
| value | value of CSeq sent by the SS within its previous request in the same dialog but increased by one |
| method | INFO |
| Contact |  |
| addr-spec | SIP URI of UE |
| Content-Type |  |
| media-type | application/vnd.3gpp.state-and-event-info+xml |
| Max-Forwards |  |
| value | non-zero value |
| Recv-Info |  |
| Info-package-type | g.3gpp.state-and-event (cf. RFC 6086 [139] for Recv-Info header) |
| Content-Length |  |
| value | length of message-body |
| Message-body | <?xml version="1.0" encoding="UTF-8"?> <state-and-event-info> <direction> initiator</direction> < event>call-accepted</event> </state-and-event-info> |

NOTE 1:	Branch parameter values sent by SS are different within a test case execution.

<End of modification>