3GPP TSG-RAN5 Meeting #109	R5-255796

# Dallas, Texas, USA, 17th - 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-1 | CR | 0019 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections in 37.579-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - At RAN5#108 R5-254019 has changed the names of px_MCX_InitialRegistration_TypeOfPDN1/2/3 - Table 5.4.4.3.4-1 refers to the wrong step of Table 5.4.4.3.3-1 - Further editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PIXIT names changed according to 37.579-5 - Step reference corrected for Table 5.4.4.3.4-1 - Further editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1A, 5.4.1B, 5.4.4.3, 5.5.3.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

### 5.4.1A	UE APN/PDN support assumptions for E-UTRA/EPC

According to TS 23.280 [110] clause 5.2.7.0 an MC service UE shall use APNs for the SIP-1, HTTP-1 and CSC-1 reference points, which may be different or all the same. To limit the test specification complexity it is assumed that only one APN is used and therefore there is a single MCX PDN. In addition there might be an IMS PDN and an internet PDN so that three PDNs need to be taken into account:

1.	MCX PDN with default EPS bearer using QCI=69

NOTE 1:	It should be noted that the core specs impose a requirement that the QCI value 8 or better shall be used for the EPS bearer that transports HTTP-1 reference point messaging. Using a single APN and having for the EPS bearer QCI=69 will satisfy this.

NOTE 2:	Void.

2.	Internet PDN with default EPS bearer using QCI=9

3.	IMS PDN with default EPS bearer using QCI=5

This results in the need to handle up to three PDNs during MCX conformance tests.

NOTE 3:	It should be noted that, handling IMS and MCX with one APN is theoretically possible but may have undesirable implications e.g. VoLTE signalling could delay MCX signalling therefore the assumption is that such implementations will be undesirable and unlikely.

Consequently, for IMS and MCX it should be assumed that the UE will do 2 different registrations, i.e. for each of them there will be a separate IP connection (different IP addresses at the UE and the SS).

Depending on UE configuration PDN connectivities for the up-to three PDNs may be established. There are two major scenarios:

1.	The MCX PDN connectivity gets established automatically after switch-on during the initial registration procedure. In addition the UE may establish PDN connectivities to the IMS PDN and/or the internet PDN. The connectivity to these PDNs may be requested in any order. There can be 1, 2 or 3 PDNs.

2.	The UE requests PDN connectivities for IMS and/or internet but not for MCX. If IMS and internet are requested, it may be in any order. Establishment of the MCX PDN connectivity is triggered after the initial registration in a separate procedure. There can be 2 or 3 PDNs in total.

To serve the above scenarios the following parameters are defined in TS 37.579-5 [5]:

-	px_MCX_InitialRegistration_TypeOf1stPDN: 
First PDN registered during initial registration (either 'ims' or 'internet' or 'mcx')

-	px_MCX_InitialRegistration_TypeOf2ndPDN: 
Second PDN registered during initial registration; in addition to 'ims' or 'internet' or 'mcx' it may be 'none' to indicate that there is no second PDN connectivity requested by the UE during initial registration.

-	px_MCX_InitialRegistration_TypeOf3rdPDN: 
Third PDN registered during initial registration; in addition to 'ims' or 'internet' or 'mcx' it may be 'none' to indicate that there is no third PDN connectivity requested by the UE during initial registration.

The type of the parameters is a TTCN-3 enumerated type with values 'ims','internet', 'mcx' and 'none'.

In addition there is the parameter px_AccessPointName in TS 36.523-3 [74] which is used as default APN, i.e. for a PDN for which the UE does not provide an APN (NOTE: Any, but only one, of the three PDNs can be the one with default APN).

Regarding the default EPS bearers for the respective mission critical services the following applies for MCX conformance tests:

-	MCPTT: 
A single dedicated EPS bearer with QCI=65 is used with packet filters for the audio stream and media plane control signalling (see also TS 23.379 [126] clause 5.7.3)

-	MCVideo: 
A single dedicated EPS bearer with QCI=67 is used with packet filters for the audio and video streams and transmission control signalling (see also TS 23.281 [90] clause 5.5.3)

-	MCData:
A single dedicated EPS bearer with QCI=70 is used with packet filter for the TCP data stream (see also TS 23.282 [91] clause 5.8.3)

<End of modification>

<Start of modification>

### 5.4.1B	UE PDU session support assumptions for NR/5GC

Similar assumptions are made for the PDU sessions of NR/5GC as for the PDNs of E-UTRA/EPC in clause 5.4.1A. At initial registration the UE may request up to three PDU sessions:

1.	MCX PDU session with default QoS flow using 5QI=69 according to Table 5.7.4-1 in TS 23.501 [134]

2.	Internet PDU session with default QoS flow using 5QI=9 (same value as for Config #1 of table 4.8.4-1 in TS 38.508-1 [132])

3.	IMS PDU session with default QoS flow using 5QI=5 (same value as for Config #2 of table 4.8.4-1 in TS 38.508-1 [132])

At initial registration - before the UE goes back to idle mode - the UE may request - in any order - 0, 1, 2 or 3 of these PDU sessions. This shall be configured (re-)using px_MCX_InitialRegistration_TypeOf1stPDN, px_MCX_InitialRegistration_TypeOf2ndPDN and px_MCX_InitialRegistration_TypeOf3rdPDN as described in clause 5.4.1A. When none of the PIXITs addresses the MCX PDU session, establishment of the MCX PDU session needs to be triggered by the user.

The type of each PDU session shall be determined using pc_APN_ID_Internet, pc_APN_ID_IMS and pc_APN_ID_MCX as specified in TS 38.508-2 [133].

The non-default QoS flows for MCX over NR/5GC are

-	MCPTT: 
Single non-default QoS flows with 5QI=65 according to Table 5.7.4-1 in TS 23.501 [134] with packet filters for the audio stream and media plane control signalling.

-	MCVideo: 
Single non-default QoS flows with 5QI=67 according to Table 5.7.4-1 in TS 23.501 [134] with packet filters for the audio and video streams and transmission control signalling.

-	MCData:
Single non-default QoS flows with 5QI=70 according to Table 5.7.4-1 in TS 23.501 [134] with packet filter for the TCP data stream.

<End of modification>

<Start of modification>

#### 5.4.4.3	NR/5GC signalling

5.4.4.3.1	Initial conditions

As specified in clause 5.4.4.1.1 with the following clarifications:

- An MCX PDU session with default QoS rule #10 (5QI 69) according to table 4.8.2.1-10 in TS 38.508-1 [132] is established for MCX and SIP signalling.

NOTE 1:	The assumptions for the PDU session support, including the default QoS flow 5QI requirements in regard to the different PDU session are described in 5.4.1B.

5.4.4.3.2	Definition of system information messages

The NR/5GC default system information messages as defined in TS 38.508-1 [132] are used.

5.4.4.3.3	Procedure

Table 5.4.4.3.3-1: NR/5GC signalling for MCX CT communication

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits a Paging message. | <-- | NR RRC: Paging |
| 2-8 | Steps 2-8 of the RRC establishment procedure in TS 38.508-1 [132] Table 4.5.4.2-3 take place. | - | - |
| - | EXCEPTION: Steps 9a1-9a3 describe behaviour that depends on the context in which the procedure is used: The steps take place when the procedure is used for MCPTT or MCVideo call establishment and MCData communication establishment for using the media plane. | - | - |
| 9a1 | The SS configures a new RLC-UM data radio bearer, associated with the QoS flow to be added to the MCX PDU session and transmits an RRCReconfiguration message containing a PDU SESSION MODIFICATION COMMAND accordingly. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND |
| - | EXCEPTION: Depending upon UE implementation steps 8a2 and 8a3 can occur in any order. | - | - |
| 9a2 | The UE transmits an RRCReconfigurationComplete message to confirm the establishment of the data radio bearer associated with the added QoS flow. | --> | NR RRC: RRCReconfigurationComplete |
| 9a3 | The UE transmits a PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE |

5.4.4.3.4	Specific message contents

All specific NR/5GC signalling message contents shall be referred to TS 38.508-1 [132] clause 4.6 and 4.7 7 with the following clarifications:

-	The SERVICE REQUEST message at step 4 of Table 5.4.4.3.3-1 is expected to have the service type IE set to "mobile terminated services".

-	The RRCReconfiguration message at step 9a1 adds a single RLC-UM non-default DRB (configuration DRB(0,1)) in RRC messages and information elements of TS 38.508-1 [132]).

-	PDU SESSION MODIFICATION COMMAND message at step 9a1 adds a non-default QoS flow to the MCX PDU session as shown in Table 5.4.4.3.4-1.

Table 5.4.4.3.4-1: PDU SESSION MODIFICATION COMMAND (step 9a1, Table 5.4.4.3.3-1)

| Derivation Path: TS 38.508-1 [132] Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as in PDU SESSION ESTABLISHMENT REQUEST message at establishment of the MCX PDU session |  |  |
| Authorized QoS rules |  | Reference QoS rule #11 as defined in TS 38.508-1 [132] Table 4.8.2.1-7 using condition MCPTT, MCVIDEO or MCDATA (NOTE 1) |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #10 as defined in TS 38.508-1 [132] Table 4.8.2.3-5 using condition MCPTT, MCVIDEO or MCDATA (NOTE 1) |  |  |
| NOTE 1: Depending on the MC service (condition MCPTT, MCVIDEO or MCDATA) the QoS flow uses MCPTT: QoS rule id=10, QFI=12 and 5QI = 65 MCVideo: QoS rule id=11, QFI=13 and 5QI = 67 MCData: QoS rule id=12, QFI=14 and 5QI = 70 |  |  |  |  |

<End of modification>

<Start of modification>

#### 5.5.3.16	MCS-Regroup

##### 5.5.3.16.1	Common conditions for MCS-Regroup

The following conditions apply throughout clause 5.5.3.16:

Table 5.5.3.16.1-1: Conditions

| Condition | Explanation |
| --- | --- |
| GROUP_REGROUP | Creating a group regroup using preconfigured group |
| USER_REGROUP | Creating a user regroup using preconfigured group |
| REMOVE | Remove a user or group regroup using a preconfigured group |

##### 5.5.3.16.2	MCS-Regroup from the UE

##### -	MCPTT

Table 5.5.3.16.2-1: MCPTT-Regroup from the UE

| Derivation Path: TS 24.379 [9] clause F.7.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttregroup |  |  |  |  |
| mcpttregroup-Params |  |  |  |  |
| preconfig-group-id | Encrypted (NOTE 1) |  |  |  |
| preconfigured-group | not present |  |  |  |
|  | px_MCPTT_Group_A_ID | The URI of a group to be used as the configuration of the group regroup |  | GROUP_REGROUP, USER_REGROUP |
| mcptt-regroup-uri | Encrypted (NOTE 2) |  |  |  |
| mcptt-regroup-uri | px_MCPTT_Group_T_ID | The URI of the group regroup |  |  |
| groups-for-regroup | not present |  |  |  |
| groups-for-regroup | Encrypted (NOTE 3) |  |  | GROUP_REGROUP |
| group [1] | px_MCPTT_Group_A_ID | The URI of a group to regroup |  |  |
| group [2] | px_MCPTT_Group_B_ID | The URI of a group to regroup |  |  |
| users-for-regroup | not present |  |  |  |
| users-for-regroup | Encrypted (NOTE 4) |  |  | USER_REGROUP |
| user [1] | px_MCPTT_ID_User_A |  |  |  |
| user [2] | px_MCPTT_ID_User_B |  |  |  |
| user [3] | px_MCPTT_ID_User_D |  |  |  |
| regroup-action | "create" |  |  |  |
|  | “remove” |  |  | REMOVE |
| NOTE 1: Element content encryption either of element <preconfig-group-id> or of its sub-element <preconfigured-group> (if present) as described in Table 5.5.13.2-1 NOTE 2: Element content encryption either of element <mcptt-regroup-uri> or of its sub-element <mcptt-regroup-uri> as described in Table 5.5.13.2-1 NOTE 3: Element content encryption either of element <groups-for-regroup> or of each of its sub-elements <group> as described in Table 5.5.13.2-1 NOTE 4: Element content encryption either of element <users-for-regroup> or of each of its sub-elements <user> as described in Table 5.5.13.2-1 |  |  |  |  |

##### -	MCVideo

Table 5.5.3.16.2-2: MCVideo-Regroup from the UE

| Derivation Path: TS 24.281 [86] clause F.6.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoregroup |  |  |  |  |
| mcvideoregroup-Params |  |  |  |  |
| preconfig-group-id | Encrypted (NOTE 1) |  |  |  |
| preconfigured-group | not present |  |  |  |
|  | px_MCVideo_Group_A_ID | The URI of a group to be used as the configuration of the group regroup |  | GROUP_REGROUP, USER_REGROUP |
| mcvideo-regroup-uri | Encrypted (NOTE 2) |  |  |  |
| mcvideo-regroup-uri | px_MCVideo_Group_T_ID | The URI of the group regroup |  |  |
| groups-for-regroup | not present |  |  |  |
| groups-for-regroup | Encrypted (NOTE 3) |  |  | GROUP_REGROUP |
| group [1] | px_MCVideo_Group_A_ID | The URI of a group to regroup |  |  |
| group [2] | px_MCVideo_Group_B_ID | The URI of a group to regroup |  |  |
| users-for-regroup | not present |  |  |  |
| users-for-regroup | Encrypted (NOTE 4) |  |  | USER_REGROUP |
| user [1] | px_MCVideo_ID_User_A |  |  |  |
| user [2] | px_MCVideo_ID_User_B |  |  |  |
| user [3] | px_MCVideo_ID_User_D |  |  |  |
| regroup-action | "create" |  |  |  |
|  | “remove” |  |  | REMOVE |
| NOTE 1: Element content encryption either of element <preconfig-group-id> or of its sub-element <preconfigured-group> (if present) as described in Table 5.5.13.2-1 NOTE 2: Element content encryption either of element <mcvideo-regroup-uri> or of its sub-element <mcvideo-regroup-uri> as described in Table 5.5.13.2-1 NOTE 3: Element content encryption either of element <groups-for-regroup> or of each of its sub-elements <group> as described in Table 5.5.13.2-1 NOTE 4: Element content encryption either of element <users-for-regroup> or of each of its sub-elements <user> as described in Table 5.5.13.2-1 |  |  |  |  |

##### 5.5.3.16.3	MCS-Regroup from the SS

##### -	MCPTT

Table 5.5.3.16.3-1: MCPTT-Regroup from the SS

| Derivation Path: TS 24.379 [9] clause F.7.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttregroup |  |  |  |  |
| mcpttregroup-Params |  |  |  |  |
| preconfig-group-id |  |  |  |  |
| preconfig-group-id | Encrypted (NOTE 1) |  |  |  |
| preconfigured-group | not present |  |  |  |
|  | px_MCPTT_Group_A_ID | The URI of a group to be used as the configuration of the group regroup |  | GROUP_REGROUP, USER_REGROUP |
| mcptt-regroup-uri | Encrypted (NOTE 2) |  |  |  |
| mcptt-regroup-uri | px_MCPTT_Group_T_ID | The URI of the group regroup |  |  |
| groups-for-regroup | not present |  |  |  |
| groups-for-regroup | Encrypted (NOTE 3) |  |  | GROUP_REGROUP |
| group [1] | px_MCPTT_Group_A_ID | The URI of a group to regroup |  |  |
| group [2] | px_MCPTT_Group_B_ID | The URI of a group to regroup |  |  |
| users-for-regroup | not present |  |  |  |
| users-for-regroup | Encrypted (NOTE 4) |  |  | USER_REGROUP |
| user [1] | px_MCPTT_ID_User_A |  |  |  |
| user [2] | px_MCPTT_ID_User_B |  |  |  |
| user [3] | px_MCPTT_ID_User_D |  |  |  |
| regroup-action | "create" |  |  |  |
|  | “remove” |  |  | REMOVE |
| NOTE 1: Element content encryption of element <preconfig-group-id>’s sub-element <preconfigured-group> (if present) as described in Table 5.5.13.2-2 NOTE 2: Element content encryption of element <mcptt-regroup-uri>’s sub-element <mcptt-regroup-uri> as described in Table 5.5.13.2-2 NOTE 3: Element content encryption of each of element <groups-for-regroup>’s sub-elements <group> as described in Table 5.5.13.2-2 NOTE 4: Element content encryption of each of element <users-for-regroup>’s sub-elements <user> as described in Table 5.5.13.2-2 |  |  |  |  |

##### -	MCVideo

Table 5.5.3.16.3-2: MCVideo-Regroup from the SS

| Derivation Path: TS 24.281 [86] clause F.6.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoregroup |  |  |  |  |
| mcvideoregroup-Params |  |  |  |  |
| preconfig-group-id |  |  |  |  |
| preconfig-group-id | Encrypted (NOTE 1) |  |  |  |
| preconfigured-group | not present |  |  |  |
|  | px_MCVideo_Group_A_ID | The URI of a group to be used as the configuration of the group regroup |  | GROUP_REGROUP, USER_REGROUP |
| mcvideo-regroup-uri | Encrypted (NOTE 2) |  |  |  |
| mcvideo-regroup-uri | px_MCVideo_Group_T_ID | The URI of the group regroup |  |  |
| groups-for-regroup | not present |  |  |  |
| groups-for-regroup | Encrypted (NOTE 3) |  |  | GROUP_REGROUP |
| group [1] | px_MCVideo_Group_A_ID | The URI of a group to regroup |  |  |
| group [2] | px_MCVideo_Group_B_ID | The URI of a group to regroup |  |  |
| users-for-regroup | not present |  |  |  |
| users-for-regroup | Encrypted (NOTE 4) |  |  | USER_REGROUP |
| user [1] | px_MCVideo_ID_User_A |  |  |  |
| user [2] | px_MCVideo_ID_User_B |  |  |  |
| user [3] | px_MCVideo_ID_User_D |  |  |  |
| regroup-action | "create" |  |  |  |
|  | “remove” |  |  | REMOVE |
| NOTE 1: Element content encryption of element <preconfig-group-id>’s sub-element <preconfigured-group> (if present) as described in Table 5.5.13.2-2 NOTE 2: Element content encryption of element <mcvideo-regroup-uri>’s sub-element <mcvideo-regroup-uri> as described in Table 5.5.13.2-2 NOTE 3: Element content encryption of each of element <groups-for-regroup>’s sub-elements <group> as described in Table 5.5.13.2-2 NOTE 4: Element content encryption of each of element <users-for-regroup>’s sub-elements <user> as described in Table 5.5.13.2-2 |  |  |  |  |

##### -	MCData

Table 5.5.3.16.3-3: MCData-Regroup from the SS

| Derivation Path: TS 24.282 [87] clause D.6.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdataregroup |  |  |  |  |
| mcdataregroup-Params |  |  |  |  |
| preconfig-group-id |  |  |  |  |
| preconfig-group-id | Encrypted (NOTE 1) |  |  |  |
| preconfigured-group | not present |  |  |  |
|  | px_MCData_Group_A_ID | The URI of a group to be used as the configuration of the group regroup |  | GROUP_REGROUP, USER_REGROUP |
| mcdata-regroup-uri | Encrypted (NOTE 2) |  |  |  |
| mcdata-regroup-uri | px_MCData_Group_T_ID | The URI of the group regroup |  |  |
| groups-for-regroup | not present |  |  |  |
| groups-for-regroup | Encrypted (NOTE 3) |  |  | GROUP_REGROUP |
| group [1] | px_MCData_Group_A_ID | The URI of a group to regroup |  |  |
| group [2] | px_MCData_Group_B_ID | The URI of a group to regroup |  |  |
| users-for-regroup | not present |  |  |  |
| users-for-regroup | Encrypted (NOTE 4) |  |  | USER_REGROUP |
| user [1] | px_MCData_ID_User_A |  |  |  |
| user [2] | px_MCData_ID_User_B |  |  |  |
| user [3] | px_MCData_ID_User_D |  |  |  |
| regroup-action | "create" |  |  |  |
|  | “remove” |  |  | REMOVE |
| NOTE 1: Element content encryption of element <preconfig-group-id>’s sub-element <preconfigured-group> (if present) as described in Table 5.5.13.2-2 NOTE 2: Element content encryption of element <mcdata-regroup-uri>’s sub-element <mcdata-regroup-uri> as described in Table 5.5.13.2-2 NOTE 3: Element content encryption of each of element <groups-for-regroup>’s sub-elements <group> as described in Table 5.5.13.2-2 NOTE 4: Element content encryption of each of element <users-for-regroup>’s sub-elements <user> as described in Table 5.5.13.2-2 |  |  |  |  |

<End of modification>