3GPP TSG-RAN5 Meeting #109	R5-255980

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-1 | CR | 0022 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to domain name for MCPTT confidentiality protection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 37.579-2 Table 5.5.13.3-1 specifies the domain name for MCPTT confidentiality protection as “mc1-encryption.3gppnetwork.org”, and refers to TS 24.379, cl. 6.6.2.3.4 and TS 23.003, cl. 26.2. TS 24.379 cl 6.6.2.3.4 states that “the string "@" followed by the domain name for MCPTT confidentiality protection is as specified in 3GPP TS 23.203” As per TS 23.003 cl. 26.2: A Domain Name for MC Services confidentiality protection used in a host part of a SIP URI indicates that the user part of the SIP URI contains a confidentiality protected MC Services identity. This Domain Name shall be the string "mc1-encrypted.3gppnetwork.org". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the the domain name for MCPTT confidentiality protection as “mc1-encrypted.3gppnetwork.org” in Table 5.5.13.3-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Associated with TTCN CR R5s250419 |  |  |  |  |  |  |  |  |

### 5.5.13	Default XML messages and elements for XML security

#### 5.5.13.1	XML signature for integrity protection of MIME bodies

Table 5.5.13.1-1: XML signature MIME body from the UE

| Derivation Path: TS 24.379 [9] annex F.6.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Signatures |  | list of N signatures for the signed XML bodies of a SIP message |  |  |
| Signature [n] |  | n {1..N} |  |  |
| id | any value if present |  |  |  |
| SignedInfo |  |  |  |  |
| CanonicalizationAlgorithm | any value | canonicalisation method e.g. "http://www.w3.org/TR/2001/REC-xml-c14n-20010315" |  |  |
| SignatureAlgorithm | "HMAC-SHA-256" | Hashing algorithm to be applied to sign the SignedInfo with the key given in the KeyInfo |  |  |
| Reference |  |  |  |  |
| URI | same value as the Content-ID of the XML MIME body the signature belongs to |  |  |  |
| DigestAlgorithm | "SHA-256" | Hashing algorithm to be applied to sign the data object |  |  |
| DigestValue | Hash signing the data object (referred to by the URI) |  |  |  |
| SignatureValue | Hash signing the SignedInfo | The signing key is derived from the CSK according to TS 33.180 [94] Annex F.1.4 with FC = 0x52 XPK-ID = CSK-ID |  |  |
| KeyInfo |  |  |  |  |
| KeyName | base64 encoded CSK-ID |  |  |  |

Table 5.5.13.1-2: XML signature MIME body from the SS

| Derivation Path: TS 24.379 [9] annex F.6.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Signatures |  | list of N signatures for the signed XML bodies of a SIP message |  |  |
| Signature [n] |  | n {1..N} |  |  |
| id | "signature" & n |  |  |  |
| SignedInfo |  |  |  |  |
| CanonicalizationAlgorithm | "http://www.w3.org/TR/2001/REC-xml-c14n-20010315" | canonicalisation method |  |  |
| SignatureAlgorithm | "HMAC-SHA-256" | Hashing algorithm to be applied to sign the SignedInfo with the key given in the KeyInfo |  |  |
| Reference |  |  |  |  |
| URI | same value as the Content-ID of the XML MIME body the signature belongs to |  |  |  |
| DigestAlgorithm | "SHA-256" | Hashing algorithm to be applied to sign the data object |  |  |
| DigestValue | Hash signing the data object (referred to by the URI) |  |  |  |
| SignatureValue | Hash signing the SignedInfo | The signing key is derived from the CSK according to TS 33.180 [94] Annex F.1.4 with FC = 0x52 XPK-ID = CSK-ID |  |  |
| KeyInfo |  |  |  |  |
| KeyName | base64 encoded CSK-ID |  |  |  |

#### 5.5.13.2	XML <EncryptedData> element for encryption of XML element content

Table 5.5.13.2-1: XML <EncryptedData> element from the UE

| Derivation Path: XML Encryption Syntax, Version 1.1 [108] clause 9.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| EncryptedData |  |  |  |  |
| Type attribute | "http://www.w3.org/2001/04/xmlenc#Content" if present |  |  |  |
| EncryptionMethod | if present |  |  |  |
| Algorithm attribute | "http://www.w3.org/2009/xmlenc11#aes128-gcm" |  |  |  |
| KeyInfo | if present |  |  |  |
| KeyName | base64 encoded CSK-ID | The CSK-ID is provided by the UE at CSK distribution |  |  |
| CipherData |  |  |  |  |
| CipherValue | encrypted XML element content | The encryption key is derived from the CSK according to TS 33.180 [94] Annex F.1.4 with FC = 0x51 XPK-ID = CSK-ID | TS 33.180 [94] clause 9.3.4.2 |  |

Table 5.5.13.2-2: XML <EncryptedData> element from the SS

| Derivation Path: XML Encryption Syntax, Version 1.1 [108] clause 9.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| EncryptedData |  |  |  |  |
| Type attribute | "http://www.w3.org/2001/04/xmlenc#Content" |  |  |  |
| EncryptionMethod |  |  |  |  |
| Algorithm attribute | "http://www.w3.org/2009/xmlenc11#aes128-gcm" |  |  |  |
| KeyInfo |  |  |  |  |
| KeyName | base64 encoded CSK-ID | The CSK-ID is provided by the UE at CSK distribution |  |  |
| CipherData |  |  |  |  |
| CipherValue | encrypted XML element content | The encryption key is derived from the CSK according to TS 33.180 [94] Annex F.1.4 with FC = 0x51 XPK-ID = CSK-ID | TS 33.180 [94] clause 9.3.4.2 |  |

#### 5.5.13.3	Encrypted XML URI attribute

Table 5.5.13.3-1: Encrypted XML URI attribute

| Delivery Path: RFC 3261 [22] clause 19.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| SIP URI |  |  |  |  |
| scheme | "sip" |  |  |  |
| user | semicolon separated list of: |  | TS 24.379 [9] clause 6.6.2.3.4 |  |
|  | base64 encoded encrypted URI | The encryption key is derived from the CSK according to TS 33.180 [94] Annex F.1.4 with FC = 0x51 XPK-ID = CSK-ID |  |  |
|  | "iv=" & base64 encoded 96-bit random initialisation vector (IV) | IV as used by AES-128 encryption algorithm |  |  |
|  | "key-id=" & base64 encoded encryption key identifier (XPK-ID) | with XPK-ID = CSK-ID |  |  |
|  | "alg=128-aes-gcm" | AES-128 encryption algorithm |  |  |
| password | not present |  |  |  |
| host | "mc1-encrypted.3gppnetwork.org" |  | TS 24.379 [9] clause 6.6.2.3.4; TS 23.003 [69] clause 26.2 |  |
| port | not present |  |  |  |
| uri parameters | not present |  |  |  |
| headers | not present |  |  |  |