3GPP TSG-RAN5 Meeting #109	R5-255788

# Dallas, USA, 17th November 2025 – 21th November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5491 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NB-IoT UP PUR test case 22.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF 160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In theTable 22.1.5.3.3-3 the dl-CarrierFreq-r13 IE is defined incorrectly because it is defined in ASN.1 as a sequence. - Definition of a value for ul-CarrierFreq-r13 is not strictly necessary, if absent, the same TX-RX frequency separation and carrier frequency offset as for the anchor carrier applies according to TS 36.331. - The resumeIdentity-r13 IE is defined 3 times in the Table 22.1.6.3.3-3. The entries have to be merged into one. - The resumeID-r13 in the Table 22.1.6.3.3-5 is defined 3 times. The etries have to be merged into one. - Two pur-ConfigID-r16 entries in the Table 22.1.6.3.3-6 have to be merged into one. - Table 22.1.6.3.3-2, Table 22.1.6.3.3-5 and Table 22.1.6.3.3-6 should refer to the Table 22.1.6.3.2-1. - The contents of the Table 22.1.6.3.3-4 do not defer from the default ones in the Table 8.1.6.1-2a in TS 36.508 and this makes the table redundant. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected definition of dl-CarrierFreq-r13 IE. - Set the value of ul-CarrierFreq-r13 to “Not present”. - Deleted redundant resumeIdentity-r13 entries and merged rows in the Table 22.1.6.3.3-3. - Deleted redundant resumeID-r13 entries and merged rows in the Table 22.1.6.3.3-5. - Deleted redundant pur-ConfigID-r16 entry and merged rows in the Table 22.1.6.3.3-6. - Updated references to the Table 22.1.6.3.2-1 in the Table 22.1.6.3.3-2, Table 22.1.6.3.3-5 and the Table 22.1.6.3.3-6. - Voided the Table 22.1.6.3.3-4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TC prose will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 22.1.6	NB-IoT / NTN / User Plane CIoT EPS Optimization / PUR

##### 22.1.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE with a suspended RRC connection state with pur-Config }

ensure that {

when { UE has data to be sent in the UL using PUR}

then { UE sends RRCResumeRequest-NB using the PUR uplink grant }

}

(2)

with { UE in E-UTRA RRC_IDLE state and pur-ResponseWindowTimer running and having previously sent an RRCResumeRequest-NB message using PUR}

ensure that {

when { UE receives RRCConnectionSetup-NB }

then { UE responds with RRCConnectionSetupComplete-NB without pur-ConfigID }

}

(3)

with { UE in E-UTRA RRC_IDLE with a suspended RRC connection state with pur-Config available and having sent RRCConnectionResumeRequest-NB without using PUR}

ensure that {

when { The SS sends RRCConnectionSetup-NB}

then { UE replies with RRCConnectionSetupComplete-NB including pur-ConfigID }

}

(4)

with { UE in E-UTRA RRC_IDLE with a suspended RRC connection state and PUR-timeAlignmentTimer has expired }

ensure that {

when { The UE has UL data to send}

then { UE sends RRCConnectionResumeRequest-NB including the resumeIdentity and AS security context stored and no data on DTCH and without using the PUR uplink grant }

}

##### 22.1.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.300[10] clause 7.3d.4, 3GPP TS 36.321[14] clauses 5.4.7.1 and 5.4.7.2 and TS 36.331[17] clause s 5.3.1.1, 5.3.3.1, 5.3.3.1c, 5.3.3.4 and 5.6.23.2. Unless otherwise stated these are Rel-17 requirements

[36.300[10], 7.3d.4]

Figure 7.3d-3: Transmission using PUR for the User Plane CIoT EPS Optimisation

0.	The UE has validated the PUR resource according to the configured criteria.

1	Same as step 1 in MO-EDT for User Plane CIoT EPS/5GS optimisations in Figure 7.3b-2 and 7.3b-2a except that the UE transmits over the PUR resource instead of a resource allocated in the random access response.

If the user data are too large to be fully included in the transmission using PUR, the UE can use PUR to transmit RRCConnectionResumeRequest and a segment of the user data. The procedure will fall back to the legacy RRC Connection Resume procedure; a new C-RNTI can be assigned.

After step 1, the (ng-)eNB may request the UE to abort the transmission using PUR by sending a Layer 1 fallback indication. UE actions upon reception of Layer 1 fallback indication are left up to UE implementation.

2..7 Same as MO-EDT for User Plane CIoT EPS/5GS optimisations in Figure 7.3b-2 and 7.3b-2a.

8	Same as step 8 in MO-EDT for user Plane CIoT EPS/5GS optimisations in Figure 7.3b-2 and 7.3b-2a except that a Time Advance Command can also be included.

NOTE 1:	If the MME/AMF or the (ng-)eNB decides to move the UE to RRC_CONNECTED mode, RRCConnectionResume message is sent in step 8 to fall back to the RRC Connection resume procedure. In that case, the RRCConnectionResume message is integrity protected and ciphered with the keys derived in step 1 and the UE ignores the NextHopChainingCount included in the RRCConnectionResume message; a new C-RNTI can be assigned. Downlink data can be transmitted on DTCH multiplexed with the RRCConnectionResume message. In addition, an RRCConnectionSetup can also be sent in step 8 to fall back to the RRC Connection establishment procedure.

NOTE 2:	If neither RRCConnectionRelease nor, in case of fallback, RRCConnectionResume is received in response to RRCConnectionResumeRequest using PUR, the UE considers the UL data transmission not successful.

[TS 36.321[14], clause 5.4.7.1]

Transmission using PUR is initiated by the RRC layer. When transmission using PUR is initiated, RRC layer provides MAC with the following information:

-	PUR-RNTI;

-	Duration of PUR response window pur-ResponseWindowTimer;

-	UL grant information.

If the MAC entity has a PUR-RNTI, the MAC entity shall for each TTI for which RRC layer has provided uplink grant for transmission using PUR:

-	deliver the uplink grant, and the associated HARQ information to the HARQ entity for this TTI.

After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer:

-	if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer:

-	the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT.

-	else:

-	the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes.

While pur-ResponseWindowTimer is running, the MAC entity shall:

-	if the PDCCH transmission is addressed to the PUR-RNTI and contains an UL grant for a retransmission:

-	if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer:

-	restart pur-ResponseWindowTimer at the last subframe of a PUSCH transmission corresponding to the retransmission indicated by the UL grant plus 4 subframes plus UE-eNB RTT.

-	else:

-	restart pur-ResponseWindowTimer at the last subframe of a PUSCH transmission corresponding to the retransmission indicated by the UL grant plus 4 subframes.

-	if L1 ACK for transmission using PUR is received from lower layers; or

-	if PDCCH transmission is addressed to the PUR-RNTI and the MAC PDU is successfully decoded:

-	stop pur-ResponseWindowTimer;

-	if L1 ACK for transmission using PUR is received from lower layers or the MAC PDU contains only Timing Advance Command MAC control element:

-	indicate to upper layers the transmission using PUR was successful;

-	if repetition adjustment for transmission using PUR is received from lower layers:

-	indicate the value of the repetition adjustment to upper layers.

-	discard the PUR-RNTI.

-	else if fallback indication for PUR is received from lower layers:

-	stop pur-ResponseWindowTimer;

-	indicate to upper layers PUR fallback indication is received;

-	if repetition adjustment for transmission using PUR is received from lower layers:

-	indicate the value of the repetition adjustment to upper layers.

-	discard the PUR-RNTI.

-	if the pur-ResponseWindowTimer expires:

-	if PUR was transmitted in a non-terrestrial network:

-	if no notification of a reception of a PDCCH transmission addressed to the PUR-RNTI containing an UL grant for a retransmission was received after the start of pur-ResponseWindowTimer:

-	indicate to upper layers the transmission using PUR has failed;

-	discard the PUR-RNTI.

-	else:

-	indicate to upper layers the transmission using PUR has failed;

-	discard the PUR-RNTI.

[TS 36.321[14], clause 5.4.7.2]

MAC entity may be configured with timer pur-TimeAlignmentTimer by upper layers as specified in TS 36.331 [8], clause 5.3.8.3.

The MAC entity shall:

-	when pur-TimeAlignmentTimer configuration is received from upper layers:

-	start or restart pur-TimeAlignmentTimer.

-	when pur-TimeAlignmentTimer is released by upper layers:

-	stop the pur-TimeAlignmentTimer, if running.

-	when a Timing Advance Command MAC control element is received or PDCCH indicates timing advance adjustment as specified in TS 36.212 [5] and if a NTA has been stored or maintained:

-	if the Timing Advance Command MAC control element or PDCCH indicating timing advance adjustment is addressed with a PUR-RNTI:

-	apply the Timing Advance Command or the timing advance adjustment;

-	start or restart the pur-TimeAlignmentTimer, if configured;

-	indicate to upper layers that the Timing Advance value has been adjusted.

-	upon considering a Random Access procedure successfully completed:

-	start or restart the pur-TimeAlignmentTimer, if configured;

-	indicate to upper layers that the Timing Advance value has been adjusted;

-	if a temporary NTA has been stored, delete the stored temporary NTA.

-	upon considering a Random Access procedure unsuccessfully completed, if a temporary NTA has been stored:

-	set the NTA to the stored temporary NTA;

-	delete the stored temporary NTA.

Upon request from upper layers, MAC entity shall indicate whether pur-TimeAlignmentTimer is running.

[TS 36.331[17] clause 5.3.1.1]

In case of CP-EDT or CP transmission using PUR, the data are appended in the RRCEarlyDataRequest and RRCEarlyDataComplete messages, if available, and sent over SRB0. In case of UP-EDT or UP transmission using PUR, security is re-activated prior to transmission of RRC message using the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure and the radio bearers are re-established. The uplink data are transmitted ciphered on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH. In the downlink, the data, if available, are transmitted on DTCH multiplexed with the RRCConnectionRelease message on DCCH. In response to a request for EDT or transmission using PUR, E-UTRA/EPC or E-UTRA/5GC may also choose to establish or resume the RRC connection.

[TS 36.331[17], clause 5.3.3.1]

Figure 5.3.3.1-3: RRC connection resume (suspended RRC connection or RRC_INACTIVE), or UP-EDT fallback or fallback from UP transmission using PUR to RRC connection resume, successful

Figure 5.3.3.1-4: RRC connection resume (suspended RRC connection or RRC_INACTIVE) or UP-EDT fallback or fallback from UP transmission using PUR to RRC connection establishment, successful

Figure 5.3.3.1-5: RRC connection resume or UP-EDT or UP transmission using PUR, network reject (suspended RRC connection or RRC_INACTIVE) or release (suspended RRC connection)

Figure 5.3.3.1-6: RRC connection resume (RRC_INACTIVE), network release or suspend or UP-EDT or UP transmission using PUR, successful...

The purpose of this procedure is to establish an RRC connection, to resume a suspended RRC connection, to move the UE from RRC_INACTIVE to RRC_CONNECTED, to perform EDT or to perform transmission using PUR. RRC connection establishment involves SRB1 (and SRB1bis for NB-IoT) establishment. The procedure is also used to transfer the initial NAS dedicated information/ message from the UE to E-UTRAN.

E-UTRAN applies the procedure as follows:

-	When establishing an RRC connection:

-	to establish SRB1 and, for NB-IoT, SRB1bis;

-	When resuming an RRC connection from a suspended RRC connection or from RRC_INACTIVE:

-	to restore the AS configuration from a stored context including resuming SRB(s) and DRB(s);

-	When performing EDT;

-	When performing transmission using PUR.

[TS 36.331[17], clause 5.3.3.1c]

A BL UE, UE in CE or NB-IoT UE can initiate transmission using PUR when all of the following conditions are fulfilled:

1>	the UE has a valid PUR configuration for the serving cell as specified in 5.3.3.20;

1>	the UE has a valid timing alignment value as specified in 5.3.3.19;

1>	the upper layers request establishment of an RRC connection; or the upper layers request resumption of an RRC connection and the UE has a stored value of the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure;

1>	the establishment or resumption request is for mobile originating calls and the establishment cause is mo-Data or mo-ExceptionData or delayTolerantAccess;

1>	for CP transmission using PUR, the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS configured for PUR.

NOTE 1:	Upper layers request or resume an RRC connection. The interaction with NAS is up to UE implementation.

NOTE 2:	It is up to UE implementation how the UE determines whether the establishment or resumption request is suitable for transmission using PUR.

[TS 36.331[17], clause 5.3.3.4]

2>	if the RRCConnectionSetup is not in response to transmission using PUR and the UE has a stored pur-Config including pur-ConfigID:

3>	include the stored pur-ConfigID;

[TS 36.331[17], clause 5.6.23.2]

A UE in RRC_CONNECTED may initiate the procedure when all of the following conditions are fulfilled:

1>	if the UE is connected to EPC:

2>	for CP transmission using PUR, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes cp-PUR-EPC; or

2>	for UP transmission using PUR, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes up-PUR-EPC;

##### 22.1.6.3	Test description

22.1.6.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 as specified in TS 36.508 clause 8.1.4.1.1 is configured.

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

-	If UE supports delaying the start of the pur-ResponseWindowTimer for NTN operation as specified in TS 36.306 [13] (pc_NB_ntn_PUR_TimerEnhancement), the SS is configured to delay the response to over the PUR received UL message by UE-eNB RTT according to TS 36.321 [14].

UE:

The UE is configured to use PUR to establish or resume the RRC connection whenever it is possible as per TS 36.331[17] clause 5.3.3.1c.

Preamble

-	UE is in state NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB) with test loop mode B according to TS 36.508 [18] in Ncell 1.

22.1.6.3.2	Test procedure sequence

Table 22.1.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| - | EXCEPTION: In parallel with below step 1 the parallel behaviour defined in table 22.1.6.3.2-2 might happen. | - | - | - | - |  |
| 1 | SS transmits 1 IP packet message containing downlink user data of 1 octet. | <-- | IP Packet (containing 1 octet user data) | - | - |
| 2 | The SS sends the RRCConnectionRelease-NB including the releaseCause set to rrc-Suspend, the resumeID, the NextHopChainingCount and PUR-Config withpur-TimeAlignmentTimer > IP PDU delay. (Note 1) | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 3 | Check: Does the UE transmit a MAC PDU containing the RRCConnectionResumeRequest-NB message including the resumeIdentity and AS security context stored at Step 1 and the data received in step 1 on DTCH over PUR? | --> | MAC PDU (RRC: RRCConnectionResumeRequest-NB, data on DTCH) | 1 | P |
| 3A | The SS sends the RRCConnectionRelease-NB. | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 3B | Test procedure in Table 8.1.5A.9.3-1 in TS 36.508[18]. | - | - | - | - |
| 4 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 5 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 6 | The SS sends CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 7 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 8 | SS transmits 1 IP packet message containing downlink user data of 1 octet. | <-- | IP Packet (containing 1 octet user data) | - | - |
| 9 | The SS sends the RRCConnectionRelease-NB including the releaseCause set to rrc-Suspend, the resumeID and the NextHopChainingCount) | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 10 | The UE transmit a MAC PDU containing the RRCConnectionResumeRequest-NB message including the resumeIdentity and AS security context stored at Step 8 and the data on DTCH using PUR. | --> | MAC PDU (RRC: RRCConnectionResumeRequest-NB, data on DTCH) | - | - |
| 11 | The SS sends RRCConnectionSetup-NB | <-- | RRC: RRCConnectionSetup-NB | - | - |
| 12 | Check: Does the UE send RRCConnectionSetupComplete-NB not including pur-ConfigID? | --> | RRC: RRCConnectionSetupComplete-NB | 2 | P |
| 13 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 14 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 15 | The SS sends CLOSE UE TEST LOOP message with IP delay > pur-TimeAlignmentTimer. (Note 1) | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 16 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 17 | SS transmits 1 IP packet message containing downlink user data of 1 octet. | <-- | IP Packet (containing 1 octet user data) | - | - |
| 18 | The SS sends the RRCConnectionRelease-NB with the releaseCause set to rrc-Suspend, the resumeID and the NextHopChainingCount | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 19 | Check: Does the UE send RRCConnectionResumeRequest-NB not over PUR? | --> | RRC: RRCConnectionResumeRequest-NB | 4 | P |
| 20 | The SS sends RRCConnectionSetup-NB | <-- | RRC: RRCConnectionSetup-NB | - | - |
| 21 | Check: Does the UE sends RRCConnectionSetupComplete-NB with SERVICE REQUEST and with pur-ConfigID included? | --> | RRC: RRCConnectionSetupComplete -NB NAS: SERVICE REQUEST | 3 | P |
| 22-25 | Steps 5 to 8 of test procedure in Table 8.1.5A.9.3-1 in TS 36.508[18]. | - | - | - | - |
| 26 | The UE loops back the IP packet received in Step 17. | --> | IP packet (containing 1 octet user data) | - | - |
| Note 1: pur-TimeAlignmentTimer = pur-TimeAlignmentTimer-r16 * periodicityAndOffset-r16 * H-SFN duration (10.24s) = 81.92s |  |  |  |  |  |

Table 22.1.6.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| 1 | The UE sends PURConfigurationRequest | --> | RRC: PURConfigurationRequest-NB | - | - |

22.1.6.3.3	Specific message contents

Table 22.1.6.3.3-1: CLOSE UE TEST LOOP (preamble and step 6 and 15)

| Derivation path: TS 36.508[18] table 4.7A-3, Condition: UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| IP PDU delay | '00000101 'B | 5 seconds delay | Preamble and Step 6 |
|  | '01011010 'B | 90 seconds delay | Step 15 |

Table 22.1.6.3.3-2: SystemInformationBlockType2-NB (preamble and all steps, Table 22.1.6.3.2-1)

| Derivation Path: TS 36.508[18] Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| earlySecurityReactivation-r16 | True |  |  |
| up-PUR-EPC-r16 | True |  |  |
| } |  |  |  |

Table 22.1.6.3.3-3: RRCConnectionRelease-NB (steps 2, 9 and 18, Table 22.1.6.3.2-1)

| Derivation path: TS 36.508[18] Table 8.1.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r13 SEQUENCE { |  |  |  |
| releaseCause-r13 | rrc-Suspend |  |  |
| resumeIdentity-r13 | ’0000000000000000000000000000000000000001’B |  | Step 2 |
|  | ‘0000000000000000000000000000000000000010’B |  | Step 9 |
|  | ‘0000000000000000000000000000000000000011’B |  | Step 18 |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-NB-v1430-IEs |  |
| redirectedCarrierInfo-v1430 | Not present |  |  |
| extendedWaitTime-CPdata-r14 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-NB-v1530-IEs |  |
| drb-ContinueROHC-r15 | Not present |  |  |
| nextHopChainingCount-r15 | 0 |  | Step 2 |
|  | 1 |  | Step 9 |
|  | 2 |  | Step 18 |
| nonCriticalExtension | Not present |  | Steps 9,18 |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-NB-v1550-IEs | Step 2 |
| redirectedCarrierInfo-v1550 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-NB-v15b0-IEs |  |
| noLastCellUpdate-r15 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-NB-v1610-IEs |  |
| resumeIdentity-r16 | Not present |  |  |
| anr-MeasConfig-r16 | Not present |  |  |
| pur-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  | PUR-Config-NB-r16 |  |
| pur-ConfigID-r16 | '00000000000000000001’B |  |  |
| pur-TimeAlignmentTimer-r16 | 1 |  |  |
| pur-NRSRP-ChangeThreshold-r16 | Not present |  |  |
| pur-ImplicitReleaseAfter-r16 | Not present |  |  |
| pur-RNTI-r16 | ’0001 0001 0001 0001’B |  |  |
| pur-ResponseWindowTimer-r16 | pp32 |  |  |
| pur-StartTimeParameters-r16 SEQUENCE { |  |  |  |
| periodicityAndOffset-r16 CHOICE { |  |  |  |
| periodicity8 | 1 |  |  |
| } |  |  |  |
| startSFN-r16 | 0 |  |  |
| startSubframe-r16 | 0 |  |  |
| hsfn-LSB-Info-r16 | 0 |  |  |
| } |  |  |  |
| pur-NumOccasions-r16 | infinite |  |  |
| pur-PhysicalConfig-r16 SEQUENCE { |  |  |  |
| carrierConfig-r16 SEQUENCE { |  |  |  |
| dl-CarrierConfig-r13 SEQUENCE { |  |  |  |
| dl-CarrierFreq-r13 SEQUENCE |  |  |  |
| carrierFreq-r13 | Downlink EARFCN of NCell 1 |  |  |
| carrierFreqOffset-r13 | Carrier frequency offset of Ncell 1 |  |  |
| } |  |  |  |
| downlinkBitmapNonAnchor-r13 | Not present |  |  |
| dl-GapNonAnchor-r13 | Not present |  |  |
| inbandCarrierInfo-r13 | Not present |  |  |
| nrs-PowerOffsetNonAnchor-v1330 | Not present |  |  |
| dl-GapNonAnchor-v1530 | Not present |  |  |
| dl-CarrierFreq-v1550 | Not present |  |  |
| } |  |  |  |
| ul-CarrierConfig-r13 SEQUENCE { |  |  |  |
| ul-CarrierFreq-r13 | Not present |  |  |
| tdd-UL-DL-AlignmentOffset-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| npusch-NumRUsIndex-r16 | 0 |  |  |
| npusch-NumRepetitionsIndex-r16 | 0 |  |  |
| npusch-SubCarrierSetIndex-r16 CHOICE { |  |  |  |
| khz3dot75 | 0 |  |  |
| } |  |  |  |
| npusch-MCS-r16 CHOICE { |  |  |  |
| singleTone | 1 |  |  |
| } |  |  |  |
| p0-UE-NPUSCH-r16 | 0 |  |  |
| alpha-r16 | al0 |  |  |
| npusch-CyclicShift-r16 | n0 |  |  |
| npdcch-Config-r16 | Table 8.1.6.3-3 in TS 36.508[18]. | NPDCCH-ConfigDedicated-NB-DEFAULT |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.1.6.3.3-4: Void

Table 22.1.6.3.3-5: RRCConnectionResumeRequest-NB(steps 3, 10 and 19 Table 22.1.6.3.2-1)

| Derivation Path: 36.508[18] table Table 8.1.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionResumeRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionResumeRequest-r13 SEQUENCE { |  |  |  |
| resumeID-r13 | ’0000000000000000000000000000000000000001’B |  | Step 3 |
|  | ‘0000000000000000000000000000000000000010’B |  | Step 10 |
|  | ‘0000000000000000000000000000000000000011’B |  | Step 19 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.1.6.3.3-6: RRCConnectionSetupComplete-NB (steps 12 and 21 Table 22.1.6.3.2-1)

| Derivation path: TS 36.508[18] table Table 8.1.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete-NB ::= SEQUENCE { |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |
| gummei-Type-r14 | Not checked |  |  |
| dcn-ID-r14 | Not checked |  |  |
| nonCriticalExtension ::= SEQUENCE { | Not checked |  |  |
| measResultServCell-r14 | Not checked |  |  |
| nonCriticalExtension ::= SEQUENCE { | Not checked |  |  |
| ng-5G-S-TMSI-r16 | Not checked |  |  |
| registeredAMF-r16 | Not checked |  |  |
| gummei-Type-v1610 | Not checked |  |  |
| guami-Type-r16 | Not checked |  |  |
| s-NSSAI-list-r16 | Not checked |  |  |
| ng-U-DataTransfer-r16 | Not checked |  |  |
| up-CIoT-5GS-Optimisation-r16 | Not checked |  |  |
| rlf-InfoAvailable-r16 | Not checked |  |  |
| anr-InfoAvailable-r16 | Not checked |  |  |
| pur-ConfigID-r16 | Not present |  | Step 12 |
|  | '00000000000000000001’B |  | Step 21 |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |