3GPP TSG Meeting #110	RP-253615

# Baltimore, USA, 8th Nov 2025 – 11th Nov 2025

# Agenda item:			9.3.4.3

# Source:					Eutelsat Group

# Title:						Motivation for WID revision on support for circular polarization

# Document for:		Information

# Overview

This paper provides motivation for the WID revision in [1] of the WID in [2].

# Discussion

## Antenna polarization

The original WID in [2] contained the following text:

- Define enhanced test methodologies to cover NTN Ka and Ku bands

  - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing

    - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority

    - FR1 OTA test method extension is not precluded, if also applicable

  - Study whether and how to support circular polarization measurements

    - Linear polarization is the baseline

To date, all radiated test systems have been based on linear polarization, and this was the motivation for the statement that linear polarization was the baseline. The need to study whether and how to support circular polarization for NTN in bands above 10 GHz was acknowledged at RAN #109 in the WID. Following discussions at RAN4 #116bis and RAN4 #117, RAN4 has concluded that circular polarization is essential for Ka band and is the deployed polarization for Ku band for LEO, with linear being used by Ku band GSO.

Consequently, RAN4 #117 agreed maintenance CRs to Rel-18 38.101-5 and 38.108 in [3] and [4] to clarify that all NTN radiated requirements are applicable for the manufacturer declared polarization, which can be circular or linear.

In addition, RAN4, having studied the need for circular polarization called out in the WID [2], agreed in the WF [5] the following wording for a revision to the WID:

- Define enhanced test methodologies to cover NTN Ka and Ku bands

  - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing

    - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority

    - FR1 OTA test method extension is not precluded, if also applicable

  - Study whether and how to support circular polarization measurements

    - Linear polarization shall be supported and is the sole fallback if CP cannot be supported.

    - The goal is to support LP and CP in a single test system

The revised WID in [1] implements the above RAN4 agreement from [5].

# Summary

Proposal: RAN agrees the revised WID in [1] to clarify the need for circular polarization according to the RAN4 agreement in [5].

# References

[1]	RP-253616 “Revised WID Enhancement of UE OTA test method and requirements for NR”, Eutelsat Group, RAN #110 Dec 2025

[2]	RP-252903 “New WID: Enhancement of UE OTA test method and requirements for NR”, Moderator (Vivo), RAN #109 Sep 2025

[3]	RP-2522420 “CR 38.101-5 Applicability of minimum requirements related to polarization” Eutelsat Group, RAN4 #117 Dec 2025

[4]	RP-2523211 “CR 38.108 Applicability of minimum requirements related to polarization” Eutelsat Group, RAN4 #117 Dec 2025

[5]	R4-2522665 “WF on NR_UE_OTA_Enh” Eutelsat Group, RAN4 #117 Dec 2025