3GPP TSG-RAN Meeting #110	  RP-253620

# Baltimore, MD, US, Dec. 2025

Title:	On CA with HPUE and 4Tx

Source:	Qualcomm Incorporated, Verizon, T-Mobile USA, AT&T

Agenda item:	9.7

Document for:	Discussion

# Introduction

Support for higher power UEs such as PC1.5 and PC1 and higher number of Tx-es (e.g. 4Tx) has been gradually introduced in previous release or is currently under work in RAN4. These enhancements will bring immediate system performance enhancements to existing deployments. To realize the full potential of these enhancements, it is important to also enable these features for CA. This will alleviate the UL Coverage and Capacity pressure in FWA applications as traffic is shifting toward increasing UL/DL ratios with newer devices and applications. FWA continues to be an important area of growth among carriers.

In this contribution, we discuss how to enable higher power classes with higher number of transmit chains/UL MIMO layers and propose to start the RAN4 specification work.

# Discussion

As system throughput demands increase, HPUEs play increasingly important roles in modern networks. Modern networks generally configure UEs with inter-band CA or similar, to ensure sufficient operational flexibility for mobility and/or load-balancing. It is rare for a UE to operate in a single band in both UL and DL. As a consequence, it is important to enable inter-band CA operation while taking advantage of the HPUE capabilities for single carrier. Otherwise, a significant part of the benefit of the HPUE efforts would be forfeited. In CA operation, the CA power class is used to convey the UE’s cumulative UL power capability. The implication is that the CA power class of the UE must also be progressively higher as higher per-band power-classes are aggregated.

Observation 1: Inter-band CA needs to be enabled to maximize the benefits of newly introduced HPUE features  .

Another aspect which should be considered is that the current RAN4 specification only cover up to 3Tx for CA. Even though 4Tx specifications are already defined for single CC, it is not possible yet to take advantage of UEs with such capabilities in carrier aggregation configurations. A UE with 4 Tx chains could also easily operate in UL CA with 2Tx in each band. Depending on the network configuration, deployment scenario and traffic conditions, using inter-band UL CA with 2Tx in each band instead of single carrier with 4Tx could lead to improved performance.

Observation 2: Inter-band UL CA is currently limited to 3Tx even though 4Tx specifications already exist for single CC.

Observation 3: Enabling inter-band UL CA with 2Tx+2Tx will bring system performance benefits.

To better illustrate the problem, we show two cases of inter-band CA with existing power classes in each band which the current specifications do not support or for which the output power would be artificially limited.

| Case | CA power class | Band1 power class | Band2 power class | Max. aggregated output power in inter-band CA | Problem |
| --- | --- | --- | --- | --- | --- |
| 1 | PC1.5 | PC1.5 (29 dBm) | PC1.5 (29 dBm) | 29.0 dBm | Not supported due to lack of 2Tx+2Tx specifications for CA. UE max. cumulative UL power would be throttled back by 3 dB compared to aggregate across UL bands |
| 2 | PC1.5 | PC2 (26 dBm) | PC1.5 (29 dBm) | 30.8 dBm | Only possible with 1Tx PC2, not with 2Tx PC2. UE with lower per-band power capability can transmit at higher power with the higher power limit R17 capability See case 1 above for comparison |

The table above illustrates multiple issues which are relatively easy to fix in the RAN4 specifications. The needed spec changes are:

- Introduction of support for 2Tx+2Tx for inter-band UL CA

- Introduction of PC1 or the new power class with 4x26dBm for CA (new CA power class)

In order to fix these issues and address the immediate commercial needs for HPUE with CA, RAN4 should start the work on 1 as soon as possible. 2 should be addressed as soon as RAN4 reaches consensus on the power class definition for 4x26dBm HPUE which is currently under discussion in Rel-20, and RAN2 will introduce the corresponding power class signaling. It should be noted that case 2 in the table above can be immediately solved just by 1 above, there is no need for any new signaling.

Proposal 1: RAN4 to start the work on defining requirements for 2Tx+2Tx for inter-band UL CA under TEI.

Proposal 2: RAN2 to introduce a new CA power class based on the outcome of the discussion on 4x26dBm HPUE power class.

The actual specification changes for 1 are straightforward, a list is presented in the Annex of this document. The changes are simply incremental or specification applicability changes.

Observation 4: The changes need to introduce 2Tx+2Tx for inter-band UL CA are limited. A list is shown in the Annex.

Furthermore, RAN4 is currently working on introducing requirements for HPUE with output power targeting 4x26dBm. We will refer to this as PC1 here for simplicity even though the actual power class depends on the outcome of the corresponding RAN4 discussion. This introduction of PC1 will create additional cases shown in the table below:

| Case | CA power class | Band1 power class | Band2 power class | Max. aggregated output power in inter-band CA | Problem |
| --- | --- | --- | --- | --- | --- |
| 3 | PC ?? | PC1 (31 dBm) | PC1.5 (29 dBm) | Expectation is 33.1 dBm | CA power class missing for a UE with PC1 in one of the bands. Could be addressed by higher power limit R17. |
| 4 | PC ?? | PC1 (31 dBm) | PC1 (31 dBm) | Expectation is 34 .0 dBm | CA power class missing for a UE with PC1 in one of the bands |

The above combinations also depend on the introduction of 2Tx+2Tx and potentially a new higher power class. In order to enable these potential new power class combinations as soon as possible, RAN4 should start working on enabling CA combinations with higher output power and 4Tx.

# Conclusion

Observation 1: Inter-band CA needs to be enabled to maximize the benefits of newly introduced HPUE features  .

Observation 2: Inter-band UL CA is currently limited to 3Tx even though 4Tx specifications already exist for single CC.

Observation 3: Enabling inter-band UL CA with 2Tx+2Tx will bring system performance benefits.

Proposal 1: RAN4 to start the work on defining requirements for 2Tx+2Tx for inter-band UL CA under TEI.

Proposal 2: RAN2 to introduce a new CA power class based on the outcome of the discussion on 4x26dBm HPUE power class.

Observation 4: The changes need to introduce 2Tx+2Tx for inter-band UL CA are limited. A list is shown in the Annex.

# References

- RP-252942, New WID: UE RF enhancements for NR FR1, Phase 5, vivo (Moderator), RAN Meeting #109, Beijing, China.

# Annex – Specification changes needed

-Modify generic notes in Clause 5 applying for PC1, or to add explicit notes into desired PC1 band combinations in clause 5

-Introduction of 2T+2T UE power class table (6.2H.3.X). Alternatively, current Table 6.2H.3.1-1 (Inter-band UL CA with UL-MIMO on one band) header along with part of generic text in section 6.2H.3.1 could be modified to include both 3TX (UL-MIMO on one band) and 4TX (UL/MIMO on both bands). Modifying current table would be easier solution, easier to agree for which band combis PC1 is applicable, and to set restriction that 4Tx is e.g. applicable to PC1 only etc.

-Amendments to table 6.2H.3.1-2 to add applicable per band UE power class for PC1 REFSENS exceptions, or to add a new table in case dedicated 4Tx UE power class table is specified.

-Addition “PC1” into applicability sentences in LUT section 7A.2.3.2

-Amendments to Inter-band CA LUT’s cover delta MSD’s for PC1 MSD. This is straight-forward, with the main point of discussion being if MSD continues to scale up as earlier, of if a bit more stringent scaling is applied. If normal scaling is used, completion is very simple