3GPP TSG RAN Meeting #110						RP-253621

Baltimore, USA, December 8-11, 2025

Agenda Item:		9.3.1.3

Source:		Nokia

Title:				Selection of a UE-side Data Collection Solution

Document for:		Approval

# 1	Introduction

During RAN#106, SA2 was asked in RP-243316 to study a user plane (UP) data transfer solution for UE-side data collection. An initial checkpoint of Q2, 2025 or RAN#108 was set, which was extended (see the chair notes of RP-252864) to this present meeting. In response to RAN's request, SA2 sent an LS in S2-2510958 requesting that RAN and RAN2 review “interim agreements with remaining open issues on how to support the transfer of standardized data over UP for UE data collection for UE-side model training.”

In this paper we discuss SA2’s progress on UE-side data collection and propose to make a decision on how to progress.

# 2	Discussion

## 2.1	SA2 progress

Some principles were agreed for key issue (KI) 1 of TR 23.700-04 during SA2#172 in S2-2510959, describing the components of the UP data transfer solution for UE-side data collection. The principles can be broken down into establishment of a data transfer path, transfer of data, and management of data transfer.

Establishment of a data transfer path

The establishment of a data transfer path from a UP path termination entity to a UE is described. Two options are provided to enable the UE to discover the destination of data: direct configuration of an entity address in the UE and configuration through the core network via an N1 container via NAS. The UE establishes a connection to the UP path termination entity through a new or existing PDU session.

Data transfer

In the data transfer procedure, a data transfer request is sent from the UE side server/UE model training entity server (UMTES) (i.e. AF/OTT server for UE-side model training) to the UP path termination entity. The request contains information related to data transfer such as a UE list, list of type allocation codes (TACs), geographical area and a requested data id. The request for data transfer is sent to the UE(s). The UE uses the previously established data transfer path to transmit the collected data to the UP path termination entity. Furthermore, the UP path termination entity “can send requested data id to the gNB via AMF, which may assist the gNB to decide [a] data collection configuration”.

Management of data transfer

To give network operators control over data transfer, SA2 studied traffic differentiation, controllability over the start and stop of data transfer, and visibility of data. Traffic differentiation can be achieved through a dedicated S-NSSAI or DNN at the PDU session level or through a QoS flow for data transfer. Data transfer is controlled through commands from the UP path termination entity, which can initiate or stop data transfer, and by the UE, which can accept or reject a data transfer request. Visibility is achieved through checking the standardized data at the UP path termination entity. User consent is checked through the UDM.

## 2.2	RAN evaluation

SA2 described an end-to-end system through which a UE can be configured for data transfer, having been selected by its identity or through a general identifier like a type allocation code. These could be further evaluated by RAN WGs. The protocol between the UE and the UP data path termination entity was not described, but we can assume that protocol would be standardized and carry the standardized measurements from the UE. The requirements of controllability, differentiability and visibility were considered by SA2 and they have described concrete ways in which they can be met.

Observation: RAN WGs can further evaluate the coordination between the configurations provided by UMTES and the configurations required for data collection.

As the solution depicted in the SA2 TR can meet the RAN requirements, we think that RAN should select this solution for 5G Rel-20 normative specification, and request SA2 to finalize the study and create the necessary specifications.

Proposal 1: Respond to SA2 that RAN have selected their option 2 for UE-side data collection transfer and request SA2 to finalize the study and create the necessary specifications in Rel-20 timeframe.

In clause 7.2.1 of TR 23.700-04 the open issues are listed. In some of them ([17], [18], and [20]) the need of RAN feedback is explicitly mentioned. There are also open issues that may have RAN impacts (e.g., in [1], it is asked whether and how to align the UE data measurement with the requirement of data transfer; in [14], it is asked whether information about the requested data is sent with the data transfer request). Therefore, we think that the solution of these open issues requires investigations and feedback from RAN2 and RAN3.

Proposal 2: In the reply LS RAN also ask RAN2 and RAN3 to investigate and provide feedback on the open issues related to RAN.

We provided a draft reply LS proposal in the Annex if proposal 1 and 2 are acceptable.

Proposal 3: Use the draft reply LS provided in the Annex as a baseline for the reply LS to SA2.

# 3	Conclusion

In this TDoc we made the following observation and proposals:

Observation: RAN WGs can further evaluate the coordination between the configurations provided by UMTES and the configurations required for data collection.

Proposal 1: Respond to SA2 that RAN have selected their option 2 for UE-side data collection transfer and request SA2 to finalize the study and create the necessary specifications in Rel-20 timeframe.

Proposal 2: In the reply LS RAN also ask RAN2 and RAN3 to investigate and provide feedback on the open issues related to RAN.

Proposal 3: Use the draft reply LS provided in the Annex as a baseline for the reply LS to SA2.

# Annex: Draft reply LS to SA2

Title:	Reply LS on AI/ML UE sided data collection

Response to:	S2-2510958/ RP-253004

Release:	Release 20

Work Item:	NR_AIML_air_Ph2, FS_AIML_CN_Ph2

Source:

To:	SA2, RAN2, RAN3

Cc:	SA, SA3, SA5, RAN1

Contact Person:

#### Name:

E-mail Address:	@

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN thank SA2 for sending the LS on AI/ML UE sided data collection (S2-2510958/RP-253004).

In the LS SA2 requested feedback on whether Option 2 UP is selected by RAN.

RAN would like to inform SA2 that RAN selected Option 2 UP for normative specification work in Rel-20.

RAN also noticed that some of the open issues listed in clause 7.2.1 of TR 23.700-04 v1.2.0 are RAN related. Therefore, RAN would like to ask RAN2 and RAN3 to study the RAN related open issues and provide feedback to SA2.

2. Actions:

To SA2 group.

ACTION: 	RAN respectfully asks SA2 to continue their work on Option 2 UP as it is the selected solution for Rel-20.

To RAN2 and RAN3 group.

ACTION: 	RAN respectfully asks RAN2 and RAN3 to study the RAN related open issues of clause 7.2.1 of TR 23.700-04 v1.2.0 and provide feedback to SA2.

3. Date of Next TSG-RAN Meetings:

TSG RAN#111	9-12 March 2026,	 	Fukuoka, Japan.

TSG RAN#112	9-12 June 2026,		Singapore