3GPP TSG-RAN WG4 Meeting #117	R4-2520569

# Dallas, USA,  17th – 21st November 2025								(revision of R4-2513846)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | 0043 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of power back-off simulation results for the NTN L-bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_combinedLband-Core |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-19 introduced support for extended L-bands n250 and n251, for which the corresponding NS flags were added to support ETSI regulations. However, the power back-off simulation results, from which final A-MPR values were devised, were not captured in TR 38.863. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Power back-off simulation results are captured in Annex G (based on original contribution Apple/R4-2509533 and Qualcomm/R4-2511405 with additional corrections and clarifications). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Power back-off simulation results will remain uncaptured in the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex G:
Power back-off simulation results for the L-band

# G.1	PC3 power class

## G.1.1	General aspects

NOTE:	The purpose of the section is to capture general aspects, such as considered UE RF architectures, assumptions, considered	regulations, etc.

### G.1.1.1	NS_02N

#### G.1.1.1.1	Company A (OPPO/R4-2510521)

##### G.1.1.1.1.1	Simulation assumptions

Since filter rejection at the frequencies 1559-1610MHz is not a constant 10dB attenuation, but rather has a slope, it is hard to add the filter rejection without the slope factor. Thus, to evaluate the rejection effect of the filter, required power back-off is simulated under the following assumptions:

-	no filter rejection is assumed;

-	PC3 CP-OFDM QPSK using fixed bias PA and APT PA;

-	5, 10, 15 and 20MHz channels residing at the lowest edge of the uplink operating band n255

##### G.2.1.1.1.2	Simulation results

For PC3 CP-OFDM QPSK using fixed bias PA:

-	Power back-off 0dB is needed for 5MHz, 10MHz and 15MHz without additional filter rejection.

-	Power back-off 3dB is needed for 20MHz without additional filter rejection.

For PC3 CP-OFDM QPSK using APT PA:

-	Power back-off 0dB is needed for 5MHz and 10MHz without additional filter rejection.

-	Power back-off 2dB for 15MHz and 10.5dB for 20MHz without additional filter rejection.

Based on the simulation results, no matter which PA model is used, the filter attenuation is needed for NS_02N for PC3 to be able to operate within existing MPR margins.

### G.1.1.2	NS_09N

#### G.1.1.2.1	Company A

##### G.1.1.2.1.1	Simulation assumptions

The following simulation assumptions are used:

-	Considered Tx frequency: 1668 – 1675MHz

-	Tx power: PC3 (23dBm)

-	Calibration: 1dB MPR with DFT-s-OFDM QPSK 20MHz, 100RB0

-	Carrier Leakage: 28dBc

-	Image: 28dBc

-	CIM3: 60dBc

-	CIM5: 70dBc

-	Additional requirements: out-of-band unwanted emission limits according to ETSI EN 301681

##### G.1.1.2.1.2	Simulation results

Figure G.1.1.2.1.2-1 presents power back-off values for the 5MHz channel at the lower edge of the band. As can be seen most allocations do not need A-MPR, except the case(s) with a single RB allocated at the right edge of the 5MHz channel. The reason for that is the upper edge out-of-band emission requirement limits, which would require some A-MPR. As presented in Figure G.1.1.2.1.2-2 below, CP-OFDM slightly exceeds the limit of -30dBm/30kHz and DFT-s-OFDM is also hitting that limit.

Figure G.1.1.2.1.2-1: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1670.5MHz).

Figure G.1.1.2.1.2-2: Spectrum plot for the 5MHz channel and 1RB allocation (Fc=1670.5MHz).

Once the 5MHz channel is shifted farther towards the upper edge of the band, even higher A-MPR might be needed, which is however mostly concerns single RB allocations at the upper edge of the configured channel.

Figure G.1.1.2.1.2-3: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1671.5MHz).

Figure G.1.1.2.1.2-4: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1672.5MHz).

### G.1.1.3	NS_10N

#### G.1.1.3.1	Company A

##### G.1.1.3.1.1	Simulation assumptions

The following simulation assumptions are used:

-	Considered Tx frequency: 1626.5 – 1660.5MHz

-	Tx power: PC3 (23dBm)

-	Calibration: 1dB MPR with DFT-s-OFDM QPSK 20MHz, 100RB0

-	Carrier Leakage: 28dBc

-	Image: 28dBc

-	CIM3: 60dBc

-	CIM5: 70dBc

-	Addition requirements: out-of-band unwanted emission limits according to ETSI EN 301681

##### G.1.1.3.1.2	Simulation results

As can be seen from the Figure G.1.1.3.1.2-1 below, when the 5MHz NR channel is in the middle of the band, no additional AMPR is anticipated: the out-of-band emission requirements are not relevant because of the size of the band and the in-band ETSI emission mask does not create any additional restrictions either.

Figure G.1.1.3.1.2-1: Power back-off for CP-OFDM and DFT-s-OFDM (5MHz at Fc=1645MHz).

As for the channels either at the lower or the upper edge of the band, extra A-MPR is needed for larger allocations with DFT-s-OFDM and the right-most channel edge allocations as presented in Figure G.1.1.3.1.2-2 below. And the same observations can be done for the 5MHz channel at the upper edge of the band: left-most channel allocations will need extra A-MPR to comply with the out-of-band emission requirements.

Figure G.1.1.3.1.2-2: Power back-off for CP-OFDM and DFT-s-OFDM (5MHz at Fc=1629MHz).

Figure G.1.1.3.1.2-3: Power back-off for CP-OFDM and DFT-s-OFDM (5MHz at Fc=1658MHz).

One of the reasons why the left- and right-most channel power back-off look identical despite the fact that out-of-band emission requirements are not the same is the fact that the limiting factor is -30dBm/30kHz. Two figures below present the spectrum plot for the 5MHz channels at the left and right edges of the band, from which one can see that the most limiting factor is the -30dBm/30kHz emission limit at the offset of 2MHz from the band edge.

Figure G.1.1.3.1.2-4: Spectrum plot for the 5MHz channel at the left edge (top figure) and right edge (bottom figure) of the band.

For the 10MHz channel, G.1.1.3.1.2-4 presents results for a case when the 10MHz channel resides in the middle of the band; and as can be seen from the figure, no extra A-MPR is needed, i.e. power back-off values are within the MPR margin.

Figure G.1.1.3.1.2-4: Power back-off for CP-OFDM and DFT-s-OFDM (10MHz at Fc=1645MHz)

Figure G.1.1.3.1.2-5 and G.1.1.3.1.2-6 present results for channels residing at the left and the right edges of the band. General observations are similar to the 5MHz channel, the only noticeable difference is that the 10MHz channel at the upper edge of the band requires slightly higher power back-off due to more stringent out-of-band emission requirements.

Figure G.1.1.3.1.2-5: Power back-off for CP-OFDM and DFT-s-OFDM (10MHz at Fc=1631.5MHz).

Figure G.1.1.3.1.2-6: Power back-off for CP-OFDM and DFT-s-OFDM (10MHz at Fc=1655.5MHz).

Figure 2.3.1.3-1 presents results for the 15MHz channel located in the middle of the band. Unlike previous cases for the 5 and 10MHz channels, this time extra A-MPR is need because the power back-off values exceed the MPR margin for edge RB allocations.

Figure G.1.1.3.1.2-7: Power back-off for CP-OFDM and DFT-s-OFDM (15MHz at Fc=1645MHz).

Power back-off simulation results for the 15MHz channels residing at the band edges are logically identical to the 10MHz results presented above. The 15MHz channel at the upper edge of the band requires even higher power back-off for large allocations and the for the right-most allocations within the band.

Figure G.1.1.3.1.2-8: Power back-off for CP-OFDM and DFT-s-OFDM (15MHz at Fc=1634MHz).

Figure G.1.1.3.1.2-9: Power back-off for CP-OFDM and DFT-s-OFDM (15MHz at Fc=1653MHz).

The 20MHz channel will require even higher power back-off. As can be seen from Figure G.1.1.3.1.2-10, when the 20MHz channel is in the middle of the band, only edge RB allocations need extra A-MPR.

Figure G.1.1.3.1.2-10: Power back-off for CP-OFDM and DFT-s-OFDM (20MHz at Fc=1645MHz).

For the channel at the lower edge of the band, the left-most allocations will need the highest power back-off reaching 5-6.5dB. And similarly, the right-most allocations within the channel at the upper edge of the band will require power back-off in the order of  5.5dB.

Figure G.1.1.3.1.2-11: Power back-off for CP-OFDM and DFT-s-OFDM (20MHz at Fc=1636.5MHz).

Figure G.1.1.3.1.2-12: Power back-off for CP-OFDM and DFT-s-OFDM (20MHz at Fc=1650.5MHz).

### G.1.1.4	NS_14N

#### G.1.1.4.1	Company A

##### G.1.1.4.1.1	Simulation assumptions

The following simulation assumptions are used:

-	PA calibrated to 1 dB MPR with 30 dBc ACLR with DFT-s-OFDM QPSK 100 RB waveform

-	-28 dBc IQ-image

-	-28 dBc LO-leakage

-	-60 dBc CIM3 and -70 dBc CIM5

Additional emission limits considered in Table G.1.1.4.1.1-1. Only backoff due to additional emissions limits is shown in figures in clause G.1.1.4.1.2.

Table G.1.1.4.1.1-1: Additional emission requirements

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. |  |  |  |

##### G.1.1.4.1.2	Simulation results

Only backoff due to additional emissions limits is shown.

Figure G.1.1.4.1.2-1: 5MHz at Fc=1629MHz.

Figure G.1.1.4.1.2-2: 10MHz at Fc=1631.5MHz.

Figure G.1.1.4.1.2-3: 15MHz at Fc=1634MHz.

Figure G.1.1.4.1.2-4: 20MHz at Fc=1636.5MHz.