3GPP TSG-RAN4 Meeting #117	R4-2520163

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.194 | CR | 0003 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.194, Correction on A-IoT BS Annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To correct Annex of characteristics of the interfering signal for A-IoT BS |  |  |  |  |  |  |  |  |
| Summary of change: |  | To supplement missing Annex of characteristics of the interfering signal for A-IoT BS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The characteristics of the interfering signal for A-IoT BS would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

Annex A (informative):
D2R channel bandwidth

The following describes the equation to derive BS D2R channel bandwidth.

For BS D2R CBW:

D2R CBW for BS (kHz)

= ceiling ((2SB Transmission BW_without SFO× (1/2) +2× Small frequency shift_without SFO)/0.9)

=ceiling ( (2+2R)/Tb × (1+∣SFO∣)/0.9)

=ceiling ( (1+R)/ (Tc ×R) × (1+∣SFO∣)/0.9)

The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band).

Annex B (normative):
Reference measurement channels

B.1	Fixed Reference Channels for reference sensitivity level, ACS, in-band blocking, out-of-band blocking, (BPSK, OOK)

| Reference channel | A-FR1-A1-1 | A-FR1-A1-2 | A-FR1-A1-3 | A-FR1-A1-4 |
| --- | --- | --- | --- | --- |
| DSB (kHz) | 15 | 15 | 2880 | 2880 |
| Payload size (bits) | 96 | 96 | 96 | 96 |
| CRC (bits) | 16 | 16 | 16 | 16 |
| Preamble length | 31 | 31 | 31 | 31 |
| Midamble length | 31 | 31 | 31 | 31 |
| Midamble configuration I | 48 | 48 | 48 | 48 |
| FEC | 1/3 | 1/3 | 1/3 | 1/3 |
| Line code | Manchester | Manchester | Manchester | Manchester |
| Modulation | BPSK | OOK | BPSK | OOK |
| Waveform (CW) | unmodulated single tone | Unmodulated single tone | unmodulated single tone | unmodulated single tone |
| Sampling frequency（SFO） | between 0.01 and 0.1 | Between 0.01 and 0.1 | between 0.01 and 0.1 | Between 0.01 and 0.1 |
| Total symbols | 398 | 398 | 398 | 398 |

Annex C (normative): 
Characteristics of the interfering signals

The interfering signal shall be a PUSCH containing data and DM-RS symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 6 of TS38.211 [5]. Mapping of PUSCH modulation to receiver requirement are specified in table C-1.

Table C-1: Modulation of the interfering signal

| Receiver requirement | Modulation |
| --- | --- |
| Adjacent channel selectivity | QPSK |
| General blocking | QPSK |
| Receiver intermodulation | QPSK |

Annex D (Informative): 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | RAN4#114bis | R4-2505228 |  |  |  | 3GPP_TS 38.194 skeleton | 0.0.1 |
| 2025-08 | RAN4#116 | R4-2511749 |  |  |  | Agreed TP in RAN4#116: R4-2511722 TP for TR 38.194 5.3 BS channel bandwidth and 5.4 Channel arrangement R4-2511723 TP for TR 38.191 section 3 Definitions, symbols and abbreviations R4-2511725 draft TP for TS 38.194 to introduce base station output power and transmit ON/OFF power R4-2511727 TP for TR 38.194 6.5 Unwanted emissions and 6.6 Transmitter intermodulation R4-2511728 draft TP to TS 38.194 on Transmitted signal quality R4-2511731 TP to TS38.194: REFSENS requirement for A-IoT BS and FRC R4-2511732 TP to TS38.194 : ACS , Inband blocking, OOB and Spurious R4-2511742 TP to TS 38.194 on CW frequency error and unwanted emssion R4-2511743 TP to 38.194 on general and CW output power | 0.0.2 |
| 2025-09 | RAN#109 | RP-252745 |  |  |  |  | 1.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-09 | RAN#109 |  |  |  |  | Approved by plenary – Rel-19 spec under change control | 19.0.0 |

==============End of change==============