3GPP TSG-RAN4 Meeting #117	R4-2521615

# Dallas, USA, 17 – 21 Novermber, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6179 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions-Core) CR on core requirements for R17 NTN R19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In NGSO scenario, UE cannot measure neighbor cell in SMTC and receive data or measure L1 for the serving cell simultaneouly if UE does not support parallelMeasurementWithoutRestriction-r17. This is reflected by scaling factor Klayer1 and scheduling restriction in 9.2C.5.3.2 for intra-freqeuncy measurement without gap, but it is missing for inter-frequency measurement wihtout gap in clause 9.3C.7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add for scaling factor Klayer1 and scheduling restriction inter-frequency measurement without gap in clause 9.3C.7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE capability parallelMeasurementWithoutRestriction-r17 is not accounted in the requirements for inter-frequency measurement without gap. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3C.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 9.3C.7	Inter-frequency measurements without measurement gaps

#### 9.3C.7.1	Inter-frequency Cell identification

If UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network, UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3C.7.1-1.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in

Table 9.3C.7.1-2 for target cells operating with 20 PRB SSB BW in FR1-NTN;

Table  9.3C.7.1-3 for target cells operating with 12 PRB SSB BW in FR1-NTN.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3C.7.2-1.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1.5.1. .

Kp is the scaling factor for a SSB frequency layer to be measured without measurement gaps. Kp = Ntotal_SAN / Navailable_SAN, where Navailable_SAN and Ntotal_SAN are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where

-	If UE supports parallelMeasurementGap-r17 and is configured with concurrent measurement gaps, MGRP_max is the maximum MGRP across all configured per-UE measurement gap. Otherwise, MGRP_max is the MGRP of configured measurement gap.

-	Starting from the beginning of any SMTC occasion:

-	Ntotal_SAN is the total number of SMTC occasions within the window, including those overlapped and non-overlapped with measurement gap occasions within the window, and

-	Navailable_SAN is the number of SMTC occasions within the window W that do not collide with any non-dropped MG occasion within or outside the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in clause 9.1C.8.3. The collision rule between SMTC occasion and measurement gap occasion is defined in clause 9.1C.9.1

Kp = [1] when Navailable_SAN = 0 and measurement gap sharing in clause 9.1.2.1a shall apply.

Kp = 1 when inter-frequency SMTC is fully non overlapping with measurement gaps.

Klayer1_measurement: it is scaling factor for sharing between L3 and L1 measurement, and Klayer1_measurement =1, if GEO satellites are measured on the carrier, or if LEO satellites are measured on the carrier and UE supports parallelMeasurementWithoutRestriction-r17, otherwise

Klayer1_measurement =1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement =1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

Ksatellite: it is a satellite specific scaling factor.

- If SMTCs do not overlap with each other, and if LEO and/or GEO satellite(s) is/are required to be measured within SMTC

  - Ksatellite = 1, if GSO satellite(s) is/are measured on the carrier

  - Ksatellite=Num of LEO satellites to be measured in the SMTCnumber of LEO satellites UE is capable to measure in one SMTC, if LEO satellite(s) is/are measured on the carrier.

- If SMTCs partially overlap with each other, and if LEO and/or GEO satellite(s) is/are required to be measured within overlapped SMTCs

  - Ksatellite=number of overlapped SMTCs, if only GEO satellite(s) is/are measured on the carrier

  - Ksatellite=iNum of LEO satellites to be measured in the SMTC inumber of LEO satellites UE is capable to measure in one SMTC, if only LEO satellite(s) is/are measured on the carrier.

Table 9.3C.7.1-1: Time period for PSS/SSS detection, (FR1-NTN)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600 ms, ceil( 5 x Klayer1_measurement x Kp) x SMTC period )Note 1 x CSSFinter Ksatellite |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(1.5x 5 x Klayer1_measurement x Kp) x max(SMTC period,DRX cycle)) x CSSFinter Ksatellite |
| DRX cycle>320 ms | ceil(5 x Klayer1_measurement x Kp) x DRX cycle x CSSFinter Ksatellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

Table 9.3C.7.1-2: Time period for time index detection (FR1-NTN)

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120 ms, ceil( 3 x Klayer1_measurement x Kp ) x SMTC period)Note 1 x CSSFinter Ksatellite |
| DRX cycle≤ 320 ms | max(120 ms, ceil (1.5 x 3 x Klayer1_measurement x Kp) x max(SMTC period,DRX cycle)) x CSSFinter Ksatellite |
| DRX cycle>320 ms | Ceil(3 x Klayer1_measurement x Kp) x DRX cycle x CSSFinter Ksatellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

Table 9.3C.7.1-3: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (FR1-NTN)

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120 ms, ceil( 6 x Klayer1_measurement x Kp ) x SMTC period)Note 1 x CSSFinter Ksatellite |
| DRX cycle≤ 320 ms | max(120 ms, ceil (1.5 x 6 x Klayer1_measurement x Kp) x max(SMTC period,DRX cycle)) x CSSFinter Ksatellite |
| DRX cycle>320 ms | Ceil(6 x Klayer1_measurement x Kp) x DRX cycle x CSSFinter Ksatellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

#### 9.3C.7.2	Measurement period

The UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in clauses 10.1.4C, 10.1.9C and 10.1.14C, respectively, as shown in table 9.3C.7.2-1, if UE supports inter-frequency measurement without measurement gaps.

Table 9.3C.7.2-1: Measurement period for inter-frequency measurements without gaps (FR1-NTN)

| DRX cycle | TSSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Klayer1_measurement x Kp) x SMTC period)Note 1 x CSSFinter Ksatellite |
| DRX cycle≤ 320 ms | max(200ms, ceil(1.5x 5 x Klayer1_measurement x Kp) x max(SMTC period,DRX cycle)) x CSSFinter Ksatellite |
| DRX cycle>320 ms | ceil( 5 x Klayer1_measurement x Kp ) x DRX cycle x CSSFinter Ksatellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

#### 9.3C.7.3	Scheduling availability of UE during inter-frequency measurements

If UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network, UE is required to be capable of measuring without measurement gaps when the SSB is completely contained in the active bandwidth part of the UE. When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction.  Note that the SSB symbols to be measured in the following clauses are the SSB symbols indicated by SSB-ToMeasure [2], if it is configured; otherwise, all L SSB symbols within the SMTC window duration defined in clause 4.1 of TS 38.213 [3] are included. For UL, the scheduling restriction applies to UL symbols that fully or partially overlap with the restricted symbols as defined below.

##### 9.3C.7.3.1	Void

##### 9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN

For UE which do not support simultaneousRxDataSSB-DiffNumerology-Inter-r16 [14] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement

-	If UE performs inter-frequency measurements without measurement gaps in a FDD band, UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration.

##### 9.3C.7.3.3	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO

For UE which do not support the capability parallelMeasurementWithoutRestriction-r17 the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement on a neighbor cell served by a different satellite in LEO.

-	If UE performs inter-frequency measurements without measurement gaps in a FDD band, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration.

<End of Change 1>