3GPP TSG RAN#110 	RP-253699

# Baltimore, MD, USA, December 8-11, 2025

Source:	T-Mobile USA, Rogers

Title:	Motivation for Introduction of a new NR NTN PCS band (1850-1915 MHz UL and 1930-1995 MHz DL)

Agenda item:			9.1.5.2

Document for:	Discussion

# 1. Introduction

This paper presents the motivations, regulatory context and brief scope of work for the proposal to define a 3GPP NR NTN band with the range of UL 1850-1915 MHz, DL 1930-1995 MHz. The paper also proposes to introduce a work item to define this band in 3GPP as part of Rel-20 work.

# 2. Discussion

## 2.1 Background and context

The introduction of Non-Terrestrial Networks (NTN) into 3GPP NR specifications has significantly expanded the scope of cellular connectivity, enabling coverage in previously unreachable areas through satellite systems. With the successful definition of NTN bands in prior releases — including n255, n256 (Rel-17), n254 (Rel-18), and n252 (Rel-19) — the 3GPP ecosystem now has a robust technical framework to support satellite-based NR communications.

In March 2024, the U.S. Federal Communications Commission (FCC) formally adopted rules under the "Supplemental Coverage from Space" (SCS, not to be confused with Subcarrier Spacing) initiative [3]. This regulatory action allows terrestrial Mobile Network Operators (MNOs) to collaborate with non-geostationary satellite operators to offer coverage using licensed terrestrial spectrum. The rule enables direct-to-device (D2D) services for standard 3GPP UEs, expanding service reach into underserved and remote regions.

During the FCC SCS proceedings, coexistence with other services, including satellite services in Band 256/n256 was studied in detail, including concerns raised about the overlap between the PCS band downlink and the B256/n256 uplink. The FCC studied the issue in detail during the process, requested more information from the parties involved, and in the end concluded SpaceX “therefore will maintain 100% of the time an interference level below the harmful interference threshold for Omnispace satellites and other satellites performing uplink operations in the 1990-1995 MHz band.” [4] The Order and Authorization also states, “Should harmful interference ever occur, under the SCS rules, SpaceX must cease such operations.”[4] This reiterates what was stated in the FCC Report and Order on SCS which stated that they “are enabling certain SCS operations domestically in the United States while also reminding our licensees that SCS operations shall not cause harmful interference to other countries’ operations that conform to the ITU Radio Regulations and shall eliminate any harmful interference immediately. We will continue to monitor ongoing international work on SCS and adjust our approach as needed. We believe, however, that the record in this proceeding provides a sufficient basis for moving forward with authorizing SCS services and adopting rules applicable to their operation.”[3]

Building upon this, T-Mobile USA and Starlink initiated LTE-based SCS services in the PCS G-block (1910–1915 MHz UL, 1990–1995 MHz DL), validating the feasibility of spaceborne services using existing mobile spectrum. As industry transitions from proprietary LTE solutions to NR, standardization of a corresponding NR NTN band is both timely and essential.

Noting that “any expansion of mobile coverage to underserved and unserved areas of the country could provide significant benefit to Canadians”, Canada’s regulator, ISED, introduced a similar framework as the FCC in February 2025 [7]. It permits MNOs to provide SCS (termed Supplemental Mobile Coverage From Space) to underserved and unserved users over a selection of terrestrial frequencies, including PCS. Per the Decision, “Space stations providing SMCS are authorized on a no-interference, no-protection basis in accordance with No 4.4 of the ITU Radio Regulations and the Canadian Table of Frequency Allocations” (CTFA). That is, a licensee that causes harmful interference to a station operating in accordance with ITU Radio Regulations and the CTFA, “must immediately take steps to mitigate the interference, including, if necessary, ceasing the operations of their interfering space station.”

Despite the strict regulations, ISED considers SCS services to be both viable and valuable in Canada. Under the provisions allowed by the Decision, Rogers and Starlink launched SCS services in PCS spectrum in July 2025.

## 2.2 Motivation

While re-using existing terrestrial NR bands (e.g., n25) for NTN was considered, this path would necessitate substantial modifications to current specifications, potentially impacting terrestrial deployments and introducing compatibility concerns. For example:

- 38.331 points to 38.101-5 for “an NTN cell.”

  - p-Max Value in dBm applicable for the intra-frequency neighbouring NR cells. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell.

- 38.306 refers in many places to “NTN bands.”

- For many IEs, 38.306 says that some fields are “only applicable bands in 38.101-5 and HAPS bands in 38.104.” If we start using TN bands defined in 38.101-1 for NTN, 36.306 does not point to these bands as being applicable for NTN.

- For dozens of IEs 38.306 says, “For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands and all FDD-FR2 NTN bands respectively.” If we use the same band for TN and NTN, then all the values shall be set with the capability value consistently for all FR1 TN and NTN bands. This limits implementation flexibility and would be problematic.

- For handovers, how would a gNB know if a UE supports NTN on a given TN band, since nonTerrestrialNetwork-r17 is a per-UE capability, and not a per-band capability? Potentially indirectly via IEs that are only applicable for NTN, but that might confuse legacy UEs if an IE that is only applicable for NTN is indicated for a TN band.

- How to prevent a TN-only UE from trying to RACH on an NTN network? PLMN may block some UEs from trying to access an NTN network, but it is possible that some TN-only UEs don’t have the PLMN in their forbidden list and try to RACH without doppler pre-compensation and sufficient timing advance. MIB barring can prevent legacy TN-only UEs from attempting to RACH, but MIB barring is not an ideal solution.

Therefore, to maintain technical clarity and regulatory alignment, a new NTN-specific band — functionally equivalent to n25 but adapted for satellite access — is proposed.

This new NR NTN PCS band (1850–1915 MHz UL / 1930–1995 MHz DL) would:

- Enable standardized NR-based satellite access using PCS spectrum under FCC rules.

- Ensure alignment with the performance and regulatory constraints of existing terrestrial spectrum allocations.

- Facilitate long-term scalability of SCS use cases and promote global NTN ecosystem growth.

- Prevent TN-only UEs from attempting to RACH on an NTN cell.

- Allow the gNBs to know if the UE supports NTN on a particular band, since the NTN bands would have their own band number.

## 2.3 Objectives of the Work Item

### 2.3.1 Objectives for NR NTN

The Work Item will define a new NR NTN FDD band with the following key elements:

- Specify a new NR NTN band with frequency range: 1850–1915 MHz UL and 1930–1995 MHz DL (entire PCS band).

- Re-use NR band n25 requirements where applicable.

- Channel bandwidth and SCS: Align with existing NR NTN bands (e.g., n256, n255).

| SCS (kHz) | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 | 20 |
| 30 |  | 10 | 15 | 20 |
| 60 |  | 10 | 15 | 20 |

- Support channel raster points at step of 100 kHz and at step of 10 kHz as agreed in RAN WG4

- Support Power Class 3 (+23 dBm).

- Specification updates: RF core and RRM requirements for both SAN and UE, and the corresponding RRM requirements.

## 2.4 Market and Regulatory Drivers

The FCC’s and ISED’s SCS frameworks provide clear regulatory foundations in the United States and Canada, respectively. Standard NR NTN support in PCS spectrum will:

- Enhance rural and emergency connectivity solutions.

- Avoid fragmentation through a common, standard band framework.

- Encourage innovation from chipset, device, and satellite vendors.

2.5 Precedents in 3GPP

- Historically in 3GPP, operators do not block the Work Items proposed by other operators. If an operator has regulatory approval for a band, there is usually not objection from other operators.

- T-Mobile expressed concern about UE-UE coexistence between n252 and n25 prior to RAN approval of the Work Item, but we did not block the creation of the Work Item. We worked with the proponents to find solutions within the Work Item.

- 3GPP has not done satellite-satellite simulations before.

- 3GPP standardized n77, covering 3.3-4.2GHz, before there was an ITU IMT allocation for 3.7-4.2 GHz. WRC-15 only covered up to 3.7 GHz.

# 3. Conclusion

The proposed NR NTN PCS band will serve as a standardized foundation for SCS services over 5G NR. This effort will support ongoing commercial deployments and catalyze new service models, while ensuring technical coherence within 3GPP specifications. Approval of this Work Item for Rel-20 will position the 3GPP ecosystem to meet regulatory, technical, and commercial needs in the NTN space. The background, motivation, brief objectives and market and regulatory drivers have been discussed above.

Proposal:  Introduce a new WID for NR-NTN band with the UL part of 1850-1915 MHz and the DL part of 1930-1995 MHz.

The new WID for NR-NTN is submitted in [x]

# Reference

- RP-221169, Rel-17 Solutions for NR to support non-terrestrial networks (NTN) WID

- TR 38.863, “Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects”

- Single Network Future: Supplemental Coverage from Space, REPORT AND ORDER AND FURTHER NOTICE OF PROPOSED RULEMAKING, US Federal Communications Commission, March 14, 2024, https://docs.fcc.gov/public/attachments/FCC-24-28A1.pdf

- Request for Deployment and Operating Authority for the SpaceX Gen2 NGSO Satellite System, ORDER AND AUTHORIZATION, US Federal Communications Commission, November 26, 2024, https://docs.fcc.gov/public/attachments/DA-24-1193A1.pdf

- Space Bureau and Wireless Telecommunications Bureau Seek Comment on Filings of SpaceX and T-Mobile Requesting to Establish Supplemental Coverage from Space Application for Authority for Modification of the SpaceX NGSO Satellite System to Add a Direct to Cellular System, ORDER, US Federal Communications Commission, March 7, 2025 https://docs.fcc.gov/public/attachments/DA-25-197A1.pdf

- Letter from Omnispace to the FCC, August 30, 2024, https://apps.fcc.gov/els/GetAtt.html?id=357673

- Decision on a Policy, Licensing and Technical Framework for Supplemental Mobile Coverage by Satellite, Innovation, Science and Economic Development Canada, February 2025, https://ised-isde.canada.ca/site/spectrum-management-telecommunications/en/learn-more/key-documents/consultations/decision-policy-licensing-and-technical-framework-supplemental-mobile-coverage-satellite

- RP-253700, New WID: Introduction of NR-NTN PCS band, T-Mobile USA, Rogers, Telstra, Ericsson, Qualcomm, Samsung, RAN#110, December 2025

# Appendix

This appendix discusses concerns about PCS band satellite downlink interfering with the MSS S-Band 256/n256 uplink in other regions.

- corresponding RRM requirements.

## A.1 Concerns raised during the FCC’s NPRM process on SCS

Reverse-band, satellite-to-satellite interference risk. Omnispace argued that using PCS spectrum for space-to-Earth downlinks inside the U.S. conflicts with the International Table’s Earth-to-space (uplink) allocation for MSS outside the U.S., creating the potential for satellite-to-satellite interference into S-band MSS uplinks (i.e., the B256/n256 uplink). They urged the FCC to impose limits to protect MSS systems operating internationally under the ITU Radio Regulations. [3]

Band-specific complexity in PCS G-block (1910–1915 / 1990–1995 MHz). Omnispace highlighted the technical difficulty of enabling SCS in this “reverse-band” pairing and submitted technical analyses criticizing SpaceX’s interference modeling (e.g., constellation dynamics). DISH/EchoStar and Globalstar supported ensuring that new SCS ops do not harm existing satellite systems. [3]

Process/standards posture. Omnispace also claimed that proceeding ahead of international/3GPP consensus could exacerbate cross-border interference concerns. [3]

## A.2 FCC response to concerns raised during the FCC’s SCS Proceedings

Non-interference basis & case-by-case review. The Commission did not exclude PCS from SCS eligibility, but stressed that SCS is authorized only on a non-interference basis (domestic and cross-border). Being in an “SCS Band” does not guarantee an SCS grant; each application will undergo rigorous technical review. [3]

Reverse-band (PCS↔S-band) interference specifically analyzed. Citing Omnispace’s filings in the 1990–1995/1910–1915 MHz case, the FCC noted that the Space Bureau requested additional interference demonstrations from SpaceX, including a Monte Carlo constellation-level analysis; that analysis was submitted [3] and in November 2024, the FCC Space Bureau granted SpaceX authority to operate on these spectrum bands.  [4]

Aggregate OOBE/PFD protections for adjacent services. To protect adjacent terrestrial operations while enabling SCS, the Commission adopted an aggregate out-of-band PFD limit of −120 dBW/m²/MHz at 1.5 m AGL (including the PCS G-block band). Although this primarily addresses adjacent-band terrestrial protection (not the MSS uplink conflict), it reflects the FCC’s overall protective posture when introducing SCS. [3] In March 2025, the FCC Space Bureau granted SpaceX relaxation of these OOBE limits to -110.6 dBW/m²/MHz.[5]

Market-boundary protections unchanged. The FCC kept existing market-area field-strength limits for PCS (47 dBµV/m) and other mobile bands, applying them in aggregate to SCS beams at and beyond licensed areas. [3]

Other engineering levers. The Commission declined to set a universal minimum satellite elevation-angle requirement (system-dependent), kept terrestrial device power/OOBE limits for SCS earth stations, and left in-market PFD to private agreements between SCS partners—while reserving oversight. [3]

## A.3 Takeaways for the PCS→S-band (B256/n256) UL issue

The core interference concern Omnispace raised is the reverse-band conflict: a powerful satellite downlink in 1990–1995 MHz within the U.S. potentially desensing MSS satellite receivers outside the U.S. that are listening for uplinks around 1980–2010 MHz. The FCC acknowledges the risk and pushes it into case-by-case licensing with strict non-interference obligations and, in practice, heavy technical showings (e.g., constellation-level Monte Carlo) before any grant. [3]

The Commission did not adopt a blanket PCS carve-out for this reason, but it put applicants on notice that reverse-band proposals like PCS G-block SCS must demonstrate they will not cause harmful interference to MSS uplinks and that harmful interference must be halted immediately if it occurs under ITU RR 4.4. [3]

## A.4 SpaceX post-SCS NPRM filings

Monte-Carlo constellation analysis submitted & relied on by the Bureau. In granting SpaceX’s SCS authority, the Space Bureau states SpaceX “submitted a Monte Carlo analysis … that simulates its 7,500-satellite” direct-to-cell network and time-based statistics, and the Bureau relied on those materials in concluding SpaceX can hold interference below the harmful-interference threshold 100% of the time for Omnispace and other MSS uplinks when operating in 1990–1995 MHz. The Order cites SpaceX ex partes Nov 14, 2023 and Nov 30, 2023, a Feb 14, 2024 letter, and a June 20, 2024 response to an FCC information request as the technical record containing that analysis and supporting data. [4]

Follow-ups with requested data & patterns (late 2024). The Bureau documents additional information requests on June 7, 2024 and Sept 6, 2024 and SpaceX responses on June 20, 2024 and Sept 13, 2024—including reference antenna patterns and a CSV file of the specific pattern data points on Sept 18, 2024—to support the analysis. [3]

## A.5 Omnispace post-SCS NPRM filings

Measured-interference ex parte (spring/summer 2024). Omnispace filed measurements alleging 3–5 dB noise-floor rise at its MSS satellite receiver when Starlink D2D payloads were active over CONUS; it followed with a July 5, 2024 letter amplifying those concerns and arguing that SpaceX’s materials understated risk. [6]

## A.6 Current Status

The Space Bureau authorization (DA-24-1193A1, Nov. 26, 2024) is the clearest window into what was actually submitted and how it was weighed; it records the Monte-Carlo analysis and the subsequent data hand-offs and then explains why the Bureau concluded SpaceX can protect MSS uplinks in 1990–1995 MHz under the adopted conditions. [4]

Separately (edge-case but relevant to PCS borders), in March 2025 the FCC granted SpaceX a targeted OOBE/PFD waiver at −110.6 dBW/m²/MHz for the 5-MHz edges adjacent to its SCS downlink; that waiver is about adjacent-band terrestrial protection, not the reverse-band MSS-uplink issue, but it’s part of the same post-R&O record. [5]