3GPP TSG RAN Meeting #110	   RP-253702

Baltimore, USA, December 8-11, 2025

Title: 	Views on 6G RAN WG Study

Source: 	ZTE Corporation, Sanechips

Agenda item:	8.2.2

Document for:	Discussion and decision

- Introduction

In RAN#108, 6G SID “New SID: Study on 6G Radio” was approved [1].  The study has been kicked off in RAN1 in Q3 and RAN2/RAN3/RAN4 in Q4. Some progress has been made in WGs but some issues may need discussion in RAN. In this contribution, our views on the aspects of numerology, AI/ML and channel coding are provided.

- Issues from 6G RAN WG Study

  - Numerology

After three meetings of discussion in RAN1, the study on numerology for 6GR has made very good progress. Overall, the key agreements and pending issues for this topic can be summarized as following.

| Frequency range | Sub 6GHz | Around 7GHz | FR2-1 (24.25 GHz – 52.6GHz) |
| --- | --- | --- | --- |
| Subcarrier spacing | FDD 15kHz, TDD 30kHz FFS: 30kHz for FDD | 30kHz | 120kHz FFS: Numerology for SSB |
| Maximum channel bandwidth (Hz) | 100M for 15kHz SCS 200M for 30kHz SCS Note: Majority view, although hasn’t been agreed explicitly yet. | Network side: 400M UE side: 200M or 400MHz | 400M or 800M |
| Note: Green color text refers to agreements in RAN1; Red color text refers to pending issues in RAN1. |  |  |  |

One issue raised by companies in RAN1 that may require RAN guidance is whether to allow study of 30kHz SCS for FDD for Sub 6GHz in addition to 15kHz SCS. Based on our understanding, 30kHz SCS for sub 6GHz FDD is implementable from both network and UE perspective. The key concern from companies is 30kHz SCS may not be suitable for MRSS. Thus, it would be beneficial if operators can share their views on whether this is useful for the scenarios where clean refarming would be done without the need of MRSS. It does not belong to multiple options because this is for a different scenario.  From our perspective, 30kHz SCS for sub-6GHz FDD can be further studied to understand the benefits of such scenarios. The other pending issues (marked in red in the above table) can be studied in working group level. This includes the support of 400MHz at the UE side around 7GHz.

Proposal 1: 30kHz SCS for sub-6GHz FDD can be further studied in RAN1 for scenarios where there is no MRSS.

  - AI/ML - RAN1 use cases

After three meetings of discussion in RAN1, RAN1 has collected 41 AI/ML use cases, including 36 use cases on AI for network and 5 use cases on network for AI (e.g., AI/ML services and digital twin). From next meeting, RAN1 will distribute each AI/ML use case led by RAN1 to respective agenda, with the aim to conduct fair comparison between AI-based solution and non-AI based solution. However, some companies raised the concern on the workload in RAN1 due to such a large number of collected AI/ML use cases and proposed to perform the down-selection at RANP level. However, from our perspective, the down-selection at this stage is premature due to the following reasons:

- RAN1 still needs to more time to conduct comprehensive study for each AI/ML use case to evaluate its performance gain, complexity, generalization, etc., especially considering there are new AI/ML use cases firstly proposed in 6G;

- It doesn’t mean that all the AI/ML use cases will be studied until the end of Rel-20, each respective agenda can make their own decision on whether to continue the study of the corresponding AI/ML use case at long as sufficient study is conducted;

- The overall workload on AI/ML use cases can be alleviated and can be balanced among different topics since it is distributed into each respective agenda.

One potential way forward for this topic is to have a check point at RAN#112 (June of 2026). After 6 months study, RAN1 can better understand the overall situation of the study of all AI/ML use cases.

Observation 2: It is premature to down-select AI/ML use cases at this stage due to lack of sufficient study. Further study and future down-selection of AI/ML use cases can be kept in working group level.

  - Channel coding

During RAN #109 meeting, there were a lot of online and offline discussions trying to find a common understanding for the peak data rate for 6GR. Our view is that the peak data rate shall be at least 100 Gbps in DL and 50 Gbps in UL, which is supported by 32 companies including 11 operators from different regions in RAN #109 [2]. However, no consensus has been made on this as there was still different views from other companies. In our companion contribution in [3] this meeting, we propose an alternative of reporting peak data rate per carrier.

According to the working assumption in RAN1#122bis, the target peak data rate of 6G data channel coding is assumed to be 2 times of that defined in TR38.913[4]. However, the channel bandwidth around 7GHz (including the upper 6GHz band) has been agreed to be up to 400MHz at network side. This corresponds to a fourfold increase in supported maximum channel bandwidth compared with 5G FR1. Meanwhile, the endorsed peak spectral efficiency of IMT-2030 has been agreed to be twice that of IMT-2020 [5]. As a result, the IMT-2030 peak data rate per carrier/FR can be eight times of IMT-2020 at least for network side, as shown in Table 1. Therefore, significantly higher throughput is required by 6G communications, driven by increased spectral efficiency and bandwidth.

Agreement in RAN1#122bis - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

Table 1 Peak spectral efficiency and data rate per carrier

| Parameters | Requirement values |
| --- | --- |
| Maximum channel bandwidth | 4x of 5G FR1(Agreed, at network side) |
| Peak spectral efficiency | 2x of IMT-2020 (Agreed) |
| Peak data rate per carrier | 8x of IMT-2020 (Derived from 4x bandwidth and 2x peak spectral efficiency) |

- Observation 3: The maximum bandwidth around 7GHz (including upper 6GHz band) has been agreed as four times that of 5G FR1 and the peak efficiency of IMT-2030 has been agreed to be twice that of IMT-2020. The combination of these enhancements will result in 6G peak data rate per carrier that is eight times that of IMT-2020.

Channel coding, as a key physical layer technology in wireless communication, is a critical factor in realizing higher data rate requirements. In RAN1#122bis meeting, the following agreement was reached for 6GR data channel coding [6].

Agreement for data channel - For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH) - For 6G LDPC - Working assumption: For data rate within NR range, reuse of NR LDPC design is supported - For data rate beyond NR range, study LDPC extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Applicability of the potential LDPC extension to data rate within NR range will be further discussed

From our perspective, the necessity on channel coding extension does not solely depend on the IMT-2030 TPR of peak data rate. Extension to support more efficient channel coding is needed as long as it is beyond the capability of the current NR commercialization (e.g. peak data rate of commercial device types in NR) considering both UE and network sides. In addition, decoding complexity is much more critical while encoder implementation is simple enough which often involves software only. So, we should consider differently for downlink and uplink, i.e., decoder complexity at UE side and base station side respectively for DL and UL. Based on these considerations, we share our views on the understanding of data rate within/beyond NR range in the working assumption.

Downlink

From the perspective of downlink, we think it should be defined by DL bandwidth within a single carrier for a UE, rather than aggregated bandwidth across multiple carriers since not all device types support multiple carriers. To support immersive communication, which is one of the promising applications in 6GR, XR devices are designed to be wearable and user-friendly, while providing excellent user experience including immersive experience and long battery life. To this regard, 6G XR device needs to provide better throughput performance within limited form factor (hence limited chip area), making single-carrier operation a more practical and feasible choice. On the other hand, not all normal UEs support carrier aggregation in current commercial network, this is because normal UEs with function of multi-carrier aggregation are implemented/deployed in the later release instead of the first release during the 5G commercialization. In this case, 6G new LDPC design should be designed to ensure high energy efficiency, area efficiency and improve user experience while providing higher throughput in single carrier. For instances, 5G XR device peak data rate is 1Gbps in single carrier in mid-band (e.g., 3.5GHz, 100MHz) given 2Rx antennas. In our considerations, 6G XR device peak data rate can be increased to up to 4Gbps in single carrier in around 7GHz band as the increase of bandwidth (e.g., 400MHz). Therefore, 6G LDPC design should be designed to ensure implementation simplicity and reduce power consumption for XR devices so that 6GR can be more successful.

- Observation 4: NR data range should be considered as NR data rate supported per carrier which depends on capability of different device types and network implementation. To support portable devices for immersive communication, 6G channel coding should be designed to support high data rate with more efficient implementation and reduced power consumption for XR devices with strict form-factor limitations and area efficiency requirements.

- The data rate for a XR device (i.e. around 1Gbps in NR) should be enhanced to a higher data rate (e.g., 4Gbps) with the same form factor (i.e., similar chip area).

Uplink

From the perspective of uplink, decoder throughput of a base station depends on aggregated UL data rates of multiple UEs in a carrier. Therefore, cell throughput should be considered at base station side. 6G cell data rate can be much higher than 5G NR, as 3x IMT2020 average spectral efficiency and 4x IMT2020 higher area capacity have been agreed in RAN#109 for IMT-2030[7]. For uplink transmission, one UE usually cannot occupy a full bandwidth in a carrier. Therefore, it would lead to asymmetric throughput between base station and each UE. We provide some system level simulation results in Table 2. Accordingly, we can observe that even UE experience data rate for one UE may be moderate (e.g., 60Mbps), the cell data rate of base station is much higher in both single TRP and multi- TRP case.

Table 2 UE/BS throughput in single/multi-TRP deployment per carrier

| Bandwidth | Maximum # of layers per TRP or per cell-site | UE experienced throughput (Gbps) | BS experienced throughput (Gbps) |
| --- | --- | --- | --- |
| 200MHz | 48 | 0.06 | 3.814 |
| 200MHz | 48 | 0.06 | 48.32 (for 15 TRPs) |
| Note: the BS experienced throughput is based on realistic scheduling in system level simulation |  |  |  |

- Observation 5: To make sure efficient channel decoding processing for high cell throughput in network side for UL, it’s important to make sure all the 6G UEs are capable of new 6G channel coding even though each individual UE throughput is lower than 5G peak data rate.

- Whether a base station needs to switch to a new channel coding method (to support beyond its original NR capability) depends on its decoder resource per carrier and the network load which should be determined by the network itself.

Last but not least, for applicability of the 6G LDPC code, we think 6G LDPC code can be used within NR range along with acceptable performance-complexity trade-off. If a new decoder is implemented, it will be more cost/energy efficient to make the best out of the new decoder by covering NR range as well.

- Observation 6: Efficient 6G LDPC design with better performance-complexity trade-off should be applied within NR range.  If a new decoder is implemented, it is more cost/energy efficient to make the best out of the new decoder by covering NR range as well.

In RAN1#123, some progress on evaluation assumption for study of BG(s) and PCM(s) for LDPC extension has been reached. It is expected that companies will provide evaluation results to investigate into the benefits of different proposals on LDPC extension.  For example, some companies proposed to reduce number of iterations while maintaining similar performance.  Adpative number of iterations has been supported in the current network implementation.  However, there is a risk on performance loss with the current NR channel coding or the adaptive scheme needs to be very conservative.  Some proposals on LDPC extension actually ensure performance robustness when doing adaptation on the number of iterations for more efficient LDPC processing.  Applicability of LDPC extension on within NR range can be considered with the possibility of reuse NR LDPC design with configuration.  Switching mechanism can be further discussed when there is better understanding on benefits under different data region.

- Proposal 2: RAN1 study on channel coding should not depend on the progress of peak data rate discussion in RAN/ITU-R WP5D.

- Continue RAN1 study on data channel coding without the need of RAN intervention at least until the checkpoint in June 2025.

- Summary

In this contribution, our views on the identified issues are provided with the following observations and proposal:

Numerology

Proposal 1: 30kHz SCS for sub-6GHz FDD can be further studied in RAN1 for scenarios where there is no MRSS.

AI/ML- RAN1 use cases

Observation 2: It is premature to down-select AI/ML use cases at this stage due to lack of sufficient study. Further study and future down-selection of AI/ML use cases can be kept in working group level.

Channel coding

- Observation 3: The maximum bandwidth around 7GHz (including upper 6GHz band) has been agreed as four times that of 5G FR1 and the peak efficiency of IMT-2030 has been agreed to be twice that of IMT-2020. The combination of these enhancements will result in 6G peak data rate per carrier that is eight times that of IMT-2020.

- Observation 4: NR data range should be considered as NR data rate supported per carrier which depends on capability of different device types and network implementation. To support portable devices for immersive communication, 6G channel coding should be designed to support high data rate with more efficient implementation and reduced power consumption for XR devices with strict form-factor limitations and area efficiency requirements.

- The data rate for a XR device (i.e. around 1Gbps in NR) should be enhanced to a higher data rate (e.g., 4Gbps) with the same form factor (i.e., similar chip area).

- Observation 5: To make sure efficient channel decoding processing for high cell throughput in network side for UL, it’s important to make sure all the 6G UEs are capable of new 6G channel coding even though each individual UE throughput is lower than 5G peak data rate.

- Whether a base station needs to switch to a new channel coding method (to support beyond its original NR capability) depends on its decoder resource per carrier and the network load which should be determined by the network itself.

- Observation 6: Efficient 6G LDPC design with better performance-complexity trade-off should be applied within NR range.  If a new decoder is implemented, it is more cost/energy efficient to make the best out of the new decoder by covering NR range as well.

- Proposal 2: RAN1 study on channel coding should not depend on the progress of peak data rate discussion in RAN/ITU-R WP5D.

- Continue RAN1 study on data channel coding without the need of RAN intervention at least until the checkpoint in June 2025.

- Reference

- RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

- RP-252041, Views on IMT-2030 TPRs. ZTE Corporation, T-Mobile USA, SoftBank, CBN, China Broadnet, Telstra, Verizon, Qualcomm Inc., Charter Communications, China Telecom, Turkcell, Vestel, Kyocera Corporation, DeepSig Inc, CableLabs, UNISOC, Spreadtrum, Türk Telekom, ABS, Khalifa University, Shanghai Jiao Tong University, NERC-DTV Co. Ltd., Queens University Belfast, Ruijie Networks, TCL, Pengcheng Laboratory, Continental Automotive Technologies GmbH, Sanechips, InterDigital, VIAVI, Fujitsu, CHTTL. RAN#109

- RP-253193, Views on RAN KPIs. ZTE Corporation, Sanechips. RAN#109

- Chair notes of RAN1#122bis

- Meeting report of RAN#109