3GPP TSG-RAN Meeting #110	RP-253703

# Baltimore, USA, 8th Dec 2025 – 11th Dec 2025

Agenda item:	9.1.5

Source: 	ViaSat Satellite Holdings Ltd, IITH, WiSig, Omnispace,Terrestar

Title: 	Views on specifying TN frequency bands for NTN

Document Type:	Discussion

Document for:	Discussion

## 1.	Background

After studying the feasibility of satellite connectivity using 5G UEs in Rel-15 and Rel-16, 3GPP developed normative specifications in Release 17. The direct to device connectivity via satellite) is currently supported for following 3GPP based UEs:

- FR1 and FR2 handheld and non-handheld NR UEs

- FR1 eMTC handheld UEs

- FR1 NB-IoT handheld and non-handheld UEs

3GPP RAN4 WG has focused on satellite spectrum that has been globally identified for MSS. Given the successful development of 3GPP standards for satellite bands and the value proposition (resiliency, ubiquitous coverage, etc.) of direct to device connectivity via satellite service, administrations globally have been exploring the use of IMT terrestrial frequency bands on a secondary basis to complement/extend connectivity in areas with poor to no terrestrial coverage.

To formalize the global regulations pertaining to use of IMT frequencies for MSS, ITU-R has resolved [1] to study the feasibility of utilizing IMT frequency bands for MSS without causing harmful interference to services within the same frequency band as well as those in the adjacent frequency bands.

Figure 1:  Segment from [1]

Observation 1: Several NTN bands have been defined as part of 3GPP NR and IoT specifications that may be adjacent to frequency bands that are authorized for TN bands.

Observation 2: Given the interest in enabling more MSS spectrum, ITU-R has initiated a study to evaluate the feasibility of utilizing IMT frequency bands for MSS without causing harm to other existing MS and MSS services in the vicinity of those bands.

At RAN#109 meeting, some companies had introduced proposals to specify NTN bands for IMT frequencies. After several online sessions, ad-hoc sessions and offline deliberations, no consensus was reached during the meeting [2] on the proposed work item or a potential way forward. During offline deliberation at RAN#109 meeting, there was a general agreement among participants that if the work were to be kicked-off within 3GPP, that such bands will be subject to regulatory work currently in progress in ITU-R and that the 3GPP work should identify and develop requirements to mitigate coexistence issues related to use of TN bands for NTN.

Given that the companies were encouraged to provide additional views at future RAN TSG meetings related to this topic, this discussion paper provides additional background related to our views on specifying NTN operations via IMT terrestrial frequency bands. Furthermore, it proposes a way forward if 3GPP and proponents are unwilling to wait to begin the work until after the regulatory work within the ITU-R has been completed.

## 2.	Overview of sharing studies between MSS and MS

Over the years, ITU-R Working Parties 4C and 5D have conducted sharing studies across several frequency bands to evaluate co-channel and adjacent frequency band co-existence of mobile and mobile-satellite services in same and different geographic areas. We bring attention to two specific studies that ITU-R has performed related to sharing studies between MS and MSS in L-band (ITU-R Resolution 223) and S band (ITU-R Resolution 212). Both studies concluded that technical and operational measures may be needed for the two services to co-exist.

The conclusions from the study of L-Band co-existence [3] indicate a need for a i) guard band, ii) reduced IMT BS and UE unwanted emissions and/or iii) PFD levels appliable to IMT BS to protect NTN UEs.

The S-band study [4] concluded a need to implement technical and operational measures to protect MSS satellite receivers, including limiting transmit power from TN BSs if visible to the satellite (that is if the coverage areas of MS and MSS are not sufficiently separated by distance).

Observation 3: Prior ITU-R studies for L-band have shown the need for technical measures such as guard band and/or reduced power and emission levels for TN and NTN services in adjacent channel/band to co-exist in the same geographic area.

Observation 4: Prior ITU-R studies for S-band have shown the need for technical measures such as reduced transmission power and emission levels for the TN and NTN services in the same channel/band to co-exist in geographically separate areas.

## 3.	Unique aspects of S-Band IMT frequency bands for TN and its implications for NTN

Authorization of MSS on a secondary basis in S-band n2/n25 results in new co-existence scenarios that have never been considered previously within 3GPP. Authorization of MSS on a secondary basis in n2/n25 in some countries of Region 2 results in co-channel/adjacent channel BS being deployed in space (satellite access nodes) where free space propagation poses a far more substantial threat to NTN n256 receiver performance in regions/countries that are much farther away than studied by ITU-R [4] as part of the S-band study. This is because the DL of n2/n25 either overlaps or is adjacent to the UL of NTN band b256/n256 as shown in Figure 2 below and summarized below

Downlink of n2:  1930-1990MHz

Downlink of n25:  1930-1995MHz

Uplink of n256:  1980 – 2010MHz

Figure 2: Band n2/n25 frequency ranges and NTN – NTN co-existence scenario

During the regulatory proceedings related to supplemental coverage from space (SCS) in the US, one company had submitted preliminary measurements [5] and simulation results [6] that showed the potential of harm to n256 uplink operations several thousands of kms away if satellite access nodes (SANs) operating in n2/n25 frequency band were deployed to provide coverage over North America.

Figure 3: Measurement results of impact from n2/n25 based satellite operation to n256 operation over rest of the world [5]

Figure 4: Simulation results of impact from n2/n25 based satellite operation to n256 operation over rest of the world [6]

It is important to note that the above measurements and simulations are from a single company and may or may not accurately represent the aggressor and victim system parameters that various n256 NTN operators may be planning and/or deploying. However, these submissions do show a need for an independent and a thorough technical study within 3GPP to evaluate co-existence of NTN operation in n2/n25 frequency band with NTN operation in n256, and in the event there is performance degradation observed due to such an operation, determine transmit power limits and/or protection thresholds for n256 that will reduce the impact to acceptable levels (e.g., 5% reduction in 5%-tile uplink throughput performance).

Observation 5: There are evaluations from some companies that justify the need for 3GPP to perform its independent co-existence study to evaluate the impact of NTN operations in n2/n25 and n256.

During the same proceedings, studied also showed a need to specify a PFD levels for NTN operation in n2/n25 so as not to impact adjacent TN operations in n2/n25 in the same geographic area [7]. As shown in Figure 5below, inadequate ACLR and/or co-existence limits can negatively impact of adjacent TN operations.

As indicated below, there is a precedent of 3GPP developing co-existence requirements over and above the regulatory limits to ensure minimal impact on neighbouring frequency bands.

Figure 5: Impact of NTN operation in n2/225 to adjacent n2/n25 TN deployments [7]

Observation 6: Prior non-3GPP work seems to suggest the need for 3GPP to perform its independent co-existence study to evaluate the impact of NTN operations in frequency range of n2/n25 on TN operations in n2/n25.

Furthermore, these reversed use of frequencies ranges between North America (n2/n25 DL) and Europe (n1 UL) will be an issue if EU authorizes MSS in band n1. In fact, if care is not taken and technical measures are not specified for NTN band operating in the frequency range of n2/n25, it can foreclose the future use of n1 frequency range for NTN operation over several countries across Europe and Asia.

## 4.	Review of TN band n7 and the primary MSS allocation in 2500 MHz

Figure 6 depicts TN and NTN allocation in 2500 MHz. Like n2/n25, MSS band overlaps with 20 MHz of TN band n7.

Figure 6: Band n7 frequency ranges and NTN – NTN co-existence scenario

However, public information suggest that Telstra may be considering NTN deployment in 2500 – 2510 MHz / 2620 – 2630 MHz with its foreign satellite partner. If that is indeed accurate, then there seems to be adequate guard band between the two systems for the NTN SAN – NTN SAN co-existence to be not an issue. The UE – UE co-existence will be an issue only if Australia has authorized MSS in 2500 MHz within its territory, otherwise that is a non-issue.

Observation 7: If the definition of new NTN band in the TN n7 frequency range is limited to 2500 – 2510 MHz UL and 2620 – 2630 MHz DL and if it can be confirmed that ACMA has not authorized MSS in 2500 MHz within its territory then NTN – NTN co-existence may not be issue and such a bounded NTN band in a small portion of the TN n7 frequency range can be considered with input from n38/n41 terrestrial operators in the region.

## 4.	Co-existence in 3GPP RAN4 WG

Global and regional radio regulations are used by RAN4 WG in developing technical specifications. In addition to the radio regulations, co-existence assessments across frequency bands have been another important input into the development of technical standards within RAN4 WG for new bands and UE power classes. Co-existence assessments have been used to limit impact of newer bands and UE power classes in adjacent channel/bands operations to within acceptable limits.

Depending on deployment models, additional requirements are specified for both BS (e.g., co-location as well as spurious co-existence limits for BS [8, 9]) and UEs (e.g., additional spurious emissions limits for UEs [10, 11]).

Observation 8: 3GPP has previously performed co-existence studies and specified technical requirements to ensure that sufficient protection is available to adjacent channel/band operations.

Accordingly, prior to NTN band specifications, RAN4 performed detailed co-existence evaluation for NTN bands to specify SAN ACLR/ACS limits to ensure impact to adjacent TN operation is minimal [12]. Furthermore, to protect TN band UEs, spurious co-existence limits have been specified for NTN bands [13,14]. During Rel-17, given the sparse number of NTN bands and well-formed regulations for global MSS bands, to efficiently utilize the limited number of TUs allocated for NTN, NTN – NTN co-existence evaluation was relegated to Phase 2 and not performed. As part of 6G, in anticipation of potential future use of TN frequency bands for NTN, RAN4 has prioritized NTN – NTN coexistence study [15].

Observation 9: To efficiently use allocated TUs, during 5G due to well-formed global regulations for MSS bands, NTN – TN was prioritized and NTN-NTN was pushed out into the future.

Observation 10: As part of 6G, co-existence NTN – NTN scenario has been prioritized in anticipation of future use of TN frequency bands for NTN.

More recently, at the behest of n2/n25 operators in the US, 3GPP agreed to perform co-existence studies as part of the introduction of NTN band b252/n252 which resulted in specifying spurious co-existence limits that were significantly more stringent than required by the regulations [13, 14].

Observation 11: 3GPP has developed additional technical co-existence emissions requirements to protect adjacent TN band operations from NTN operations despite relaxed regulatory requirements.

Even after the band n252 was specified, n2/n25 operator reiterated their concerns in RAN#108 [16].

There is no reason to circumvent the time-tested approach of performing co-existence studies that has been applied to developing technical specifications for TN and NTN bands. It would be remiss on part of 3GPP to study co-existence scenarios to protect TN bands from NTN bands but not NTN bands in primary MSS bands from terrestrial IMT spectrum bands authorized for secondary MSS service.

Observation 12: There is no technical reason to not perform co-existence studies to evaluate the impact of TN IMT frequency bands when deployed as NTN band on the existing NTN bands (e.g., n256).

Observation 13: If MSS operation in IMT band n1 is authorized by regional regulators globally then lack of protection from NTN operation in n2/n25 frequency range could jeopardize the use IMT band n1 as a future NTN band across several European, Asian and South American countries.

## 5.	WRC-27 timing and efficient use of 3GPP resources

As part of the WRC-27, ITU-R is expected to develop regulations on the use of terrestrial IMT bands for NTN. Ideally, to avoid duplicative studies and efficiently use limited 3GPP resources, it is preferable to wait for the studies to complete in ITU-R and wait for guidance from ITU-R before starting 3GPP specific co-existence studies as part of the work item.

Observation 14: WRC-27 is expected to develop radio regulations related to the use of terrestrial IMT frequency bands for NTN.

We also recognize that the WRC-27 timeline might be too long a time duration to wait for some of the proponents who have regulatory requirements associated with the deployment of their terrestrial IMT spectrum. Even though RAN4 lacks additional TUs for initiating new non-spectrum work such as co-existence studies, given that the NTN-NTN co-existence scenario for S-band has already been prioritized for 6G co-existence studies during the RAN4#117 meeting [15], the co-existence study can be kicked off as part of 6G NTN-NTN co-existence agenda in RAN4#118 meeting. RAN could specify a checkpoint for December 2026 to evaluate the progress within RAN4 and ITU-R with the goal of initiating a Rel-20 spectrum WI with appropriate objectives.

Observation 15: RAN4 has already prioritized NTN-NTN scenario as part of 6G NTN co-existence studies and relevant non-spectrum work can start for assessing the impact of specifying NTN bands in the TN frequency ranges.

Observation 16: RAN TSG can specify a checkpoint can be specified for December 2026 to evaluate the study progress and initiating a spectrum work item with appropriate objectives.

## 6. Guidance to RAN4

RAN can provide guidance to RAN4 to prioritize the following study scenarios as part of the 6G NTN-NTN co-existence scenario:

- Impact of NGSO constellation deployed in the frequency range of n2/n25 (to provide service in one region) on a GSO system deployed in n256 (to provide service in another region)

  - Both co-channel and adjacent channel deployments scenarios to be considered with priority for co-channel evaluation

- Impact of NGSO constellation deployed in the frequency range of n2/n25 (to provide service in one region) on a NGSO constellation deploy in n256 (to provide service in another region)

  - Both co-channel and adjacent channel deployment scenarios to be considered with priority for co-channel evaluation

- Impact of NGSO constellation deployed in the frequency range of n2/n25 on a TN system deployed in an adjacent channel providing service in the same geographic coverage area

RAN4 can be further requested to conduct studies using realistic (feasible to implement a system with) and acceptable parameters for constellation size, deployment altitude of the satellite constellation, EIRP, G/T, beam patterns, authorized service area of both aggressor and victim systems

## 7. Conclusions

Due to the global nature of NTN systems, it would be prudent for 3GPP to wait for clarity on regulatory guidance from ITU-R before specifying TN bands for NR/IoT NTN.

Alternatively, if the wait for ITU-R work to complete in WRC-27 is deemed to be too long, RAN4 can perform co-existence study under the umbrella of 6G co-existence SI with a checkpoint in December 2026 to evaluate a kick-off for a corresponding spectrum work item with appropriate objectives.

RAN cans liaise with RAN4 to prioritize a set of co-existence scenarios (identified in Section 6) during 2026.

With respect to n7, if n38/n41 TN operators of Region 3 are ok and if the proponents are willing to limit the frequency range of n7 for the new NTN band such that it does not overlap and has adequate guard band with the primary MSS allocation such that co-existence studies are not required, then a spectrum work item to introduce a new NTN band for a small subset of the frequency range of n7 could be approved.

For convenience, we repeat 4 key observations from the paper here:

Observation 5: There are evaluations from some companies that justify the need for 3GPP to perform its independent co-existence study to evaluate the impact of NTN operations in n2/n25 and n256.

Observation 11: 3GPP has developed additional technical co-existence emissions requirements to protect adjacent TN band operations from NTN operations despite relaxed regulatory requirements.

Observation 15: RAN4 has already prioritized NTN-NTN scenario as part of 6G NTN co-existence studies and relevant non-spectrum work can start for assessing the impact of specifying NTN bands in the TN frequency ranges.

Observation 16: RAN TSG can specify a checkpoint can be specified for December 2026 to evaluate the study progress and initiating a spectrum work item with appropriate objectives.

## 8.		References

[1]	https://www.itu.int/dms_pub/itu-r/oth/0c/0a/R0C0A0000110013PDFE.pdf

[2]	RP-252971, Draft meeting report for TSG RAN meeting #109

[3]	ITU-R M.2529, Adjacent band compatibility studies of IMT systems in the mobile service in the band 1492-1518 with respect to systems in the mobile-satellite service in the frequency band 1518-1525 MHz

[4]	Annex to ITU-R Resolution 212 (WRC-19), Implementation of International Mobile Telecommunications in the frequency bands 1885-2025 MHz and 2110-2200 MHz

[5]	Omnispace filing under GN Docket No. 23-65, May 17, 2024 (https://www.fcc.gov/ecfs/document/105172055305971/1)

[6]	Omnispace filing under GN Docket No. 23-65, August 30, 2024 (https://www.fcc.gov/ecfs/document/1083120851173/1)

[7]	Analysis: SpaceX’s proposal to increase power beyond FCC limits for secondary SCS will interfere with AT&T’s Adjacent primary PCS C Block operations, AT&T, October 2024 (https://www.fcc.gov/ecfs/document/10071342522017/3)

[8]	TS 36.104, Clauses 6.4.4.3, 6.4.4.4

[9]	TS 38.104, Clauses 6.6.5.2.3, 6.6.5.2.4

[10]	TS 36.101, Clause 6.6.3.2

[11]	TS 38.101-1, Clause 6.5.3.2

[12]	TR 38.863, Clause 6

[13[	TS 36.102, Clauses 6.5A.4.3, 6.5B.4.3

[14]	TS 38.101-5, Clause 6.5.3.2

[15]	R4-2522454, WF on 6G BS RF and co-existence, Feature Lead (Ericsson)

[16]	RP-251754, Concerns about co-existence between NTN band n252 and n2/n25, T-Mobile