3GPP TSG RAN Meeting #110		RP-253709

Baltimore, USA, December 8 – 11th 2025

Agenda item:	9.1.5.2

Source:	Apple

Title:	Regulatory overview of satellite communication in terrestrial band frameworks

Release:	Rel-20

Document for:	Discussion and Decision

# 1	Introduction

The satellite bands can be classified into the so-called mobile satellite services (MSS) and fixed satellite service (FSS), whereupon the former class of MSS bands can be used with mobile handheld devices equipped with omnidirectional antennas and the FSS bands are mostly for the fixed stations that are typically equipped with directional antennas using mechanical or electrical steering. One of the most widely used FSS bands are so-called Ku- and Ka-bands total bandwidth of which is in the order of several gigahertz of spectrum resources. On the contrary to it, the MSS bands are much smaller with spectrum in the order of tens of megahertz. And since the handheld devices are mobile by nature and are usually equipped with omnidirectional antennas operating in the lower frequencies corresponding to the MSS bands, it becomes quite crucial to ensure that there is enough spectrum to provide a wide range of services for end users via satellite links.

In response to the problem of insufficient spectrum in MSS bands, Administrations and other regulatory bodies started looking into this issue trying to identify more spectrum resources for the satellite communication specifically for the handheld type devices. The outcome of these studies is a so-called "satellite communication in terrestrial bands" framework already established by several Administrations that effectively allows using the existing terrestrial band spectrum to perform satellite communication. Thus, in this discussion paper we present further details of existing frameworks and outline the aspects that could be relevant for further considerations by 3GPP.

# 2	Satellite communication in terrestrial bands

## 2.1	General considerations

This section aims to provide a general overview of the existing regulatory frameworks that allow satellite communication in terrestrial bands. Acknowledging the fact that each regulatory document is not small and covers a wide range of different aspects, our summary is constrained to several items which to our understanding are most relevant to 3GPP and/or were extensively discussed during the previous meetings:

-	Frequency ranges in which it is possible to perform satellite communication

-	Additional emission requirements (if any) including the PFD limits

-	Co-existence with existing services

-	International developments and coordination

In the following subsections we present a summary of decisions made by US FCC, Australian ACMA, Canadian ISED and UK Ofcom, following the chronological order when the corresponding decisions were officially released.

## 2.2	US FCC

On March 2023 the Federal Communications Commission (FCC) issued a Notice of Proposed Rulemaking seeking comment on a proposed regulatory framework necessary to enable so-called Supplemental Coverage from Space (SCS) [1].  In particular, in this Notice the FCC surveyed spectrum blocks used by the terrestrial wireless service providers to facilitate the development of satellite-based connectivity. And on March 2024, the Federal Communications Commission adopted a Report and Order (R&O) establishing a domestic regulatory framework to enable expanded satellite-based coverage to a terrestrial licensee’s subscribers [2]. The key points from the R&O can be summarised as follows:

-	Adopted a secondary, bi-directional, mobile-satellite service allocation in certain frequency bands.

-	Established a framework in which the Commission would authorize SCS in bands where one or more terrestrial licensees lease access to the terrestrial spectrum rights to a satellite operator.

-	Imposed technical rules and other recommendations to mitigate potential harmful interference to existing services, including radio astronomy.

-	Clarified international coordination obligations, including outlining steps to ensure that SCS operations will be consistent with relevant ITU Radio Regulations

On the list of spectrum blocks for the SCS operation, the FCC adds a secondary "satellite" allocation in the following bands allocated to terrestrial services, along with a new non-federal footnote NG33A indicating that satellite operation in these bands is subject to the Commissions SCS rules.  Table 2.2-1 below summarises frequency ranges with an exemplary mapping to the existing 3GPP terrestrial bands already defined for the same ranges (we note that the FCC identified an additional spectrum block which does not have a corresponding 3GPP band, and it is labelled as NONE in the table below).

Table 2.2-1: Summary of bands authorised by US FCC

| Acronym | Range | 3GPP range |
| --- | --- | --- |
| 600MHz | 614-652MHz | n71 663-698MHz 612-652MHz |
|  | 663-698 MHz |  |
| 700MHz | 698-769MHz | n12 699-716MHz 729-746MHz n85 698-716MHz 728-746MHz n29 717-728MHz (SDL) |
|  | 775-799MHz | n13 777-787MHz 746–756MHz n14 788-798MHz 758-768MHz |
|  | 805-806 MHz |  |
| 800MHz | 824-849MHz | n26 814-849MHz 859-894MHz n5 824-849MHz 869-894MHz |
|  | 869-894 MHz |  |
| Broadband PCS | 1850-1915MHz | n2 1850-1910MHz 1930-1990MHz n25 1850-1915MHz 1930-1995MHz |
|  | 1930-1995 MHz |  |
| AWS-H | 1915-1920MHz | NONE |
|  | 1995-2000MHz |  |

US FCC framework for the satellite communication in the terrestrial bands does not put any specific or additional requirements on the terrestrial transmitter side, i.e. "the user devices connected to SCS are expected to operate with the same transmitters and receivers used to connect within their terrestrial network" and the existing FCC rules for terrestrial communication provide appropriate transmit power and OOBE limits for terrestrial devices that will access this service. As for the satellite transmitter side, the Commission decision is to establish a simple requirement for protection of the existing terrestrial deployments by adopting a single PFD limit of  -120dBW/m2/MHz for all bands defined in Table 2.2-1 (noting an additional waiver from FCC [3] on the upper blocks of the broadband PCS band).

On the international coordination, the Commission explicitly noted that "although the operations will be consistent with the U.S. Table, these operations will not conform with the International Table, and resolution of cross-border interference will be governed by ITU Radio Regulation Article No. 4.4". The Commission view is that such an approach framework ensures that we comply with our rights and obligations arising out of our ITU treaty commitments and protects incumbent services in other countries operating in conformance with the ITU Radio Regulations.

## 2.3	Australian ACMA

On September 2024 the Australian regulator ACMA released a document titled "Operation of an IMT satellite direct-to-mobile service; Regulatory guide" [4] describing the principles underlying the ACMA’s approach to enabling satellite communication in terrestrial bands. According to the ACMA regulations, the 700MHz, 800/850/900MHz and 2.5GHz bands are suitable for satellite communication in terrestrial bands, which are already licensed to operators under Australia wide spectrum licenses. As also mentioned by the regulator, 1800MHz and 2GHz bands could also support this service, but these bands are not licensed for the country wide usage and thus are not considered by ACMA as the priority bands for the satellite communication. Table 2.3-1 provides a summary of potential bands in Australia with mapping to the 3GPP terrestrial bands (where 1800MHz and 2GHz are mentioned for the sake of consistency with the understanding that these bands are "shaded" in the ACMA document).

Table 2.3-1: Summary of bands authorised by Australian ACMA

| Acronym | Spectrum license area | 3GPP range |
| --- | --- | --- |
| 700MHz | Australia wide | n28 703-748MHz 758-803MHz |
| 800/850/900MHz | Australia wide | n5 824-849MHz 869-894MHz n8 880-915MHz 925-960MHz |
| 1800MHz | Metro and regional areas | n3 1710-1785MHz 1805-1880MHz |
| 2GHz | Metro and regional areas | n1 1920-1980MHz 2110-2170MHz |
| 2.5GHz | Australia wide | n7 2500-2570MHz 2620-2690MHz |

Australian regulations for satellite communication in terrestrial bands do not define any additional emission requirements "provided the emissions remain within the parameters of the licence conditions". In other words, all existing terrestrial requirements for UE and BS would apply.

On the international developments and coordination, the ACMA document explicitly notes ongoing discussion on the WRC27 ITU-R AI 1.13 as a means of providing regulatory legitimacy for IMT satellite direct-to-mobile communications. And until WRC27 decision is made, conformity with Article 4.4 and other provisions of the ITU Radio Regulations is the responsibility of the foreign satellite operator and the administration that filed the satellite system with the ITU.

## 2.4	Canadian ISED

On February 2025, the Canadian regulator ISED released a document [5] establishing a framework for using bands allocated to flexible use and/or commercial mobile services to support the expansion of coverage via satellite, which is referred to supplemental mobile coverage by satellite (SMCS). There is also a complementary document released on May 2025 that provides additional technical requirements for space Stations providing supplemental mobile coverage by satellite [6].

Referring to the corresponding documents from ISED, there are several bands allocated for this service summarised below in Table 2.4-1 with additional information on the matching 3GPP range and band. When compared to the US FCC bands allocated for SCS, the only major difference is that ISED also allocated the AWS-1 and AWS-3 bands, which are not present on the US FCC list.

Table 2.4-1: Summary of bands authorised by Canadian ISED

| Acronym | Range | 3GPP range |
| --- | --- | --- |
| 600MHz | 617-652 MHz | n71 663-698MHz 612-652MHz |
|  | 663-698 MHz |  |
| 700MHz | 698-756 MHz | n12 699-716MHz 729-746MHz n85 698-716MHz 728-746MHz n29 717-728MHz (SDL) |
|  | 777-787 MHz | n13 777-787MHz 746–756MHz n14 788-798MHz 758-768MHz |
| 800MHz | 824-849 MHz | n26 814-849MHz 859-894MHz n5 824-849MHz 869-894MHz |
|  | 869-894 MHz |  |
| Broadband PCS | 1850-1915 MHz | n2 1850-1910MHz 1930-1990MHz n25 1850-1915MHz 1930-1995MHz |
|  | 1930-1995 MHz |  |
| AWS-1 | 1710-1755 MHz | n66 1710-1780MHz 2110-2200MHz |
|  | 2110-2155 MHz |  |
| AWS-3 | 1755-1780 MHz |  |
|  | 2155-2180 MHz |  |

Like the US and Australian regulators, ISED decided not to introduce any additional requirements for the terrestrial transmitter. And as for the satellite transmitter, it was decided to introduce the PFD limits summarised in the table below. Unlike the US FCC decision to adopt a single PFD threshold for the adjacent channel emissions, a more detailed approach was taken by ISED whereupon different PFD thresholds are used depending on whether it is an adjacent or a co-channel deployment, which is further differentiated between bands.

Table 2.4-2: ISED PFD thresholds for the satellite transmitter.

| Band acronym | PFD threshold dBW/m2/MHz |  |
| --- | --- | --- |
|  | Adjacent channel | Co-channel |
| 600MHz | -120 | -116 |
| 700MHz |  |  |
| 800MHz |  |  |
| Broadband PCS | -113 | -106 |
| AWS-1 |  |  |
| AWS-3 |  |  |

Since there are currently no international provisions for the Canadian SMCS framework, the ISED frequencies allocated for SMCS will be filed under No. 4.4 of the ITU Radio Regulations (RR). This approach is expected to continue for the foreseeable future, pending decisions at WRC-27. ISED will continue monitoring international developments, including the progress of agenda item 1.13 for WRC-27, and may adjust spectrum policies and the SMCS framework, as appropriate.

## 2.5	UK Ofcom

On October 2025, the UK regulator Ofcom released a document defining an authorisation framework that will enable end users to lawfully connect to the satellite services [7]. Ofcom’s mission is to make communications work for everyone, wherever they are in the UK. So, satellite services operating in mobile spectrum have the potential to improve the coverage and reliability of mobile services, particularly in rural areas where delivering a signal from a terrestrial mobile mast is more challenging. They allow existing mass market mobile handsets to connect via satellite when a terrestrial mobile signal is not available.

According to the Ofcom decision the following terrestrial bands will be authorised for the satellite communication. It is also noted that some bands, including 1.4 GHz and 2.1 GHz, will require additional technical analysis and will potentially require consultation prior to authorisation.

Table 2.5-1: Summary of bands authorised by UK Ofcom

| Acronym | UE Tx/ BS Rx | UE Rx/ BS Tx | Duplex | 3GPP range |
| --- | --- | --- | --- | --- |
| 700MHz | 703-733MHz | 758-788MHz | FDD | n28 703-748MHz 758-803MHz |
|  |  | 738-758MHz | SDL | n67 738-758MHz |
| 800MHz | 832-862MHz | 791-821MHz | FDD | n20 832-862 791-821MHz |
| 900MHz | 880.1-914.9MHz | 925.1-959.9MHz | FDD | n8 880-915MHz 925-960MHz |
| 1400MHz |  | 1452-1492MHz | SDL | n75 1432-1517MHz |
| 1800MHz | 1710.1-1785MHz | 1805.1-1880MHz | FDD | n3 1710-1785MHz 1805-1880MHz |
| 2.1GHz | 1920-1979.1MHz | 2110.3-2169.7MHz | FDD | n1 1920-1980MHz 2110-2170MHz |
| 2.6GHz | 2500-2570MHz | 2620-2690MHz | FDD | n7 2500-2570MHz 2620-2690MHz |

On the unwanted emissions and PFD thresholds, the UK Ofcom also adopted additional thresholds for unwanted emissions outside the wanted channel as presented in Table 2.5-2 below. And somewhat like the approach taken by the Canadian ISED, the PFD threshold is not fixed across all bands but rather is tailored to the specific band.  It is also worth noting that UK Ofcom view is that these PFD thresholds should be also sufficient for co-existence with existing services in the adjacent channels.

Table 2.5-2: Ofcom PFD thresholds for unwanted emissions from the satellite transmitter.

| Band acronym | PFD threshold dBW/m2/MHz |
| --- | --- |
| 700MHz | -119 |
| 800MHz |  |
| 900MHz |  |
| 1400MHz | -113 |
| 1800MHz | -111 |
| 2100MHz |  |
| 2600MHz | -108 |

UK Ofcom also pays special attention to the internation coordination, conclusion for which is identical to what other Administrations said: whilst there are no specific international regulations currently in place for D2D, any such service must operate on a ‘non-interference, non-protection’ basis under Article 4.4 of the Radio Regulations. UK Ofcom is actively engaged in the international working groups preparing for WRC, including the studies considering coexistence between D2D satellite services and terrestrial mobile services, and remains committed to reviewing our authorisation framework following the outcome of WRC-27.

# 3	Further considerations for 3GPP

Based on the overview of frameworks already established by several Administrations, summary of which was presented in the previous section, our understanding is that there should be no technical obstacles for enabling satellite communication in terrestrial bands at least in those countries where this type of operation has been already authorised. While during the previous RAN#109 meeting a number of questions were raised regarding international developments and issues, such as cross-border coordination, all Administrations allowing satellite communication in terrestrial bands explicitly note that:

-	current satellite communication in terrestrial bands will be performed under Article 4.4 of the ITU Radio Regulations known as "no-interference, no-protection" basis meaning that the service must not cause harmful interference to other services that are operating on their designated frequencies and if the service experiences interference from other services that are operating within their designated frequencies, the operator cannot claim protection or raise related concerns

-	satellite operators are expected to coordinate their constellations and resolve any cases of interference that may arise between different constellations

-	existing cross-border coordination rules are applied, and it is expected that any harmful interference in the territories of other administrations will be mitigated

-	ongoing developments in WRC27 AI 1.13 will be followed whereupon domestic arrangements will be reviewed after a decision is made on this matter

We also believe that 3GPP can take a closer look, if needed, at the following technical aspects that are relevant for the 3GPP NTN based satellite communication.

-	Band plan. Administrations have clearly indicated which frequency ranges, and even frequency arrangements, are authorised for the satellite communication in terrestrial bands. In most cases there is a direct mapping to the corresponding 3GPP bands, but it can be double-checked to ensure that we have a proper support for considered frequency ranges. Furthermore, it should be also understood that depending on the country and the region we may have different bands authorised for the satellite communication. For instance, while Canadian ISED authorised AWS-1 and AWS-3, these bands are not on the US FCC list. Similarly, while both Canada and US consider the 600MHz for this type of operation, Australian ACMA does not mention that range at the same considering other frequencies, such as 2.5GHz.

-	Co-existence. At least our understanding is that potential co-existence issues with other services (or even mobile terrestrial services in the neighbouring bands) have been considered during the consultation phase when Administrations were considering which band could be authorised for this type of service. In fact, this is one of the reasons why not all the existing terrestrial bands are mentioned in the final decisions. Nevertheless, 3GPP can perform a survey of potential co-existence scenarios for the considered bands.

-	Emission and PFD limits. The prevailing approach in the existing frameworks is that existing emission limits from the corresponding terrestrial bands are applied while operating the satellite service in the terrestrial spectrum. There are, however, minor differences depending on the country, including limits and handling of co-channel and adjacent channel operation.  3GPP can review and consider this information.

# 4	Conclusions

In this discussion paper we have presented an overview of the existing frameworks established by several Administrations to enable satellite communication in terrestrial bands. Referring to the corresponding regulatory documents, our understanding is that there are no fundamental technical obstacles in performing satellite communication in the terrestrial bands which have been authorised for this type of operation by the corresponding Administrations (noting that exact list of terrestrial bands may vary depending on the country). More specifically, Administrations assume that this type of service will be performed under ITU Article 4.4 considering other regulations and cross-border coordination rules applicable to the given country. It is also worth noting that Administrations follow ongoing development of WRC27 AI 1.13 topic, outcome of which will be incorporated into the domestic rules.

Since most Administrations just define a list of frequencies or ranges authorised for the satellite communication in terrestrial bands, 3GPP can:

- document the band plan and relevant regulation, as requested by operator(s);

- perform a survey of co-existence scenario; and

- consider additional emission and PFD limits, if any.

Proposal:	We kindly ask 3GPP to consider the presented information to progress the discussion on satellite communication in terrestrial bands based on the 3GPP NTN technologies.

# 4	References

US FCC, "Single Network Future: Supplemental Coverage From Space; Space Innovation", Notice of Proposed Rulemaking, IB Docket No. 22-271, March 2023

US FCC, "Single Network Future: Supplemental Coverage from Space ", REPORT AND ORDER AND FURTHER NOTICE OF PROPOSED RULEMAKING, IB Docket No. 22-271, March 2024, URL: https://docs.fcc.gov/public/attachments/FCC-24-28A1.pdf

US FCC, "Application for Authority for Modification of the SpaceX NGSO Satellite System to Add a Direct to Cellular System", March 2025, URL: https://docs.fcc.gov/public/attachments/DA-25-197A1.pdf

ACMA, "Operation of an IMT satellite direct-to-mobile service", September 2024, URL: https://www.acma.gov.au/sites/default/files/2025-05/Regulatory%20guide_Operation%20of%20an%20IMT%20satellite%20direct-to-mobile%20service_0.pdf

ISED, "Decision on a Policy, Licensing and Technical Framework for Supplemental Mobile Coverage by Satellite", February 2025, URL: https://ised-isde.canada.ca/site/spectrum-management-telecommunications/en/learn-more/key-documents/consultations/decision-policy-licensing-and-technical-framework-supplemental-mobile-coverage-satellite

ISED, "SRSP-103 Technical Requirements for Space Stations Providing Supplemental Mobile Coverage by Satellite", May 2025, URL: https://ised-isde.canada.ca/site/spectrum-management-telecommunications/en/devices-and-equipment/standard-radio-system-plans/srsp-103-technical-requirements-space-stations-providing-supplemental-mobile-coverage-satellite

Ofcom, "Enabling satellite direct to device services in mobile spectrum bands", October 2025, URL: https://www.ofcom.org.uk/siteassets/resources/documents/consultations/category-1-10-weeks/consultation-enabling-satellite-direct-to-device-services-in-mobile-spectrum-bands/main-documents/statement-and-further-consultation-enabling-satellite-direct-to-device-services-in-mobile-spectrum-bands.pdf