3GPP TSG RAN Meeting #57                                                       RP-121399

# Chicago, USA 4-7 September, 2012                                     (revesion of RP-121292)

# Source:	Broadcom Corporation

# Title:	Study on Enhanced Interference Suppression for LTE

# Document for:	Approval

Agenda Item:	13.2

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.

# Title *: Study on Enhanced Interference Suppression for LTE

## Acronym * : FS_LTE_ISenh

## Unique identifier * 5700xy

## 1	3GPP Work Area *

| X | Radio Access |
| --- | --- |
|  | Core Network |
|  | Services |

## 2	Classification of WI and linked work items

### 2.0	Primary classification *

This work item is a … *

| X | Study Item (go to 2.1) |
| --- | --- |
|  | Feature (go to 2.2) |
|  | Building Block (go to 2.3) |
|  | Work Task (go to 2.4) |

### 2.1	Study Item

| Related Work Item(s) (if any] |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

### 2.2	Feature

| Related Study Item or Feature (if any) * |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

### 2.3	Building Block

| Parent Feature (or Study Item) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

This work item is … *

|  | Stage 1 (go to 2.3.1) |
| --- | --- |
|  | Stage 2 (go to 2.3.2) |
|  | Stage 3 (go to 2.3.3) |
|  | Test spec (go to 2.3.4) |
|  | Other (go to 2.3.5) |

#### 2.3.1		Stage 1

| Source of external requirements (if any) * |  |  |
| --- | --- | --- |
| Organization | Document | Remarks |

Go to §3.

#### 2.3.2		Stage 2  *

| Corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other source of stage 1 information |  |  |
| --- | --- | --- |
| TS or CR(s) | Clause | Remarks |

If no identified source of stage 1 information, justify: *

Go to §3.

#### 2.3.3		Stage 3 *

| Corresponding stage 2 work item (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Else, corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other justification |  |  |
| --- | --- | --- |
| TS or CR(s) Or external document | Clause | Remarks |

If no identified source of stage 2 information, justify: *

Go to §3

#### 2.3.4		Test spec *

| Related Work Item(s) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

Go to §3.

#### 2.3.5		Other *

| Related Work Item(s) |  |  |  |
| --- | --- | --- | --- |
| Unique ID | Title | Nature of relationship | TS / TR |

Go to §3.

### 2.4		Work task *

| Parent Building Block |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

## 3	Justification *

Interference reduction is seen as the final frontier for improving the system performance especially for users at the cell edge in a single frequency network. Interference rejection methods at the UE receiver have been in discussion with the standards group, e,g, MMSE-IRC. These methods exploit the structure of the interference as seen by the UE receiver.

LTE standard defines several different transmission modes (TM) which can be configured on per user basis. TM1 and TM2 apply the same complex transmission weight on data and cell specific pilots on per transmit port basis. TM4 and TM6 use pre-coding of data tones with PRB and user specific pre-coder matrix of a given rank. TM3 circulates a fixed set of predefined pre-coders of a given rank across data tones within given user allocation. TM5, TM7, TM8 and TM9 apply beamforming of a given rank to both data and user specific pilot tones within given user allocation. Furthermore, due to Cell-ID specific shift of pilot tone positions, pilot (data) tones of serving eNodeB may or may not overlap with pilot (data) tones of interfering eNodeB, even if the eNodeBs are time synchronized. The standard also in general does not require FDD LTE eNodeBs to be synchronized in time, hence interference measured on pilots may include contribution of both pilot and data tones of interfering eNodeBs. Very few combinations of TMs of interfered and interfering users allow for meaningful interference suppression processing at the interfered user receiver.

Applying interference suppression for undesired combination of TMs can lead to limited performance gains as compared to non-interference suppression processing (baseline receiver). The current state of the Release 11 LTE standard doesn’t guarantee exploiting the interference suppression processing at the user equipment (UE) to a full extent. The interference rejection receiver has been envisioned to operate with no aid from the network. Further enhancements to inter-cell interference mitigation at the receiver side can be achieved by increasing the degree of co-operation between the transmitter and the receiver.

This study item proposes to provide additional assistance for the receiver, increase the collaboration between transmitters for providing assistance to UEs, and utilize more collaboration between transmitters and the receivers. The assistance can be provided in subframe level\ in which a set of eNodeBs, causing interference to corresponding UEs, may allocate resources jointly for their transmissions. These eNodeBs collaborate in their scheduling and may provide additional information to UEs as part of enhanced interference suppression. Such assistance could be in the form of scheduling different UEs with favourable TM combinations by participating eNodeBs. This allows the UEs to reliably estimate the interference (or interference statistics) to aid in successfully applying interference suppression receiver leading to maximum performance improvement.

Advanced receivers with improved interference mitigation capabilities and network assistance can be used to better mitigate the inter-cell interference when the UE is close to the cell edge, or the intra-cell interference originating from MU-MIMO transmissions when the UE is in the inner part of the cell.

The proposed study will focus on aspects of interference suppression and to examine the signalling schemes needed to enable the UE to achieve the full benefits of interference cancellation. The proposed study also considers aspects of utilizing interference suppression to many practical deployment scenarios (e.g., intra-cell MU-MIMO, CoMP, heterogeneous network, and small cell), and related system design enablers (i.e., network signalling assistance and enhanced link adaptation).

## 4	Objective *

The objectives of the study item are:

- Identify the deployment scenarios  where advanced receivers can benefit from the network assistance or coordination

  - Evaluation should consider many practical deployment scenarios (e.g., intra-cell MU-MIMO, CoMP, heterogeneous network, and small cell)

  - Evaluation should take into account the available backhaul and other network coordination means for interference suppression

- Identify the network parameters including TM combinations to be used in joint interference suppression mode that lead to the largest throughput benefit from interference suppression

  - Evaluation may consider different network parameters including TM combinations, Synchronization, Modulation, related system design enablers (i.e., network signalling assistance and enhanced link adaptation)

- Study and evaluate feasibility and potential gain with interference suppression

  - Evaluate the link level simulation under the TM combinations

  - Evaluate the system level sector throughput

  - Identify practical receiver(s) that can be considered in interference suppression mode  for system-level evaluation and system impact investigation

  - Practical network deployment and UE implementation impairments (e.g., synchronization offset, propagation delay, UE channel estimation error, demodulation/decoding error) should be considered in the evaluation

- Develop the efficient signalling structure and required information to be exchanged to enable interference suppression

- Evaluate the feedback overhead and backhaul required to enable network assistance interference suppression

- Identify the specification impact with respect the assistance information or to network coordination aspects, enabling efficient interference cancelation.

The study item needs to cover both synchronized and unsynchronized networks and both TDD and FDD frame structures.

## 5	Service Aspects

No impact.

## 6	MMI-Aspects

No impact.

## 7	Charging Aspects

No impact.

## 8	Security Aspects

No impact.

## 9	Impacts *

| Affects: | UICC apps | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  | X |

## 10	Expected Output and Time scale *

| New specifications * [If Study Item, one TR is anticipated] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title |  | Prime rsp. WG | 2ndary rsp. WG(s) | Presented for information at plenary# |  | Approved at plenary# |  | Comments |
| 36.8xy | Study on Enhanced Interference Suppression for LTE |  | RAN1 | RAN2 | RAN#60 |  | RAN#62 |  |  |
| Affected existing specifications * [None in the case of Study Items] |  |  |  |  |  |  |  |  |  |
| Spec No. | CR | Subject |  |  |  | Approved at plenary# |  | Comments |  |

## 11		Work item rapporteur(s) *

Broadcom Corporation, Amin Mobasher (mobasher@broadcom.com)

## 12		Work item leadership *

Primary: RAN1

Secondary: RAN2, RAN4

## 13		Supporting Individual Members *

Supporting IM name

Broadcom Corporation

[Orange]

[Renesas]

[Intel]

[Qualcomm]

[MediaTek]

[Verizon]

[AT&T]

[Sprint]

[CMCC]

[China Telecom]

[China Unicom]

[Telecom Italia]

[T-Mobile]

[Ericsson]

[ST-Ericsson]

[Huawei]

[CATT]

[ZTE]

[KDDI]

[NTT DoCoMo]

[Panasonic]

[Sharp]

[Hitachi]

[Vodafone]

[TI]

[NEC]

[NSN]

[LGE]

[Motorola]

[Samsung]

[China Mobile]

[CEWiT]

[Telefonica]

[USCellular]

[CATR]

[BTInternet]

[DT]

[DISH]

[ALU]

[Rogers]

form change history:

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"