3GPP TSG-RAN4 Meeting #71 	R4-143870

# Seoul, South Korea, 19-23 May 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 2373 | rev | 2 | Current version: | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCell activation and deactivation delay requirements for 3 DL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Alcatel-Lucent, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_B2_B5_B30, LTE_CA_B2_B29_B30, LTE_CA_B4_B5_B30, LTE_CA_B4_B29_B30, LTE_CA_B2_B4_B13, LTE_CA_B2_B2_B13, LTE_CA_B4_B4_B13, LTE_CA_C_B41_3DL, LTE_CA_B1_B3_B8, LTE_CA_B1_B5_B7, LTE_CA_B2_B4_B5, LTE_CA_B2_B4_B12, LTE_CA_B2_B5_B12, LTE_CA_B2_B12_B12, LTE_CA_B2_B12_B30, LTE_CA_B3_B7_B20, LTE_CA_B3_B8_B27, LTE_CA_B4_B5_B12, LTE_CA_B4_B12_B12, LTE_CA_B4_B12_B30, LTE_CA_B1_B3_B5, LTE_CA_B1_B3_B20, LTE_CA_B1_B7_B20, LTE_CA_B7_B8_B20, LTE_CA_3DL_FB_BW, LTE_CA_B3_B7_B7, LTE_CA_B1_B19_B21, LTE_CA_B3_B3_B7, LTE_CA_C_B40_3DL |  |  |  |  |  |  | Date: |  | 2014-04-30 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | There are no requirements related to SCell activation and deactivation delay for 3 DL CA |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following RRM requirements for 3 DL CA are specified: - SCell activation and deactivation delay requirements for 3 DL CA are being introduced The requrements are defined for two cases: - While an SCell being activated or deactivated, the other SCell not activated, deactivated, configured or deconfigured. In this case exiting delay requirements apply. - While an SCell being activated or deactivated, the other SCell may also be activated, deactivated, configured or deconfigured. In this case the - SCell activation or deactivation delay is extended to accoutnt for interruption due to the activation, deactivation, configuration or deconfiguration of the other SCell. For each such interruption a maximum of 5 subframes is enough. Therefore the total SCell activation delay is expressed as: Tactivate_total = Tactivate_basic+ K*5 - Tactivate_total is expressed in subframes. - Tactivate_basic is the SCell activation delay specified in section 7.7.2; - K is the number of times the other SCell is activated, deactivated, configured or deconfigured while the SCell is being activated; SCell deactivation delay is the same as defined for 2 DL CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of SCell activation and deactivation in 3 DL CA cannot be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.1, 7.7.2, 7.7.3, 7.7.4 and 7.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged sections have been omitted >

## 7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation

### 7.7.1	Introduction

This section defines requirements for the delay within which the UE shall be able to activate a deactivated SCell and deactive an activated SCell in E-UTRA carrier aggregation. The requirements are applicable to an E-UTRA carrier aggregation capable UE which has been configured with one or two downlink SCells. The requirements shall apply for both E-UTRA FDD and TDD.

If multiple downlink SCells are activated or deactivated in the same MAC control element as defined in [17], the requirements shall apply to each of the SCells in the MAC control element.

### 7.7.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this section shall apply for the UE configured with one downlink SCell.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in subframe n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command as specified in [17] for the SCell being activated no later than in subframe n+24 provided the following conditions are met for the SCell:

-	During the period equal to max([5] measCycleSCell, [5] DRX cycles) before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SCell being activated remains detectable according to the cell identification conditions specified in section 8.3.3.2,

-	SCell being activated also remains detectable during the SCell activation delay according to the cell identification conditions specified in section 8.3.3.2.

Otherwise upon receiving the SCell activation command in subframe n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command as specified in [17] for the SCell being activated no later than in subframe n+34 provided the SCell can be successfully detected on the first attempt.

If there is no reference signal received for the CSI measurement over the delay corresponding to the minimum requirements specified above, then the UE shall report corresponding valid CSI for the activated SCell on the next available uplink reporting resource after receiving the reference signal.

If there are no uplink resources for reporting the valid CSI in subframe n+24 or n+34 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

The valid CSI is based on the UE measurement and corresponds to any CQI value specified in [3] with the exception of CQI index = 0 (out of range) provided:

-	the conditions in section 7.7 are met over the entire SCell activation delay and

-	the conditions for CQI reporting defined in Section 7.2.3 [3] are met.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in [17] for an SCell at the first opportunities for the corresponding actions once the SCell is activated.

The PCell interruption specified in section 8.3.3 shall not occur before subframe n+5 and not occur after subframe n+9 for E-UTRA FDD.

The PCell interruption specified in section 8.3.3 shall not occur before subframe n+5 and not occur after subframe n+11 for E-UTRA TDD.

Starting from subframe n+9 for E-UTRA FDD UE or subframe n+11 for E-UTRA TDD UE and until the UE has completed the SCell activation, the UE shall send CSI with CQI index = 0 (out of range) if the UE is configured to report the CQI in SCell.

### 7.7.3	SCell Deactivation Delay Requirement for Activated SCell

The requirements in this section shall apply for the UE configured with one downlink SCell.

Upon receiving SCell deactivation command or upon expiry of the sCellDeactivationTimer in subframe n, the UE shall accomplish the deactivation actions specified in [17] for the SCell being deactivated no later than in subframe n+8.

The PCell interruption specified in section 8.3.3 shall not occur before subframe n+5 and not occur after subframe n+9 for E-UTRA FDD.

The PCell interruption specified in section 8.3.3 shall not occur before subframe n+5 and not occur after subframe n+11 for E-UTRA TDD.

### 7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Muliple Downlnk SCells

The requirements in this section shall apply for the UE configured with two downlink SCells.

While activating a SCell if the UE does not receive any command to activate, deactivate, configure or deconfigure the other SCell during the SCell activation delay then the UE shall meet the SCell activation delay requirements specified in section 7.7.2.

While activating a SCell if the other SCell is activated, deactivated, configured or deconfigured by the UE then the UE shall meet the SCell activation delay requirements (Tactivate_total) according to the following expression:

Tactivate_total = Tactivate_basic+ K*5

Where:

Tactivate_total is the total time to activate a SCell and is expressed in subframes.

Tactivate_basic is the SCell activation delay specified in section 7.7.2;

K (1 ≤ K ≤ [3]) is the number of times the other SCell is activated, deactivated, configured or deconfigured while the SCell is being activated;

### 7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells

The requirements in this section shall apply for the UE configured with two downlink SCells.

The UE shall deactivate a SCell and meet the SCell deactivation delay requirements specified in section 7.7.3 regardless of whether the other SCell is activated, deactivated, configured or deconfigured or not by the UE during the SCell deactivation delay.

< Unchanged sections have been omitted >